H2-SHIFT: Convert. Don't Replace.

Round 2 Annexure: supporting evidence, calculations and plans

Team Wookies | Raunak Jain, Abhishek Aby, Divya | NIT Warangal

SRDI Igniters Innovation Challenge 2026, Engineering Track, Option 2: Hydrogen Retrofit Solution. Research cut-off 14 September 2026.

This annexure supports the 10-slide main presentation and the workbook H2SHIFT_model.xlsx. All numbers here come from the recalculated workbook (base scenario) or from the source register. Evidence classes: A verified external evidence; B our calculated estimate; C explicit assumption or sensitivity range; D proposed target requiring validation. No customer interviews, letters of interest, test results, supplier quotations or partnerships are claimed; proposed partners are proposals.

Contents: A1 Claim audit | A2 Assumptions and source register | A3 Cost model | A4 Packaging, energy balance and performance | A5 Regulatory matrix and hazard analysis | A6 Market sizing | A7 Global benchmarks | A8 Pilot design and first 90 days | A9 Budget, responsibilities and funding | A10 Judge Q&A, validation log and change log.

# A1. Claim audit: Round 1 versus evidence

Being shortlisted is evidence of competition progress, not proof of technical performance or commercial feasibility. Each Round 1 claim was checked against the research; the table gives the verdict and its Round 2 consequence.

| Original claim (Round 1) | Evidence provided in Round 1 | Verification result | Keep / refine / correct | Round 2 implication |
| --- | --- | --- | --- | --- |
| 7.5 million CNG vehicles in India; 3x growth in 8 years | CRISIL CNG outlook (cited by name) | Latest official: 81.95 lakh on road (Mar 2025, PNGRB citing Vahan); 72.10 lakh (Jul 2024). '3x in 8 years' unverifiable (no official 2016-17 baseline retrieved) | Correct | Use 8.2 million (Mar 2025); quote +40 percent Mar 2023 to Mar 2025 |
| About 65 percent of CNG vehicles are passenger cars | None | No official category split found; estimate 45 to 60 percent from factory CNG car sales and conversions | Correct | Use 55 percent as a labelled estimate (class B) with a range |
| Maruti Suzuki about 71 percent share of India's CNG passenger market; 6.2 lakh CNG cars sold FY25 | Maruti FY25 investor update | Verified: 71.6 percent (CY2024 industry data); 6.12 to 6.2 lakh FY25; FY26 above 7 lakh; June 2026 monthly share 42 percent | Keep and update | Donor pool argument stands |
| 4,000 plus retrofit centres | None | Only a vendor blog; no official count (Vahan and IGL lists exist) | Correct | Say 'authorised retrofit centres exist in every CGD city; count to be confirmed from Vahan' |
| Spark-ignition engine, gaseous injection, existing cylinder packaging make CNG to hydrogen the 'shortest technical jump' | Engineering reasoning | Partly true: engine and cylinder bay help; but no CNG fuel-system part can be reused with pure hydrogen (ISO 11439 steel limited to 2 percent hydrogen; UN R134 qualification needed); the hydrogen cylinder is 84 mm larger in diameter | Refine | Retain the argument for the engine and bay; drop any implied component reuse |
| HCNG step: 18 to 20 percent hydrogen, no vehicle modification, zero customer investment, minus 76 percent CO, minus 25 percent HC, plus 4 to 5 percent economy | IOCL HCNG reformer patent | Legal since GSR 585(E) 25 Sep 2020 (IS 17314:2019). Delhi 50-bus BS-IV trial: about minus 70 percent CO, minus 25 percent HC, NOx unchanged, plus 4 to 5 percent economy (heavy-duty). Hydrogen supplies only about 6.2 percent of the blend's energy. HCNG dispensing exists only at demonstration scale | Correct | Reframe as an air-quality measure with about 6 percent fossil-carbon displacement, dependent on CGD blending; not a decarbonisation step |
| Bi-fuel H2 plus CNG with automatic switching, CNG cylinder retained, 1.2 to 1.5 kg Type IV at 350 bar, kit ₹1.8 to 2.2 lakh | Indicative BOM (cylinder 55 to 60 percent) | Two cylinders do not fit a Dzire or WagonR boot; the factory car already has petrol as backup. A 60 L composite cylinder holds 1.44 kg nominal, 1.12 kg usable. Installed cost ₹3.2 lakh at 10,000 per year (₹2.3 lakh ex-works), ₹7.7 lakh per pilot car | Correct | Architecture becomes H2 plus petrol (petrol retained); CNG hardware removed; cost ranges revised |
| Range 400 plus km combined, refuel 3 to 5 min | None | Hydrogen 61 km per fill plus 555 km on petrol; two depot fills give about 120 km per day on hydrogen; refuel 3 to 5 min plausible for a non-cooled 350 bar fill | Refine | Quote hydrogen range separately from petrol range |
| Step 3: dedicated H2-ICE, second cylinder, lean burn plus EGR, about 100 percent tailpipe CO2-free, upgrade ₹0.8 to 1.0 lakh | None | No space for a second cylinder in the sedan; a 700 bar cylinder of the same size gives 105 km. Zero tailpipe CO2 on hydrogen km is correct but NOx remains | Correct | Step 3 becomes a conditional 700 bar or hub-scale option after 2030 |
| H2-ICE mileage 55 to 60 km/kg | Energy density plus retrofit studies | Energy balance at CNG parity gives about 60 km/kg on declared figures; fleet real-world with a lean PFI retrofit modelled at 54 km/kg (1.85 kg/100 km), range 43 to 63 | Refine | Use 1.85 kg/100 km with a stated range; cross-check on the Energy_Range sheet |
| Green hydrogen price ₹397 (2026) to ₹260-310 (2028) to ₹160-200 (2030) per kg; 'at ₹200/kg delivered running cost equals CNG' | IEA and BNEF cost outlooks | These are production-cost trajectories, not pump prices. Delivered 2030 central estimate ₹360/kg (CEEW, USD 4.2); our conservative build-up ₹584/kg at a 500 kg/day hub at 70 percent utilisation; pilot point ₹1,433/kg. CNG fuel parity needs ₹188/kg delivered | Correct | Economics restated with delivered prices; parity condition made explicit |
| CNG ₹78/kg, 26 km/kg | Current market levels | Delhi CNG ₹86.98/kg (Sep 2026, after the 2026 gas price rises); fleet real-world 24 to 28 km/kg | Update | ₹87/kg base, 25 km/kg base |
| Running-cost chart: BEV ₹1.2 to 1.8/km | None | Model gives ₹1.5/km energy cost with a 70/30 depot and public mix at ₹6 and ₹20/kWh | Keep |  |
| Fleet taxi 150 km/day, 330 days: about ₹1.5 lakh annual saving versus petrol, 18-month payback | Taxi benchmark | The donor runs on CNG, not petrol; against CNG the retrofit never pays back in 5 years at any credible hydrogen price. NITI Aayog uses 70,000 km/yr for taxis | Correct | Comparator changed to CNG (and BEV); payback claim withdrawn |
| Safety table: hydrogen has 'different risks, not higher risks with the right design'; regulation AIS-157 plus PESO plus ISO/SAE | None | Properties verified (4 to 75 percent, 0.017 mJ, 3.8x diffusivity). AIS-157 is the fuel-cell standard and its scope excludes retrofitted vehicles; AIS-195 (2023, Amd 1 2025) is the hydrogen ICE standard for new M and N vehicles; no retrofit route exists | Correct | Regulatory pathway rebuilt around AIS-195, Rule 126/115 template, PESO GCR 2025; gate 1 added |
| Burst test at or above 2.25x | None | Consistent with GTR 13 / ISO 19881 minimum burst ratio for carbon-fibre composite containers | Keep | Cite standard |
| Flywheel: 7,400 to 17,500 CNG stations by 2030; 8.6 lakh t/yr capacity awarded, only about 8,000 t/yr commissioned | Government data | 8,150 plus stations (Aug 2025); 17,700 (MWP commitment) or 18,000 (target) by 2030; 8.62 lakh tpa awarded (May 2025) versus about 8,000 tpa commissioned (Feb 2026): verified | Update | Flywheel logic kept for utilisation; price, not demand, is the binding constraint |
| Roadmap: 100 stations and 500 fleet cars (2026-27); 25,000 kits in 5 cities (2027-28); 1,00,000 plus cars (2029-30) | None | No approval route, no car-size cylinder approval, three captive dispensing points nationally; station-limited capacity about 1,400 cars by 2030 | Correct | Roadmap replaced by four gated phases: 3 prototypes (2027-28), 30 pilot cars (2028-30), conditional 1,000 to 1,500 cars after 2030 |
| Impact by 2030: 1,00,000 cars, 0.9 Mt CO2 avoided, 15,000 t/yr hydrogen demand, ₹2,000 crore retrofit industry, 10,000 jobs | None | Bounded model: about 1,400 cars, 3,600 t CO2e/yr, 924 t/yr hydrogen; kit revenue about ₹45 crore; jobs not estimated | Correct | Impact restated with ranges; job claim withdrawn |
| Global proof points: Toyota GR Corolla H2 and Hilux prototypes | Toyota | GR Corolla H2 is a liquid-hydrogen race car; the Hilux prototype (UK) is a fuel-cell vehicle; the hydrogen combustion van is the HiAce (Australia, 2023-25, less than 200 km range, SCR for NOx) | Correct | Cite HiAce, not Hilux, for hydrogen combustion |
| Suzuki hydrogen Burgman Street prototype, 70 MPa | Suzuki | Verified (JMS 2023, re-shown 2025, 'not launching anytime soon'); add Suzuki-AVL hydrogen Swift prototype (1.4 L DI, 100 kW, Vienna, May 2026) | Keep and add | Stronger small-car relevance, still pre-commercial |
| India: 70 pilot vehicles plus 16 stations, ₹410.77 crore | Government pilot projects | Verified (PIB 29 Jul 2026): 12 projects, 27 buses and 43 trucks, 16 stations, 21 routes, ₹41.58 crore released; no cars | Keep and qualify | Note absence of cars and the 4-wheeler eligibility of the scheme |
| Regulatory pathway: CMVR, AIS-157, ARAI/ICAT, PESO GCR, IS 16061, IS/ISO 17268, ISO 19880 / SAE J2601; 'mirror the CNG retrofit homologation pathway' | Standard names | Standards exist (IS 16061:2021 fuel quality; IS/ISO 17268:2020; IS 18538 series for stations) but none covers a hydrogen retrofit kit; AIS-157 excludes retrofits; the CNG template (AIS-024/028, Rule 115) is the right analogy but needs a new instrument | Refine | Compliance matrix with responsible parties and gate |
| NOx control: lean burn lambda at or above 2, EGR, compact SCR if required, BS-VI compliance pathway | None | Consistent with literature: near-zero NOx above lambda 2 at steady state; transients need aftertreatment; a DI engine met below 35 mg/km on WLTC only with SCR | Keep and qualify | Lean NOx trap provision budgeted; compliance is a target |
| Carbon credits USD 10 to 15/t | Voluntary market range | India voluntary prices ₹1,350 to 2,800/t (2026); global average USD 3.5/t (2025); immaterial (at most ₹0.2/km) | Correct | Credits treated as upside only |
| Kit BOM: cylinder 55 to 60 percent, injectors and rail 15 percent, ECU 10 percent, sensors/valves/TPRD 15 percent | Indicative | Model BOM at volume: cylinder and valve 49 percent of parts; pilot cylinder cost dominates even more | Refine | BOM sheet with pilot and volume columns |

# A2. Assumptions, evidence classes and source register

## A2.1 Model inputs (base, downside for the retrofit, upside)

| Key | Parameter | Base | Downside | Upside | Unit | Class | Sources | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General frame |  |  |  |  |  |  |  |  |
| decision_year | Decision year for the owner's choice (first commercial kits after certification) | 2,028 | 2,028 | 2,028 | year | C |  | Pilot vehicles run from 2028; owner decision modelled at commercial availability |
| horizon | Analysis horizon (fixed at 5 years in the cash-flow table) | 5.000 | 5.000 | 5.000 | years | C |  | Remaining useful life of a 3-year-old fleet sedan |
| disc | Discount rate (nominal) | 0.120 | 0.140 | 0.100 | fraction | A | W4-S29 | NBFC fleet loan rates 10-16 percent |
| days | Operating days per year | 330.000 | 330.000 | 330.000 | days | C | R1 | Fleet taxi benchmark (Round 1) |
| km_day | Kilometres per operating day | 167.000 | 212.000 | 150.000 | km/day | A | W4-S30 | 55,000 km/yr base; NITI Aayog Feb 2026 plans 70,000 km/yr for 4W taxis (downside); Round 1 used 150 |
| fuel_esc | Annual escalation of petrol and CNG prices | 0.030 | 0.020 | 0.040 | fraction/yr | C |  | Nominal |
| elec_esc | Annual escalation of electricity tariffs | 0.020 | 0.010 | 0.030 | fraction/yr | C |  | Nominal |
| vot | Value of in-shift time lost to refuelling or charging | 150.000 | 150.000 | 150.000 | Rs/hour | C |  | Approximate contribution margin per hour of a fleet taxi |
| Donor vehicle (Maruti Suzuki Dzire Tour S CNG, about 3 years old at decision) |  |  |  |  |  |  |  |  |
| donor_value_now | Resale value of donor at decision date (taxi-registered) | 350,000 | 400,000 | 300,000 | Rs | C | W4-S28 | New Tour S CNG ex-showroom about Rs 7.1 lakh; taxi retention 45-55 percent at 3-4 years; higher value favours selling for a BEV |
| donor_value_end | Residual value of donor at end of horizon (about 8 years old, high km) | 100,000 | 70,000 | 130,000 | Rs | C | W4-S28 |  |
| cng_price | CNG retail price, Delhi (IGL), September 2026 | 87.000 | 78.000 | 95.000 | Rs/kg | A | W4-S16 | Rs 86.98/kg on 14 Sep 2026 (aggregator); Ahmedabad Rs 94; lower CNG price is downside for the retrofit |
| cng_kmkg | CNG consumption in fleet use (real world, AC, load) | 25.000 | 27.000 | 23.000 | km/kg | C | W1-S3b;W1-S8a | Declared 32.1 km/kg (Tour S); owner reports 24-30; fleet use assumed lower |
| petrol_price | Petrol retail price, Delhi | 102.000 | 95.000 | 111.000 | Rs/L | A | W4-S19 | Rs 102.12/L verified 25 May 2026; aggregators show Rs 111 in Sep 2026 |
| petrol_kml | Petrol consumption in fleet use (real world, AC) | 15.000 | 16.000 | 14.000 | km/L | C | W1-S3a | Declared 26.1 km/L (Tour S); fleet use assumed lower |
| petrol_share_cng | Share of km run on petrol by the CNG car (CNG unavailable, queues) | 0.050 | 0.050 | 0.050 | fraction | C |  |  |
| maint_cng | Maintenance and tyres, CNG car | 0.900 | 0.700 | 1.200 | Rs/km | C | W4-S22 | CEEW 2025 Table 5 to be transcribed; Round 1 value retained |
| ins_cng | Insurance per year, CNG car | 18,000 | 18,000 | 18,000 | Rs/yr | C |  |  |
| cng_retest | CNG cylinder hydrostatic test every 3 years | 2,000 | 2,000 | 2,000 | Rs per test | A | W2-S24 | Gas Cylinder Rules: 3-yearly test |
| time_cng_h | In-shift time lost refuelling CNG per day (queues) | 0.500 | 0.400 | 0.600 | hours/day | C |  |  |
| Hydrogen retrofit (H2-ICE, port injection, 350 bar composite cylinder, petrol retained as automatic backup) |  |  |  |  |  |  |  |  |
| kit_cost | Installed conversion cost at commercial volume, incl. taxes (from BOM sheet) | 322,123 | 402,654 | 273,805 | Rs | B | BOM | Linked to BOM volume total |
| kit_cost_pilot | Installed conversion cost per pilot vehicle (from BOM sheet) | 774,375 | 774,375 | 774,375 | Rs | B | BOM |  |
| salvage_cng | Credit for removed CNG cylinder and kit | 10,000 | 0.000 | 20,000 | Rs | C |  | Requalified cylinder resale |
| downtime_days | Conversion downtime | 3.000 | 4.000 | 2.000 | days | C |  |  |
| lost_margin_day | Lost contribution per day of downtime | 1,500 | 1,500 | 1,500 | Rs/day | C |  |  |
| h2_cons | Hydrogen consumption, H2-ICE mode, fleet use (real world, AC) | 1.850 | 2.300 | 1.600 | kg/100 km | B | W1-S12a;W1-S12c;W1-S9a | Energy balance versus CNG (Energy_Range); Ford Focus H2RV 1.4, Mazda RX-8 RE 2.4, HiAce van 3.1 kg/100 km |
| h2_share | Share of km driven on hydrogen (rest on petrol backup) | 0.650 | 0.500 | 0.850 | fraction | C |  | Two depot fills per day; check Energy_Range max share |
| h2_price_y1 | Delivered hydrogen price at dispenser, year 1 (2028), unsubsidised, shared hub station | 800.000 | 1,000 | 550.000 | Rs/kg | B | W4-S22;W4-S46 | CEEW: small-network refuelling today above USD 10/kg; H2_Supply build-up gives the pilot-station price |
| h2_change | Annual change in delivered hydrogen price | -0.100 | -0.030 | -0.150 | fraction/yr | C | W4-S22 | CEEW path to USD 4.2/kg (about Rs 360) delivered in 2030 |
| maint_h2 | Maintenance and tyres, retrofitted car | 1.000 | 1.200 | 0.850 | Rs/km | C | W4-S22 | Adds injector, sensor, regulator service |
| ins_h2 | Insurance per year, retrofitted car | 21,000 | 24,000 | 19,000 | Rs/yr | C | W2-S53 | No IMT endorsement exists for hydrogen kits; assumes insurer accepts endorsed retrofit |
| h2_inspect_yr | Hydrogen system inspection and 3-yearly cylinder requalification, annualised | 4,000 | 6,000 | 3,000 | Rs/yr | C | W2-S24 |  |
| time_h2_h | In-shift time lost for hydrogen refuelling and detour per day | 0.400 | 0.600 | 0.300 | hours/day | C |  | Two short fills at a depot dispenser |
| mod_resid_factor | Residual value factor for a modified vehicle | 0.900 | 0.800 | 1.000 | fraction | C |  |  |
| kit_resid | Residual value of the kit at end of horizon | 0.000 | 0.000 | 20,000 | Rs | C |  |  |
| BEV alternative (sell donor, buy compact fleet BEV) |  |  |  |  |  |  |  |  |
| bev_price | BEV ex-showroom price incl. GST (compact fleet BEV, 24-30 kWh) | 1,050,000 | 900,000 | 1,300,000 | Rs | A | W3-S27;W3-S25 | Tata Tiago EV Rs 6.99-10.09 lakh, Punch EV Rs 9.69-12.59 lakh (2026), Xpres-T fleet EV from Rs 13.04 lakh |
| bev_onroad | Registration and first-year extras beyond ex-showroom | 30,000 | 20,000 | 40,000 | Rs | A | W4-S25 | Delhi EV Policy 2026 waives road tax and registration fee to March 2030 |
| bev_kwh100 | BEV consumption measured at the meter (incl. charging losses) | 15.000 | 13.000 | 17.000 | kWh/100 km | A | W3-S30;W4-S23 | Real-world tests 12.8-15.3 kWh/100 km at the battery (Tiago EV, Punch EV, Nexon EV) plus 5-15 percent charging losses |
| depot_tariff | Depot or home charging tariff, landed | 6.000 | 5.000 | 8.000 | Rs/kWh | A | W4-S20;W3-S32 | Delhi DERC EV-charging tariff Rs 4.50/kWh (LT) plus surcharges and taxes |
| pub_tariff | Public DC fast-charging price | 20.000 | 16.000 | 25.000 | Rs/kWh | A | W4-S21;W3-S33 | 2026 network prices Rs 16-25/kWh incl. GST |
| pub_share | Share of energy from public fast charging | 0.300 | 0.200 | 0.500 | fraction | C |  |  |
| maint_bev | Maintenance and tyres, BEV | 0.500 | 0.400 | 0.800 | Rs/km | C | W4-S22 |  |
| ins_bev | Insurance per year, BEV | 30,000 | 28,000 | 34,000 | Rs/yr | C |  |  |
| bev_resid | BEV residual value at end of horizon | 420,000 | 480,000 | 340,000 | Rs | C | W4-S22 | 40 percent of price at 5 years (no Indian fleet-EV residual data found); higher residual is downside for the retrofit |
| time_bev_h | In-shift time lost for charging per day | 0.500 | 0.300 | 1.000 | hours/day | C |  | Overnight depot charging is idle time |
| Energy and storage physics |  |  |  |  |  |  |  |  |
| cng_lhv | Lower heating value of CNG | 48.000 | 48.000 | 48.000 | MJ/kg | A | W4-S62 | IPCC natural gas 48.0 TJ/Gg |
| petrol_lhv | Lower heating value of petrol | 32.200 | 32.200 | 32.200 | MJ/L | A | W4-S62 | 44.3 MJ/kg x 0.74 kg/L |
| h2_lhv | Lower heating value of hydrogen | 120.000 | 120.000 | 120.000 | MJ/kg | A | W1-S13a |  |
| cyl_vol | Hydrogen cylinder internal (water) volume | 60.000 | 60.000 | 60.000 | L | A | W1-S2 | Luxfer G-Stor Pro V060H35 (Type III, 350 bar): 60 L, OD 400 mm, length 767 mm, 36 kg, 1.45 kg H2 |
| h2_density_350 | Hydrogen density at 350 bar, 15 C | 24.000 | 24.000 | 24.000 | kg/m3 | A | W1-S13a | Full-fill density per SAE J2601 definition |
| h2_density_700 | Hydrogen density at 700 bar, 15 C | 40.200 | 40.200 | 40.200 | kg/m3 | A | W1-S13a |  |
| fill_soc | State of charge reached in a non-cooled 350 bar fill (hot ambient) | 0.850 | 0.800 | 0.950 | fraction | C | W1-S13d | SAE J2601: 95-100 percent needs pre-cooled dispensing; small tanks may fall outside light-duty tables |
| resid_density | Residual gas density at minimum usable pressure | 1.700 | 2.000 | 1.500 | kg/m3 | C | W1-S13c | About 20 bar residual |
| fills_day | Depot fills per day available to the driver | 2.000 | 1.500 | 2.500 | fills/day | C |  |  |
| cyl_mass | Composite cylinder mass (60 L, 350 bar) | 36.000 | 42.000 | 32.000 | kg | A | W1-S2 | Luxfer Type III 36 kg; Type IV similar |
| kit_other_mass | Valve, frame, regulator, lines, ECU mass | 14.000 | 16.000 | 12.000 | kg | C |  |  |
| cng_cyl_mass | Removed CNG cylinder (55 L Type I) mass | 55.000 | 55.000 | 55.000 | kg | A | W1-S14a;W1-S14b | 50 L steel cylinder 48 kg; 60 L 61-69 kg |
| cng_kit_mass | Removed CNG regulator, rail, lines mass | 8.000 | 8.000 | 8.000 | kg | C |  |  |
| petrol_ps | Donor engine power on petrol (Tour S 1.2 L) | 81.600 | 81.600 | 81.600 | PS | A | W1-S3a | 60 kW at 5,700 rpm |
| cng_ps | Donor engine power on CNG | 70.000 | 70.000 | 70.000 | PS | C | W1-S6 | Third-party 69 bhp; official CNG rating not captured |
| h2_power_factor | H2 lean-burn peak power relative to petrol | 0.520 | 0.450 | 0.600 | fraction | B | W1-S9a;W1-S9b | About 30 percent air displacement (20 percent loss at stoichiometric) plus 35-40 percent loss at lambda 2; strategy uses lambda 1.6-2.2 |
| petrol_tank | Petrol tank capacity | 37.000 | 37.000 | 37.000 | L | A | W1-S3a |  |
| Hydrogen supply and station |  |  |  |  |  |  |  |  |
| pilot_cars | Pilot fleet size | 30.000 | 30.000 | 30.000 | cars | D |  | Statistical spread across three fleets, one dispensing point |
| h2_prod_cost | Green hydrogen production cost at plant gate (2026-28) | 400.000 | 560.000 | 300.000 | Rs/kg | A | W4-S33;W4-S34;W4-S38 | CEEW USD 3.5-5/kg; industry Rs 397-560; SIGHT ammonia auctions imply up to Rs 302 for the hydrogen molecule |
| h2_prod_cost_2030 | Green hydrogen production cost at plant gate (2030) | 240.000 | 320.000 | 180.000 | Rs/kg | A | W4-S22;W4-S57;W4-S36 | CEEW USD 2-2.5; IEA about USD 3; NITI-RMI best case USD 1.6 |
| transport_cost | Tube-trailer transport to station | 45.000 | 60.000 | 35.000 | Rs/kg | B | W4-S44 | About 150 km, 300 kg payload |
| station_capex_pilot | Dispensing point capex, pilot (compressor, cascade, one dispenser, civil, safety, PESO) | 100,000,000 | 150,000,000 | 80,000,000 | Rs | B | W4-S56;W4-S43 | Indian trade source Rs 14-20 crore for 200-500 kg/day; delivered hydrogen, no electrolyser |
| station_cap_pilot | Dispensing point design capacity, pilot | 150.000 | 150.000 | 150.000 | kg/day | C |  |  |
| station_util_pilot | Station utilisation, pilot | 0.500 | 0.350 | 0.650 | fraction | C | W4-S22 | Shared with bus/truck pilot users on the Sahibabad-Faridabad-Delhi route |
| station_capex_2030 | Station capex, 2030 hub scale (500 kg/day) | 120,000,000 | 200,000,000 | 90,000,000 | Rs | B | W4-S56 |  |
| station_cap_2030 | Station design capacity, 2030 hub | 500.000 | 500.000 | 500.000 | kg/day | C |  |  |
| station_util_2030 | Station utilisation, 2030 hub | 0.700 | 0.500 | 0.850 | fraction | C | W4-S22;W4-S46 | CEEW assumes 30 percent in 2030 rising to 80 percent by 2050; ICCT 50-70 percent |
| station_life | Station capital recovery period | 10.000 | 10.000 | 10.000 | years | C |  |  |
| station_om | Fixed operation and maintenance, share of capex per year | 0.040 | 0.050 | 0.030 | fraction/yr | A | W4-S22 | CEEW 5 percent |
| comp_kwh | Compression and dispensing electricity | 3.000 | 3.500 | 2.500 | kWh/kg | C |  | Delivered gas at 200 bar to 450 bar cascade |
| ind_tariff | Industrial electricity tariff | 9.000 | 10.000 | 8.000 | Rs/kWh | C |  |  |
| h2_loss | Venting, purge and metering losses | 0.030 | 0.050 | 0.020 | fraction | C |  |  |
| h2_margin_tax | Retailer margin and taxes on delivered hydrogen | 0.100 | 0.150 | 0.050 | fraction | C |  | GST treatment of hydrogen as automotive fuel to be confirmed |
| trailer_kg | Tube-trailer payload | 300.000 | 250.000 | 350.000 | kg | C |  | Typical 200 bar tube trailer |
| Emissions factors |  |  |  |  |  |  |  |  |
| ef_cng | Tailpipe CO2 per kg CNG | 2.690 | 2.690 | 2.750 | kg CO2/kg | A | W4-S62 | IPCC 2006 default 56.1 t/TJ at 48 TJ/Gg |
| wtt_cng | Well-to-tank uplift for CNG in India | 0.250 | 0.150 | 0.350 | fraction | C | W4-S63;W4-S17 | Imported spot LNG share raises upstream emissions; India-specific factor not found |
| ef_petrol | Tailpipe CO2 per litre petrol | 2.270 | 2.270 | 2.310 | kg CO2/L | A | W4-S62 | IPCC 2006 default |
| wtt_petrol | Well-to-tank uplift for petrol | 0.200 | 0.150 | 0.250 | fraction | C | W4-S63 |  |
| ci_h2_green | Carbon intensity, green hydrogen incl. compression and delivery | 2.000 | 3.500 | 1.200 | kg CO2e/kg H2 | A | W4-S63;W4-S64;W4-S65 | India green hydrogen standard: 2 kg CO2e/kg well-to-gate |
| ci_h2_grid | Carbon intensity, grid-electrolysis hydrogen (India grid 2025) | 39.000 | 43.000 | 35.000 | kg CO2e/kg H2 | B | W4-S59 | 0.71 t/MWh x 50-60 kWh/kg |
| ci_h2_grey | Carbon intensity, grey (SMR) hydrogen | 12.000 | 14.000 | 10.000 | kg CO2e/kg H2 | A | W4-S64 |  |
| grid_now | Grid emission factor at the meter, 2025 | 0.860 | 0.860 | 0.860 | kg CO2/kWh | A | W4-S59 | CEA v21 weighted average 0.710 t/MWh (FY2024-25) adjusted for about 17 percent T&D losses |
| grid_2030 | Grid emission factor at the meter, 2030 | 0.580 | 0.520 | 0.660 | kg CO2/kWh | A | W4-S60 | CEA NEP path 0.548 (2026-27) to 0.430 (2031-32) net, adjusted for T&D losses; lower grid factor is downside for the retrofit |
| bev_mfg | Manufacturing emissions, compact BEV incl. 27 kWh battery | 7.000 | 6.000 | 8.500 | t CO2e | B | W4-S63;W4-S66 | Glider 4-6 t plus 27 kWh x 70 kg CO2e/kWh |
| bev_life_km | Lifetime km over which BEV manufacturing is amortised | 150,000 | 180,000 | 120,000 | km | C |  |  |
| kit_mfg | Manufacturing emissions of the retrofit kit | 0.500 | 0.700 | 0.400 | t CO2e | B |  | Carbon-fibre cylinder dominant |
| nox_bs6 | BS-VI NOx limit, M1 positive ignition | 60.000 | 60.000 | 60.000 | mg/km | A | W4-S71 |  |
| Market sizing |  |  |  |  |  |  |  |  |
| cng_stock | CNG vehicles on road in India, all categories (March 2025) | 8,195,000 | 7,500,000 | 8,800,000 | vehicles | A | W3-S2 | 81.95 lakh (PNGRB citing Vahan, Aug 2025); 72.10 lakh in Jul 2024 (Lok Sabha reply); Round 1 said 7.5 million |
| cng_pv_share | Share of passenger cars in the CNG stock | 0.550 | 0.450 | 0.600 | fraction | B | W3-S10;W3-S12;W3-S15 | No official split; estimated from cumulative factory-CNG car sales (Maruti about 2.6 million FY20-FY26 plus others) and aftermarket conversions; Round 1 said 65 percent |
| eligible_share | Share of CNG cars that are factory-fitted, boot-cylinder layout, under 6 years old at 2028 | 0.500 | 0.450 | 0.550 | fraction | B | W3-S10;W3-S12 | Factory CNG sales FY23-FY26 versus total CNG cars |
| fleet_share | Share of eligible cars in commercial or institutional fleets | 0.200 | 0.150 | 0.250 | fraction | C | W3-S18;W3-S25 | Delhi alone registers about 60,000-85,000 taxis; no national fleet split published |
| hub_share | Share of fleet cars in cities with a hydrogen hub by 2030 | 0.400 | 0.300 | 0.500 | fraction | C | W3-S36;W3-S37 | MNRE corridors: Delhi-NCR (Sahibabad-Faridabad-Delhi), Ahmedabad-Vadodara-Surat, Pune-Mumbai, Kochi and others |
| adopter_share | Plausible early-adopter share (return-to-base fleets) | 0.050 | 0.030 | 0.080 | fraction | C |  | Hypothesis; needs customer validation |
| hub_stations_2030 | Fleet-hub stations serving cars by 2030 | 8.000 | 4.000 | 15.000 | stations | D | W3-S37 | 16 MNRE pilot stations sanctioned for buses and trucks; assumes half add light-duty dispensing, conditional on gates |

## A2.2 Source register (309 entries; access date 14 September 2026)

Prefix W1 engineering, W2 safety and regulation, W3 market, infrastructure and benchmarks, W4 economics and emissions. 'Excerpt' or 'summary' in Limitations means the original could not be opened through the build environment's network policy and the entry rests on indexed text of the page; re-verify before external use. Additional gap-closure sources read in full through the search index: PIB PRID 2006052 (14 Feb 2024), PIB PRID 2107795 (3 Mar 2025), Scribd copy of AIS-157 scope, Fuel Cells Works and RushLane on the Suzuki-AVL hydrogen Swift (May 2026), Luxfer hydrogen product page, Hexagon Purus Type 4 table (PDF read).

| ID | Claim supported | Publisher | Title | URL | Date | Section | Status | Limitations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W1-S1 | Hexagon Type IV cylinder dimensions/mass/H2 capacity (350 & 700 bar) | Hexagon Purus | Hydrogen Type 4 cylinder information | https://s3.eu-central-1.amazonaws.com/hexagonpurus-website/HPU_0222_12_HydrogenType4_Tabelle_2pages.pdf | REV. 08/22 | p.2 table | spec | "frequently built units; minimum quantities may apply" |
| W1-S2 | Luxfer Type III 350 bar 60/74/205 L cylinders | Luxfer Gas Cylinders | G-Stor Pro H2 Type 3 (EU) spec table | https://www.luxfercylinders.com/product/g-stor-h2-hydrogen-cylinders/ | 2023–25 (page) | EU table | spec | Firecrawl excerpt; Type III not Type IV |
| W1-S3a | Tour S petrol power/torque, FE, tanks, GC | Maruti Suzuki | Dzire Tour S page / Maruti Suzuki Commercial page | https://www.marutisuzuki.com/tour/dzire ; https://www.marutisuzukicommercial.com/dzire-tour-s | 2025–26 | spec block | spec | excerpt; CNG power not in excerpt |
| W1-S3b | Tour S S-CNG 32.12 km/kg | Maruti Suzuki | Media release: Maruti Suzuki launches the All-new Tour S | https://www.marutisuzuki.com/content/dam/arena-eds/corporate/press-release/assets/documents/2023/february/Media_Release_Maruti_Suzuki_launches_the_All-new_Tour_S.pdf | Feb 2023 | body | spec (declared) | excerpt |
| W1-S3c | Dzire 4th gen kerb/GVW/FE/boot/tanks | Maruti Suzuki | Arena Dzire brochure; Dzire price page | https://www.marutisuzuki.com/content/dam/msil/arena/in/en/assets/cars/dzire/documents/Arena-Dzire-Brochure.pdf ; https://www.marutisuzuki.com/arena/dzire/price | 2024–26 | spec table | spec | excerpt |
| W1-S3d | WagonR kerb 850, GVW 1340, tank 32, boot 335; engines K10C/K12N | Maruti Suzuki | WagonR price page; WagonR page | https://www.marutisuzuki.com/arena/wagon-r/price ; https://www.marutisuzuki.com/arena/wagon-r | 2025–26 | spec table | spec | excerpt; variant for kerb/GVW not stated |
| W1-S3d2 | WagonR Tour H3 25.22 km/l, 34.41 km/kg | Maruti Suzuki | WagonR Tour H3 page | https://www.marutisuzuki.com/tour/wagonr | 2025–26 | FAQ | spec | excerpt |
| W1-S3e | Swift 1197 cc 60 kW @5700 | Maruti Suzuki | Swift page | https://www.marutisuzuki.com/arena/swift | 2025–26 | spec | spec | excerpt |
| W1-S3f | Ertiga 1462 cc, 75.8 kW @6000, 139 Nm @4300 | Maruti Suzuki | Ertiga page | https://www.marutisuzuki.com/arena/ertiga | 2025–26 | spec | spec | excerpt |
| W1-S3g | 1.5 L K-series S-CNG 64.6 kW @5500, 121.5 Nm | Maruti Suzuki | Press release: All New Brezza S-CNG | https://www.marutisuzuki.com/corporate/media/press-releases/2023/march/maruti-suzuki-introduces-all-new-brezza-s-cng | Mar 2023 | body | spec | same engine as Ertiga S-CNG; excerpt |
| W1-S3h | Ertiga CNG 121.5 Nm @4200 | Maruti Suzuki | Arena Ertiga brochure | https://www.marutisuzuki.com/content/dam/msil/arena/in/en/assets/cars/ertiga/document/Arena-Ertiga-Brochure.pdf | 2024 | spec | spec | excerpt |
| W1-S3i | Ertiga kerb 1150–1205/1250–1255, GVW 1760/1820, tanks 45/60 | Maruti Suzuki | Ertiga price page; 6-page brochure | https://www.marutisuzuki.com/arena/ertiga/price ; https://www.marutisuzuki.com/content/dam/msil/arena/in/en/assets/cars/ertiga/document/ErtigaBrochure_6_Pgs_High_Res.pdf | 2024–26 | spec | spec | excerpt |
| W1-S3j | Ertiga 20.51 km/l, 26.11 km/kg | Maruti Suzuki | Press release Next-Gen Ertiga | https://www.marutisuzuki.com/corporate/media/press-releases/2022/april/inspire-togetherness-again-maruti-suzuki-next-gen-ertiga-arrives-at-arena-showrooms | Apr 2022 | body | spec | excerpt |
| W1-S3k | Ertiga Tour M 21.10 km/l, 26.54 km/kg, 45 L | Maruti Suzuki | Ertiga Tour M page | https://www.marutisuzuki.com/tour/ertiga | 2025–26 | FAQ | spec | excerpt |
| W1-S3l | Eeco Tour V 60.5 kW @6000, 105.5 Nm @3000, 20.20 km/l | Maruti Suzuki | Eeco Tour V page | https://www.marutisuzuki.com/tour/eeco | 2025–26 | spec | spec | excerpt |
| W1-S3m | Eeco S-CNG 27.05 km/kg, 104.4 Nm @3000 | Maruti Suzuki | Press release New Eeco | https://www.marutisuzuki.com/corporate/media/press-releases/2022/november/maruti-suzuki-new-eeco-more-power-more-fuel-efficient-and-more-style | Nov 2022 | body | spec | excerpt |
| W1-S3n | Eeco kerb 970, 19.71 km/l, price from ₹5,20,900 | Maruti Suzuki | Eeco price page | https://www.marutisuzuki.com/arena/eeco/price | 2025–26 | spec | spec | excerpt |
| W1-S3o | Eeco CNG kerb 1085, 26.78 km/kg | Maruti Suzuki | Arena Eeco brochure | https://www.marutisuzuki.com/content/dam/msil/arena/in/en/assets/cars/eeco/document/Arena-Eeco-Brochure.pdf | 2024–25 | spec | spec | excerpt |
| W1-S3p | Eeco S-CNG 65 L tank | Maruti Suzuki | Arena Eeco page | https://www.marutisuzuki.com/arena/eeco | 2025–26 | body | spec | excerpt |
| W1-S3q | Swift 24.8/25.75 km/l | Maruti Suzuki | Swift page | https://www.marutisuzuki.com/arena/swift | 2025–26 | spec | spec | excerpt |
| W1-S3r | Swift S-CNG 32.85 km/kg | Maruti Suzuki | Press release Epic New Swift S-CNG; Swift S-CNG brochure | https://www.marutisuzuki.com/corporate/media/press-releases/2024/september/epic-new-swift-s-cng ; https://www.marutisuzuki.com/content/dam/msil/arena/in/en/assets/cars/swift/document/Swift_SCNG_VerticalBrochure.pdf | Sep 2024 | body | spec | excerpt; torque truncated |
| W1-S3s | Swift kerb 920/990, GVW 1355/1425, tanks 37/55 | Maruti Suzuki | Swift price page | https://www.marutisuzuki.com/arena/swift/price | 2025–26 | spec | spec | excerpt |
| W1-S4 | Aura CNG 50.5 kW, 95.2 Nm, tanks 37/65, dims, price | Hyundai Motor India | Aura specification/performance/price pages | https://www.hyundai.com/in/en/find-a-car/aura/specification ; https://www.hyundai.com/in/en/find-a-car/aura/performance ; https://www.hyundai.com/in/en/find-a-car/aura/price | 2026 | spec table | spec | excerpt; kerb weight absent |
| W1-S5 | Tigor iCNG specs, ARAI mileage, tanks, price | CarDekho (third party) | Tata Tigor specifications | https://www.cardekho.com/tata/tigor/specs | 2025–26 | spec table | spec (third party) | not OEM page |
| W1-S5b | Twin-cylinder frees 200–205 L boot | carandbike (third party) | Tata Tigor page | https://www.carandbike.com/tata-cars/tigor | 2025–26 | body | claimed | third party |
| W1-S6 | Tour S CNG 69 bhp/101.8 Nm, 34.3 km/kg, kerb 1045, boot 382, GC 163 | CarDekho / ZigWheels (third party) | Dzire Tour S CNG overview; Tour S specs | https://www.cardekho.com/overview/Maruti_Dzire_Tour_S/Maruti_Dzire_Tour_S_CNG.htm ; https://www.zigwheels.com/maruti-suzuki-cars/dzire-tour-s/specifications/ | 2026 | spec | spec (third party) | search-summary |
| W1-S7 | 2025–26 ex-showroom prices (Tour S CNG ₹7.10 L; WagonR CNG ₹5.79–7.62 L; Ertiga CNG ~₹9.5 L; Eeco AC CNG ₹6.46 L; Swift VXi CNG ₹7.57 L) | CarDekho/CarWale/ZigWheels (aggregators) | various price pages | https://www.cardekho.com/overview/Maruti_Dzire_Tour_S/Maruti_Dzire_Tour_S_CNG.htm ; https://www.carwale.com/maruti-suzuki-cars/wagon-r/cng/ ; https://www.carwale.com/maruti-suzuki-cars/ertiga/cng/ ; https://www.cardekho.com/overview/Maruti_Eeco/Maruti_Eeco_5_Seater_AC_CNG.htm ; https://www.cardekho.com/overview/Maruti_Swift/Maruti_Swift_VXi_CNG.htm | Sep 2026 snapshot | price block | third-party | search-summary; unverified |
| W1-S7b | WagonR CNG 56 bhp/82.1 Nm, 60 L, 33.47 km/kg | CarWale (third party) | WagonR CNG page | https://www.carwale.com/maruti-suzuki-cars/wagon-r/cng/ | 2026 | body | spec (third party) | search-summary |
| W1-S8a | Dzire CNG real-world 28–30 km/kg city | Cars24 (blog) | Maruti Suzuki Dzire Real World Mileage | https://www.cars24.com/article/maruti-suzuki-dzire-real-world-mileage/ | 2025–26 | body | anecdotal | not a fleet study |
| W1-S8b | WagonR CNG real 24–28 km/kg; 3-yr hydrotest | VahanBazaar (blog) | CNG Mileage Real-World India | https://vahanbazaar.in/tips/cng-mileage-real-world-india-vahanbazaar | 2026 | body | anecdotal | low confidence |
| W1-S9a | H2 ≈30 % of mixture volume; ~20 % power loss at stoich PFI | Int. J. Hydrogen Energy (Elsevier) | Hydrogen-fuelled ICEs: Direct Injection versus Port-Fuel Injection | https://www.sciencedirect.com/science/article/pii/S0360319924028106 | 2024 | intro/results | observed/reviewed | excerpt + search-summary; full text blocked |
| W1-S9b | λ=2 torque −34.9–40.7 %; BTE 40.4 % (DI e-turbo); NOx <10 ppm above λ 2.7; BTE ~38 % lean | multiple (ACS Omega 2024; IJHE 2025; Springer IJAT 2026) | see URLs | https://pubs.acs.org/doi/10.1021/acsomega.4c02222 ; https://www.sciencedirect.com/science/article/pii/S036031992501715X ; https://link.springer.com/article/10.1007/s12239-026-00469-z | 2024–26 | abstracts | observed | search-summary; attribution to individual papers not verified |
| W1-S9c | Supercharging + EGR power recovery | Int. J. Hydrogen Energy | Increasing the power output of hydrogen ICEs by means of supercharging and EGR (Verhelst et al.) | https://www.sciencedirect.com/science/article/abs/pii/S036031990900442X | 2009 | title/abstract | observed | content blocked |
| W1-S9d | ">30 % of gasoline" power with supercharged stoich + EGR | LAPSE/PSE Community (review preprint) | Progress of Performance, Emission, and Technical Measures of H2 ICEs | https://psecommunity.org/wp-content/plugins/wpor/includes/file/2302/LAPSE-2023.9491-1v1.pdf | 2023 | excerpt | reviewed | wording ambiguous |
| W1-S9e | Backfire mitigation by injection control (review exists) | Int. J. Hydrogen Energy | Hydrogen port fuel injection: Review of fuel injection control strategies to mitigate backfire | https://www.sciencedirect.com/science/article/abs/pii/S036031992401348X | 2024 | title | reviewed | content blocked |
| W1-S9f | Timed manifold injection overcomes backfire (IIT Delhi) | Int. J. Hydrogen Energy | Performance characteristics of a hydrogen fuelled S.I. engine using timed manifold injection (Das) ; Hydrogen engine: R&D programmes in IIT Delhi | https://www.sciencedirect.com/science/article/abs/pii/036031999190038K ; https://www.sciencedirect.com/science/article/abs/pii/S0360319901001781 | 1991; 2002 | abstracts | observed | search-summary |
| W1-S9g | Knock at spark >26° CA; smaller advance than gasoline | OSTI ETDE record (Indian study) | Implications of combustion parameters on the ... | https://www.osti.gov/etdeweb/servlets/purl/21400931 | n.d. | abstract | observed | search-summary; blocked |
| W1-S9h | NOx peak at λ=1.3, ~2.7× λ=1 | KAUST repository (Onorati et al., Int. J. Engine Res.) | The potential of hydrogen ICEs for heavy-duty ... / role of H2 for future ICEs | https://repository.kaust.edu.sa/bitstreams/e16f5716-15c8-4c8f-9951-bab74c8db2c9/download | 2022 | excerpt | reviewed | excerpt only |
| W1-S10a | Bosch PFI for H2 engines product page | Bosch Mobility | Port fuel injection for hydrogen engines | https://www.bosch-mobility.com/en/solutions/powertrain/gas/port-fuel-injection-for-hydrogen-engines/ | 2024–25 | page | announced | blocked; title via search |
| W1-S10b | PHINIA low-pressure H2 PFI injectors; DI-CHG10/15 ratings | PHINIA; MarkLines | Hydrogen Fuel Injectors; IAA 2024 note | https://www.phinia.com/technologies/technologies/hydrogen-solutions/fuel-injectors ; https://www.marklines.com/en/news/314084 | 2024 | page | announced | search-summary |
| W1-S10c | Bosch DI injector 17 g/s @40 bar, 2026; Liebherr H964/H966; ~60 bar limit | Power Progress; Mobility Engineering Tech | Fuel injection must follow the hydrogen; Liebherr Q&A | https://www.powerprogress.com/news/fuel-injection-must-follow-the-hydrogen/8036089.article ; https://www.mobilityengineeringtech.com/component/content/article/tohe/pub/regulars/q-a/47237 | 2024–25 | body | announced | search-summary |
| W1-S10d | OMB Saleri HPR 350 two-stage regulator; Bosch–OMB agreement; H2 valve catalogue | OMB Saleri; Green Car Congress | Hydrogen valves; catalogue 2023; Bosch partners with OMB Saleri | https://www.omb-saleri.it/products/hydrogen/ ; https://www.omb-saleri.it/uploads/categories/catalogo2023h2.pdf ; https://www.greencarcongress.com/2021/11/20211118-bosch.html | 2021–23 | page/catalogue | production | search-summary |
| W1-S10e | Lexier 350 bar HPR and OTV | Lexier | Pressure Regulator 350 bar; On Tank Valve 350 bar | https://lexierweb.com/en/hydrogen/valves/pressure-regulators/hpr-350bar/ ; https://lexierweb.com/en/hydrogen/vehicles-components/on-tank-valve-350bar/ | 2025 | page | production | search-summary |
| W1-S10f | Poppe+Potthoff TOPAQ OTV 2.0, R134/HGV 3.1 tests, TPRD variants | Poppe + Potthoff; Hydrogen Central | On Tank Valve; certification news | https://www.poppe-potthoff.com/hydrogen/on-tank-valve/ ; https://hydrogen-central.com/hydrogen-tank-valves-by-poppe-potthoff-successfully-complete-required-as-part-of-the-certification-according-to-r134/ | Sep 2024 | page | production-intent | search-summary |
| W1-S10g | Sensirion STC42A automotive H2 sensor specs | Sensirion | STC42A product page / press | https://sensirion.com/products/catalog/STC42A ; https://sensirion.com/company/news/press-releases-and-news/article/sensirion-announces-global-availability-of-the-stc42a-automotive-h2-sensor-for-thermal-runaway-detection | 2024–25 | datasheet summary | production | search-summary |
| W1-S10h | Bosch H2 leak-detection sensors | Bosch Mobility | Sensors for fuel cell-electric systems | https://www.bosch-mobility.com/en/solutions/powertrain/fuel-cell-electric/sensors-for-fuel-cell-electric-systems/ | 2025 | page | production | search-summary |
| W1-S10i | NTM SenseH2 | NTM Sensors / AZoSensors | SenseH2 Hydrogen Sensor | https://www.azosensors.com/equipment-details.aspx?EquipID=339 | n.d. | page | production | search-summary |
| W1-S11a | ISO 11439: ≤2 % H2 for steel UTS >950 MPa; 0.1 % wet gas | iso11439.com FAQ (secondary) | CNG Cylinder standard ISO 11439 – FAQ | http://www.iso11439.com/faq.php | n.d. | FAQ | standard text (secondary) | standard itself not opened |
| W1-S11b | MoRTH allowed 18 % H-CNG in CNG engines; CMVR amendment | Autocar Professional | MoRTH allows use of H-CNG in CNG engines | https://www.autocarpro.in/news-national/morth-allows-use-of-hcng-in-cng-engines-67299 | Sep 2020 | body | policy | search-summary |
| W1-S11c | ARAI: 18 % HCNG −70 % CO, −25 % HC, +4–5 % fuel economy (HD BS-IV) | IAS Gyan (secondary, quoting PIB) | Notification allowing use of H-CNG issued | https://www.iasgyan.in/daily-current-affairs/notification-allowing-use-of-h-cng-issued | 2020 | body | reported | secondary; PIB blocked |
| W1-S11d | IOCL reformer + 50 DTC buses, Rajghat Depot-1, Oct 2020 | IndianOil; pv magazine India | IndianOil press release 58744; Delhi HCNG trial | https://iocl.com/NewsDetails/58744 ; https://www.pv-magazine-india.com/2020/10/21/pollution-hit-delhi-starts-hydrogen-enhanced-cng-trial-for-buses/ | Oct 2020 | body | announced | IOCL page blocked |
| W1-S11e | SIAM-led trial: 18 % chosen; NMHC −58 %, THC −23 %, NOx −50 % | NPC / DOE (topic paper) | NPC Future Transportation Fuels Study – Topic paper 25 HCNG | https://www.npc.org/FTF_Topic_papers/25-HCNG.pdf ; https://www.energy.gov/sites/default/files/2022-10/25-HCNG.pdf | 2012 (data ~2010) | body | reported | blocked; search-summary |
| W1-S12a | BMW Hydrogen 7: 191 kW, 8 kg LH2, ~201 km, 3.7 / 2.1 kg/100 km | Wikipedia summarising Wallner et al., IJHE 2008; BMW press | BMW Hydrogen 7; Fuel economy and emissions evaluation of BMW Hydrogen 7 Mono-Fuel demonstration vehicles | https://en.wikipedia.org/wiki/BMW_Hydrogen_7 ; https://www.sciencedirect.com/science/article/abs/pii/S0360319908011439 ; https://www.press.bmwgroup.com/global/article/detail/T0012215EN/ | 2006–08 | body | observed (lab cycle) | primary blocked; summary |
| W1-S12b | Mazda RX-8 Hydrogen RE: 110 L @350 bar, 2.4 kg, ~107 hp, ~100 km | Wikipedia; Green Car Journal | Mazda RX-8 Hydrogen RE | https://en.wikipedia.org/wiki/Mazda_RX-8_Hydrogen_RE ; https://greencarjournal.com/dont-miss/mazdas-hydrogen-rx-8-re/ | 2006 | body | claimed | secondary |
| W1-S12c | Ford Focus H2RV: 2.3 L SC H2 ICE, 110 hp + 33 hp, 45 mi/kg, 125 mi | EE Power; AutoIntell | Ford Unveils Hydrogen ICE; H2ICE Ford Focus | https://eepower.com/news/ford-unveils-hydrogen-internal-combustion-engine/ ; https://www.autointell.com/News-2003/September-2003/September-2003-4/September-24-03-p4.htm | 2003 | body | claimed | secondary |
| W1-S12c2 | Ford E-450 H2ICE shuttle: 6.8 L V10 SC, 350 bar 26 GGE tank, ~150 mi | Green Car Congress; TRID | Ford H2ICE Shuttle Bus; Engineering the Ford H2 IC Engine Powered E-450 | https://www.greencarcongress.com/2004/09/ford_hsub2subic.html ; https://trid.trb.org/view/1813766 | 2004–06 | body | claimed | hp not confirmed |
| W1-S12d | Toyota Hydrogen HiAce: 3.4 L V6, 120 kW, 354 Nm, SCR/Euro VI, 7 vehicles, Nov 2023 | Toyota Australia (via RACQ/CarExpert/WhichCar) | Toyota unveils Hydrogen HiAce; reviews | https://www.toyota.com.au/discover/news/toyota-unveils-hydrogen-powertrain-technology-for-pilot-program-in-australia ; https://www.racq.com.au/articles/drive/2023/11/toyota-tests-hydrogen-fuelled-v6-powered-hiace ; https://www.whichcar.com.au/news/hydrogen-powered-toyota-hiace-revealed-with-200km-range | Nov 2023 | body | announced | Toyota pages blocked |
| W1-S12e | HiAce ~6.2 kg, 141 L, <200 km, >70 L/100 km; CPB month-long pilot | Drive.com.au; CarExpert; CPB Contractors | 2024 Toyota HiAce hydrogen prototype review; CPB pilot news | https://www.drive.com.au/reviews/2024-toyota-hiace-hydrogen-prototype-quick-drive/ ; https://www.carexpert.com.au/car-reviews/2024-toyota-hydrogen-hiace-prototype-review-quick-drive ; https://www.cpbcon.com.au/news-and-media/2024/cpb-contractors-completes-australian-first-toyota-hydrogen-fuelled-vehicle-pilot | 2024 | body | claimed/derived | consumption derived by me |
| W1-S12f | GR Corolla H2 liquid hydrogen; range doubled; tank >220 L; 2026 superconducting pump | Toyota Global Newsroom / Toyota Europe | Super Taikyu releases | https://global.toyota/en/newsroom/corporate/43550115.html ; https://newsroom.toyota.eu/evolved-liquid-hydrogen-powered-gr-corolla-to-participate-in-super-taikyu-fuji-24-hours-race/ | 2023–26 | body | announced | no kg/100 km |
| W1-S12g | HySE formation and member roles | Honda Global | Ministerial approval granted to form HySE | https://global.honda/en/newsroom/news/2023/c230517eng.html | 17 May 2023 | body | announced | — |
| W1-S12h | Ashok Leyland/RIL H2-ICE truck; >20 trucks ~250,000 km | Business Standard; Hydrogen Central | Indian CV majors drive hydrogen transition; AL & RIL unveil | https://www.business-standard.com/industry/auto/indian-cv-majors-drive-hydrogen-transition-with-h2-vehicles-125061600725_1.html ; https://hydrogen-central.com/ashok-leyland-ril-unveil-indias-first-hydrogen-powered-heavy-duty-truck/ | 2023–25 | body | announced | press |
| W1-S12i | IIT Delhi H2 SI genset with KOEL and IOCL R&D | IIT Delhi | Technology for Hydrogen Utilization in SI Engine Generator | https://home.iitd.ac.in/show.php?id=16&in_sections=Press | n.d. (2023–24) | press | announced | — |
| W1-S12j | Delhi hydrogen 3-wheeler fleet (15 vehicles) | capacity4dev (EU) | World's first hydrogen-powered three-wheelers launched in New Delhi | https://capacity4dev.europa.eu/discussions/worlds-first-hydrogen-powered-three-wheelers-launched-new-delhi_en | 2012 | body | announced | old |
| W1-S13a | 24.0 g/L (35 MPa) and 40.2 g/L (70 MPa) at 15 °C | MDPI Energies | Effect of Hydrogen Refueling Parameters on Final State of Charge | https://www.mdpi.com/1996-1073/12/4/645 | 2019 | §2 | modelled (EOS) | excerpt |
| W1-S13b | 39.75 kg/m³ at 700 barg, 20 °C | Demaco | The energy density of hydrogen | https://demaco-cryogenics.com/blog/energy-density-of-hydrogen/ | n.d. | body | modelled | vendor blog |
| W1-S13c | Regulator to ≤20 bar; ~5 % unusable; 20 bar min | USPTO patents (Hyundai/Toyota-type) | e.g. US 8927171; US 11394041; US 10868315 | https://image-ppubs.uspto.gov/dirsearch-public/print/downloadPdf/8927171 | 2010s–2020s | descriptions | design practice | patents, not standards |
| W1-S13d | J2601 scope ≥49.7 L; 85 °C; T40 3-min 95–100 % SOC; T20/T30 slower | SAE / ANSI store; CEC | SAE J2601-2020 abstract; SAE J2601 status presentation | https://webstore.ansi.org/standards/sae/sae26012020 ; https://efiling.energy.ca.gov/GetDocument.aspx?tn=66608 | 2020; 2014 | abstract | standard | full tables not accessible |
| W1-S13e | Non-communication fills need tight pre-cooling tolerance; 85 °C liner limit | OSTI (NREL); Atawey | Impact of hydrogen SAE J2601 fueling methods; Understanding SAE J2601 | https://www.osti.gov/servlets/purl/1389635 ; https://atawey.com/en/sae-j2601-understanding-the-international-standard-for-gaseous-hydrogen-refueling/ | 2017; 2025 | body | reviewed | search-summary |
| W1-S13f | Time Technoplast PESO approvals (250 L Type IV H2; BV 150 L) | Business Standard; Machine Maker; Upstox | PESO approval news | https://www.business-standard.com/markets/capital-market-news/time-technoplast-gets-final-approval-from-peso-for-manufacturing-composite-cylinders-124062100121_1.html ; https://themachinemaker.com/news/time-technoplast-receives-peso-approval-to-make-hydrogen-cylinders/ | Mar–Jun 2024; May 2025 | body | announced | press |
| W1-S13g | Everest Kanto Type-4 composite cylinders | Everest Kanto (via search) | Type IV cylinders | https://ekcuae.com/3d-flip-book/type-4-composite-cylinder/ | n.d. | page | announced | blocked |
| W1-S13h | Mirai 2021: 5.6 kg; 52/25.3/64.9 L = 142.2 L | Encyclopedia.pub (citing Toyota 2022 Mirai product guide) | Application of Hydrogen Storage Tanks | https://encyclopedia.pub/entry/47333 | 2023 | §3 | spec (secondary) | Toyota guide not opened |
| W1-S13i | Nexo 156.6 L / 6.33 kg; new Nexo 162.2 L / 6.69 kg | Encyclopedia.pub; InsideEVs; Hyundai News | see URLs | https://encyclopedia.pub/entry/47333 ; https://insideevs.com/news/755466/2026-hyundai-nexo-hydrogen-fuel-cell-official-specs/ ; https://www.hyundainews.com/assets/documents/original/52348-2023NEXOProductSpecifications20220720.pdf | 2018–2025 | spec | spec | secondary |
| W1-S13j | DOE Record 19008 costs/capacities | US DOE HFTO (Strategic Analysis) | Onboard Type IV Compressed Hydrogen Storage System – Cost and Performance Status 2019 | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/19008_onboard_storage_cost_performance_status.pdf | 25 Nov 2019 | p.1–2 | modelled (DFMA) | blocked; search-summary |
| W1-S13k | DOE Record 24006 costs | US DOE HFTO (Strategic Analysis) | Onboard Type IV Compressed Hydrogen Storage System – Cost and Performance Status | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/24006-onboard-storage-cost-performance-status.pdf | 28 Apr 2025 | p.1–2 | modelled (DFMA) | blocked; search-summary |
| W1-S13l | ~$33/kWh low-volume (2013 basis) | US DOE Record 13010 / SA 2013 final report | Onboard storage performance and cost | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/13010_onboard_storage_performance_cost.pdf ; https://www.osti.gov/servlets/purl/1082754 | 2013 | — | modelled | low confidence |
| W1-S13m | 15-yr life / 11,000 cycles (GTR 13, GB/T 42612) | ResearchGate (comparative analysis paper); UNECE GTR 13 | A comparative analysis of the RCS for on-board high-pressure hydrogen storage cylinders; GTR 13 Amend.1 | https://www.researchgate.net/publication/374204598 ; https://unece.org/sites/default/files/2023-07/ECE-TRANS-180-Add.13-Amend1e.pdf | 2023 | body | standard | search-summary |
| W1-S13n | AIS-195A: GCR 2016; PESO may use IS 16735, ISO 19881, R134, GTR 13 | ARAI (HMR) | AIS-195A Safety and Procedural Requirements for hydrogen vehicles | https://hmr.araiindia.com/api/AISFiles/AIS-195A_dc4d3241-c4b7-466f-8c37-6bd23851f0cc.pdf | 2023–24 | clause on containers | standard | blocked; search-summary |
| W1-S13o | PESO Gas Cylinders (Amendment) Rules 2024 (Hydrogen) | PESO | Gas Cylinders (Amendment) Rules, 2024 (Hydrogen) | https://peso.gov.in/web/en/gas-cylinders-amendment-rules-2024-hydrogen | 2024 | page | regulation | blocked |
| W1-S14a | 50 L CNG cylinder: 48 kg, 835 mm, 316 mm | GAIL Gas (CGD) | CNG FAQ | https://gailcgd.gail.co.in/CGD/Entry/CNGfaq | n.d. | FAQ | spec (typical) | excerpt |
| W1-S14b | 60 L Type 1: OD 325 mm, 61.4 kg, ISO 11439, 200 bar | Anhui Clean Energy (vendor) | Diameter 325 mm 60L CNG cylinder | https://www.gascylindersupplier.com/diameter-325-mm-60l-cng-cylinder | n.d. | page | spec | non-Indian vendor |
| W1-S14c | 60 L Type 1: 69 kg | PWENT (vendor) | Steel 60 Liter CNG Gas Cylinder 200 Bar | https://pwent.eu/en/products/products/gas-cylinder-products-high-pressure-low-pressure/cng-gas-cylinder/steel-60-liter-cng-gas-cylinder-200-bar/ | n.d. | page | spec | vendor |
| W1-S14d | R110: 20 g / 8 g for M1, N1 (10/5 M2,N2; 6.6/5 M3,N3) | UNECE | Regulation No. 110 Rev.3 | https://www.unece.org/fileadmin/DAM/trans/main/wp29/wp29regs/2015/R110r3e.pdf | 2015 | container installation clause | regulation | excerpt (verbatim) |
| W1-S14e | AIS-024 / AIS-028 Ver.4 CNG code of practice | ARAI (HMR) / MoRTH | AIS-024 (Version 4) AIS-028 (Version 4) | https://hmr.araiindia.com/Control/AIS/825201593205AM4_AIS-024_and_AIS-028_CNG-Ver4_F.pdf | 2015 | — | standard | blocked |
| W2-S1 | H-CNG added to CMVR Rule 115; IS 17314:2019 | MoRTH (Gazette); PIB | G.S.R. 585(E) "hydrogen CNG"; PIB "Notification allowing use of H-CNG issued" | https://morth.nic.in/sites/default/files/notifications_document/GSR%20585(E)%2025%20September%202020%20hydrogen%20CNG.pdf ; https://www.pib.gov.in/PressReleasePage.aspx?PRID=1659706 | 25-09-2020 | Rule 115 insertion | In force | Blocked; wording from search summary |
| W2-S2 | IS 17314:2019 H-CNG spec, 18 % H2 | BIS (via PIB/press) | IS 17314:2019 Hydrogen enriched CNG for automotive purposes | (BIS catalogue; not accessed) | 2019 | — | In force | 18 % figure from press, not standard text |
| W2-S3 | Rule 115 CNG kit provisions; s.52 14-day endorsement | Indian Kanoon (CMVR text) | Section 115 / 115B, CMVR 1989 | https://indiankanoon.org/doc/10585089/ ; https://indiankanoon.org/doc/67316961/ | — | Rule 115 (sub-rules not numbered here) | In force | Summary only |
| W2-S4 | ARAI retrofitment certification (Rule 115, Rule 126 agencies) | ARAI | Vehicle Retrofitment Certification in India | https://www.araiindia.com/certification/certification/retrofitment | — | — | Current | Blocked |
| W2-S5 | Retrofit framework (CNG/LPG/EV; AIS-123) | EVreporter (ARAI compiled) | Regulatory Framework for Retrofitting in India | https://evreporter.com/regulatory-framework-for-retrofitting-in-india/ | — | — | — | Secondary |
| W2-S6 | BS-VI CNG/LPG retrofit permitted | MoRTH; PIB; Business Standard | G.S.R. 625(E) "retrofitment in BS VI" | https://morth.gov.in/sites/default/files/notifications_document/7GSR%20625(E)%2011th%20August%202022%20retrofitment%20in%20BS%20VI.pdf ; https://pib.gov.in/PressReleasePage.aspx?PRID=1853835 | 11-08-2022 | — | In force | Blocked |
| W2-S7 | AIS-024/028 Version 4 exist | ARAI (HMR) | AIS-024 (Version 4) / AIS-028 (Version 4) | https://hmr.araiindia.com/Control/AIS/825201593205AM4_AIS-024_and_AIS-028_CNG-Ver4_F.pdf | — | — | Published | Blocked; title only |
| W2-S8 | TA cost Rs 40–50 lakh, 3-yr validity | Autocar Professional | Delays in MoRTH guidelines for BS VI CNG kits hurting retrofit business | https://www.autocarpro.in/news-national/delays-in-morth-guidelines-for-bs-vi-cng-kits-hurting-retrofit-business-92660 | 2022 (approx.) | — | — | Industry claim |
| W2-S9 | Rule 126 provisos (retrofit self-certification; imported CBUs for testing/research) | Commercial Law Publishers (CMVR e-book) | The Central Motor Vehicles Rules, 1989 — page 335 (Rule 126) | https://ebook.commerciallawpublishers.com/fa/cmvr/files/basic-html/page335.html | ed. incl. GSR 221(E) 30-03-2021 | Rule 126 provisos | In force (pre-2026 text) | Verbatim via Firecrawl; may pre-date 2026 amendment |
| W2-S10 | 2026 amendment substituting Rule 126 provisos; self-certification | The Bar Bulletin; TeamLease RegTech | Centre Notifies Changes to Motor Vehicle Type Approval Norms…; Draft CMV (…Amendment) Rules 2025 | https://thebarbulletin.com/centre-revises-motor-vehicle-type-approval-norms-self-certification-retrofitted-vehicles/ ; https://teamleaseregtech.com/updates/article/52441/draft-central-motor-vehicles-amendment-rules-2025/ | 2026 | Rule 126 | In force (per source) | GSR no./date NOT FOUND |
| W2-S11 | AIS 157:2020 notified for M/N FCEVs; ISO 14687 | MoRTH via India Code; GKToday; Mercom | 9th GSR 579(E) dated 23 September 2020 hydrogen fuel cell | https://www.indiacode.nic.in/ViewFileUploaded?path=AC_CEN_30_42_00009_198859_1517807326286/notificationindividualfile/&file=9th_gsr_579%28e%29_dated_23_september_2020_hydrogen_fuel_cell.pdf ; https://indiacode.ecourtsindia.com/rules/7ec3e5bb/ ; https://gktoday.in/current-affairs/standards-for-safety-evaluation-of-vehicles-propelled-by-hydrogen-fuel-cells ; https://www.mercomindia.com/inclusion-hydrogen-fuel-central-motor-vehicles-act | 23-09-2020 | new rule (see S12) | In force | Blocked; snippet |
| W2-S12 | FCEV rule numbered 125MA; fuel IS 16061:2021; H2-ICE Rule 125M | Applus IDIADA | IRIS Highlights – July 2026 | https://www.digitalsolutions.applusidiada.com/blogs/news/iris-highlighs-july-2026 | 07-2026 | — | — | Secondary digest |
| W2-S13 | Rule 125M; AIS 195:2023; M & N | MoRTH | 8th GSR 746(E) dated 16 October 2023 Hydrogen ICE | https://morth.nic.in/sites/default/files/notifications_document/8th%20GSR%20746(E)%20dated%2016th%20october%202023%20Hydrogen%20ICE.pdf | 16-10-2023 | Rule 125M | In force | Blocked; summary |
| W2-S14 | GSR 720(E) 04-10-2023 also cites AIS 195:2023 | (search index; MoRTH) | — | — | 04-10-2023 | — | Unresolved | Not verified |
| W2-S15 | AIS-195 with Amd 1 exists; Annexure VIII LH2 | ARAI (HMR); Scribd copy | AIS-195 with Amd 1 | https://hmr.araiindia.com/api/AISFiles/AIS-195%20with%20Amd%201_eccd18bd-9eae-4e08-bbb2-7823b7964666.pdf ; https://www.scribd.com/document/856143651/AIS-195-cc6897f6-30be-4dc9-b188-b7c52017bebb | 2023 / Amd 1 05-2025 | 1.0 Scope; Annex VIII | Published | Blocked; scope not captured |
| W2-S16 | AIS-195A CEV; TPRD definitions/tests | ARAI (HMR); Scribd | AIS-195A Safety and Procedural Requirements for … Hydrogen Powered CEV | https://hmr.araiindia.com/api/AISFiles/AIS-195A_dc4d3241-c4b7-466f-8c37-6bd23851f0cc.pdf ; https://www.scribd.com/document/1057373433/AIS-195-A | 2024–25 | 1.0 Scope; TPRD clauses | Published | Fragments only |
| W2-S17 | AIS-206 L-category scope; "AIS 157 and AIS 195 published for M & N" | Scribd (AIS-206 copy) | Hydrogen Vehicle Safety Standards AIS-206 | https://www.scribd.com/document/902873977/AIS-206 | 2025 | 1.0 Scope | Published/draft | Fragment |
| W2-S18 | AIS-157A/DF (FC CEV) draft | ARAI (HMR) | Draft AIS-157A/DF June 2024 | https://hmr.araiindia.com/api/AISFiles/AIS157A_CEV_DF_67e85335-2f04-40ee-ac66-ecb704d7bd46.pdf | 06-2024 | Scope | Draft | Blocked |
| W2-S19 | ARAI AIS list updated 7 May 2026 | ARAI (HMR) | List of published AIS (07-05-2026) | https://hmr.araiindia.com/storage/ais-details/1778147654_0_List_of_AIS_published-07_05_2026.pdf | 07-05-2026 | — | Current | Blocked |
| W2-S20 | Hydrogen registration mark | MoRTH via Complinity/autopdp | CMV (Fourth Amendment) Rules 2025, G.S.R. 819(E) | https://complinity.com/legal-update/central-motor-vehicles-fourth-amendment-rules-2025-21166/ | 03-11-2025 | — | In force | Secondary |
| W2-S21 | 7-year permit exemption incl. hydrogen | MoRTH via Autocar Pro/ChiniMandi | Electric, Ethanol, Hydrogen CVs Get 7 Year Permit Exemption | https://www.autocarpro.in/news/electric-ethanol-hydrogen-cvs-get-7-year-permit-exemption-133500 | 07-2026 (Gazette 6 July) | s.66(1) MV Act | In force | GSR no. not captured |
| W2-S22 | Draft +5 yr age limit (Rule 88) | MoRTH via Business Standard/FPJ | Govt proposes 5-yr age extension for EV, CNG, hydrogen CVs | https://www.business-standard.com/industry/auto/govt-proposes-5-yr-age-extension-for-ev-cng-hydrogen-commercial-vehicles-126081800231_1.html | 18-08-2026 | Rule 88 | Draft | Secondary |
| W2-S23 | Draft GCR 2024 GSR 157(E) hydrogen provisions | DPIIT/PESO via TaxGuru, Lexplosion, RenewableWatch | Draft Gas Cylinders (Amendment) Rules, 2024 | https://taxguru.in/corporate-law/draft-gas-cylinders-amendment-rules-2024.html ; https://peso.gov.in/web/en/gas-cylinders-amendment-rules-2024-hydrogen | 05-03-2024 | defs; rule 4; Sch. I | Draft (superseded by 2025 rules) | Blocked |
| W2-S24 | GCR 2025 GSR 225(E): H2 cylinder on vehicle; 3-yr test; 15-yr life; codes | DPIIT via Lexplosion, LegalitySimplified, TeamLease, Sparrow RMS | Gas Cylinders (Amendment) Rules, 2025 | https://lexplosion.in/ministry-of-commerce-and-industry-notifies-the-gas-cylinders-amendment-rules-2025/ ; https://www.legalitysimplified.com/gas-cylinders-amendment-rules-2025/ ; https://sparrowrms.in/sparrow-rms/wp-content/uploads/2025/04/GCR-amendment-2025-1.pdf | 11-04-2025 | rule 2, 4, 35(?), Sch. I/IV | In force | Blocked; clause numbers unverified |
| W2-S25 | GSR 386(E), 501(E), 839(E) 2025 amendments | DPIIT via Worldtradescanner, LegalitySimplified, TeamLease | GCR Third/Fourth Amendment Rules 2025 | https://worldtradescanner.com/DPIIT%20Notification%20G.S.R.%20501(E)%20dated%2028%20July,%202025.htm ; https://worldtradescanner.com/DPIIT%20Notification%20G.S.R.%20839(E)%20dated%2012%20November%202025.htm | 16-06 / 28-07 / 12-11-2025 | rule 4 | In force | Secondary |
| W2-S26 | SMPV(U) 2025 amendment hydrogen | PESO; TaxGuru | SMPV(Unfired)(Amendment) Rules, 2025 | https://peso.gov.in/web/en/smpvunfiredamendment-rules-2025 ; https://taxguru.in/corporate-law/static-mobile-pressure-vessels-unfired-amendment-rules-2025.html | eff. 05-06-2025 | defs; Form AS-1 | In force | Blocked |
| W2-S27 | PESO Form-H online module for H2 dispensing stations | IndiaLaw; LegalitySimplified; VahanBazaar | PESO's New Online Module for Licensing Compressed Hydrogen Dispensing Stations | https://www.indialaw.in/blog/civil/pesos-module-hydrogen-stations-in-india/ | 14-05-2025 | GCR Form H | Current | Secondary |
| W2-S28 | PESO SOPs (SMPV 02-12-2024; GCR Jan 2025) | PESO | SOP for SMPV(U) Rules 2016; GCR SOP | https://peso.gov.in/web/sites/default/files/2024-12/SOP%20for%20SMPV(U)%20Rules,%202016-Final%20-%2002.12.2024.pdf ; https://www.peso.gov.in/web/sites/default/files/2025-01/GCR%20SOP.pdf | 2024–25 | — | Current | Blocked |
| W2-S29 | PESO licence to IOCL Faridabad for H-CNG dispensing | Business Standard | PESO grants permission to Indian Oil to set up hydrogen plant | https://www.business-standard.com/content/b2b-chemicals/peso-grants-permission-to-indian-oil-to-set-up-hydrogen-plant-115051100485_1.html | 05-2015 | — | Historical | Secondary |
| W2-S30 | Type IV H2 cylinder PESO approvals (Time Technoplast) | Business Standard; Autocar Pro; company PR | Time Technoplast gets final approval from PESO…; 250 L approval | https://www.business-standard.com/markets/capital-market-news/time-technoplast-gets-final-approval-from-peso-for-manufacturing-composite-cylinders-124062100121_1.html ; https://www.timetechnoplast.com/pdf/investors-calendar/press-release/approval-for-manufacturing-of-type-iv-composite-cylinders-for-hydrogen.pdf | 03-2024 / 06-2024 / 01-2026 | — | Current | Company-reported |
| W2-S31 | Indian hydrogen standards landscape (AIS 195A, AIS 206, IS 19036, IS 16061, IS 18538-8) | MNRE with CEEW (PIB-hosted) | Green Hydrogen Standards and Approval Systems in India | https://static.pib.gov.in/WriteReadData/specificdocs/documents/2024/may/doc2024510336201.pdf ; https://nghm.mnre.gov.in/admin/uploads/resources/171557269282523Green%20Hydrogen%20Standards%20and%20Approval%20Systems%20in%20India.pdf | 05-2024 | tables | Current | Blocked |
| W2-S32 | BIS hydrogen standards list (23) | BIS | List of Indian Standards related to Hydrogen | https://www.bis.gov.in/wp-content/uploads/2023/10/List_Standards_Hydrogen.pdf | 10-2023 | — | Current | Blocked |
| W2-S33 | MNRE transport pilot guidelines | MNRE; JSA; Business Standard | Scheme Guidelines for pilot projects … green hydrogen in the transport sector | https://mnre.gov.in/en/notice/scheme-guidelines-for-implementation-of-pilot-projects-for-use-of-hydrogen-in-the-transport-sector-under-the-national-green-hydrogen-mission-nghm/ | 14-02-2024 | objectives; steering committee | In force | Blocked |
| W2-S34 | 5 pilots, 37 vehicles, 9 HRS, 10 routes, Rs 208 cr | Lok/Rajya Sabha reply via DownToEarth, DD News, Elets | As told to Parliament (25 March 2025) | https://www.downtoearth.org.in/environment/as-told-to-parliament-march-25-2025-37-cross-country-vehicles-9-fuelling-stations-planned-under-green-hydrogen-mission ; https://ddnews.gov.in/en/india-rolls-out-pilot-projects-for-hydrogen-powered-buses-and-trucks/ | 25-03-2025 | — | Current | Secondary |
| W2-S35 | Mirai pilot with ICAT | Zee News; BusinessToday | Gadkari inaugurates pilot project … Toyota Mirai | https://zeenews.india.com/electric-vehicles/nitin-gadkari-inaugurates-pilot-project-to-test-viability-of-hydrogen-vehicles-in-india-2445693.html | 16-03-2022 | — | Historical | Press |
| W2-S36 | Tata H2 truck trials, 16 vehicles, 24 months | Tata Motors; News on AIR | Tata Motors Drives India's Green Future with Country's First Hydrogen Truck Trials | https://www.tatamotors.com/press-releases/tata-motors-drives-indias-green-future-with-countrys-first-hydrogen-truck-trials/ ; https://www.newsonair.gov.in/first-trials-of-hydrogen-powered-heavy-duty-trucks-flagged-off | 04-03-2025 | — | Current | Company/press |
| W2-S37 | ISO 14687 grades; 2025 edition | ISO | ISO 14687:2019 / ISO 14687:2025 Hydrogen fuel quality — Product specification | https://www.iso.org/standard/69539.html ; https://www.iso.org/standard/82660.html | 2019 / 2025 | Table 1 grades | 2025 current | Blocked; abstract |
| W2-S38 | ISO 17268 pressure classes | ISO | ISO 17268:2020; ISO 17268-1:2025 | https://www.iso.org/standard/68442.html | 2020 / 2025 | scope | Current | Abstract |
| W2-S39 | SAE J2601 scope, edition | H2Tools (PNNL) | SAE J2601 Fueling Protocols for Light Duty Gaseous Hydrogen Surface Vehicles | https://h2tools.org/fuel-cell-codes-and-standards/sae-j2601-fueling-protocols-light-duty-gaseous-hydrogen-surface | 4th ed. 05-2020 | — | Current (revision open) | Verbatim page text captured |
| W2-S40 | ISO 19881:2025; ISO 19880-1/-2 | ISO; iTeh; H2Tools | ISO 19881:2025; ISO/AWI 19880-1; ISO 19880-2:2025 | https://www.iso.org/standard/19881 ; https://www.iso.org/standard/93897.html ; https://www.iso.org/obp/ui/en/#!iso:std:85839:en | 2025 | scope | Current | Abstracts |
| W2-S41 | GTR 13 Amend. 1 (2023); R134 02 series | UNECE; InterRegs; GAR | ECE/TRANS/180/Add.13/Amend.1 | https://unece.org/sites/default/files/2023-07/ECE-TRANS-180-Add.13-Amend1e.pdf ; https://www.interregs.com/articles/spotlight/297/updated-un-ece-regulations-relating-to-hydrogen-fuelled-vehicles-published | 21-06-2023 | — | In force | Blocked |
| W2-S42 | GTR 13 numeric limits (4 %/8 %; leak; single failure) | Frontiers in Energy Research | Analysis of safety technical standards for hydrogen storage in fuel cell vehicles | https://www.frontiersin.org/journals/energy-research/articles/10.3389/fenrg.2024.1376739/full | 2024 | — | — | Review; "118 NL/h" vs NL/min discrepancy |
| W2-S43 | H2 properties | H2Tools (PNNL) | Basic Hydrogen Properties Chart; Flammability; Auto-Ignition | https://h2tools.org/basic-hydrogen-properties-chart ; https://h2tools.org/faq-keywords/flammability | — | — | Current | Blocked; summary |
| W2-S44 | H2 vs other fuels (relative) | US DOE EERE | Hydrogen: Similar but Different (fact sheet) | https://www1.eere.energy.gov/hydrogenandfuelcells/pdfs/h2_safety_fsheet.pdf | — | — | Current | Blocked |
| W2-S45 | NFPA 2 repair-garage ventilation | NFPA via UpCodes; NFPA | NFPA 2 (2023) Hydrogen Technologies Code — repair garages | https://up.codes/s/repair-garages-for-vehicles-fueled-by-lighter-than-air-motor-fuels ; https://www.nfpa.org/product/nfpa-2-hydrogen-technologies-code/p0002code | 2023 ed. | Ch. 6; repair garages | Current | Secondary |
| W2-S46 | ISO/TR 15916:2015 → ISO/TS 15916:2026 | ISO | Basic considerations for the safety of hydrogen systems | https://www.iso.org/standard/56546.html ; https://www.iso.org/standard/89423.html | 2015 / 2026 | — | TS current | Abstract |
| W2-S47 | UK guidance (HSG261; hydrogen hub; withdrawn C&U note) | HSE; GOV.UK | HSG261; Hydrogen Safety; Construction and use of hydrogen-powered vehicles [withdrawn] | https://www.hse.gov.uk/pubns/books/hsg261.htm ; https://solutions.hse.gov.uk/safe-net-zero/hydrogen-safety ; https://www.gov.uk/government/publications/construction-and-use-of-hydrogen-powered-vehicles/construction-and-use-of-hydrogen-powered-vehicles | — | — | HSG261 current; GOV.UK note withdrawn | Blocked |
| W2-S48 | Kjørbo root cause | Nel ASA; gasworld; Gexcon | Status and Q&A regarding the Kjørbo incident; Root cause … identified | https://nelhydrogen.com/status-and-qa-regarding-the-kjorbo-incident/ ; https://www.gasworld.com/story/root-cause-of-norway-hydrogen-station-explosion-identified/ | 06/07-2019 | — | — | Company statements |
| W2-S49 | Santa Clara causes | Air Products (via H2Tools); Gexcon | Report on the June 2019 Hydrogen Explosion and Fire Incident in Santa Clara | https://h2tools.org/sites/default/files/2021-06/AP_Santa_Clara_Incident_Review_Report_Rev1.pdf | 2021 | — | — | Blocked; summary |
| W2-S50 | Gangneung 2019; Busan 2024; HIAD | ScienceDirect (IJHE); CleanTechnica | Evaluating hydrogen risk management policy PR: Lessons learned from three hydrogen accidents in South Korea | https://www.sciencedirect.com/science/article/abs/pii/S0360319923012132 ; https://cleantechnica.com/2025/01/16/korean-hydrogen-explosions-leaks-auction-failure-impeachment-may-cause-pivot/ | 2023 / 2025 | — | — | Abstract/press |
| W2-S51 | Euro NCAP Mirai 2021; Nexo 2018 | Euro NCAP; Hyundai | Toyota Mirai — Euro NCAP 2021 results — 5 stars; Hyundai Nexo 5-star | https://www.euroncap.com/en/results/toyota/mirai/43501 ; https://www.hyundai.com/worldwide/en/newsroom/detail/all-new-hyundai-nexo-achieves-maximum-five-star-rating-in-euro-ncap-safety-assessment-0000001089 | 2021 / 2018 | — | — | Blocked; summary |
| W2-S52 | DOE on NOx from hydrogen combustion | US DOE EERE | Does the use of hydrogen produce air pollutants such as nitrogen oxides? | https://www.energy.gov/eere/fuelcells/does-use-hydrogen-produce-air-pollutants-such-nitrogen-oxides | — | — | Current | Requested /cmei/fuels/ URL not found; content from summary |
| W2-S53 | IMT-25; Rs 60 TP loading | India Motor Tariff via GoDigit; IRDAI policy wordings; Acko; HDFC Ergo | India Motor Tariff (IMT) Endorsements — IMT.25 | https://www.godigit.com/content/dam/godigit/directportal/en/downloads/car/Motor-IMT%20Endorsements.pdf ; https://www.hdfcergo.com/blogs/car-insurance/insurance-guide-for-cng-lpg-converted-cars-2025 | — | IMT-25 | Current | Blocked; summary |
| W2-S54 | Form 22D (Rule 47B(1)); RTO documents | MoRTH (Parivahan); Policybazaar; RTOIndiaHelp | FORM 22D — Endorsement of alteration/retrofitment | https://parivahan.gov.in/sites/default/files/DownloadForm/cmvr/FORM-22D.pdf ; https://www.policybazaar.com/rto/form-22d/ | — | Rule 47B(1) | Current | Blocked |
| W2-S55 | Maruti warranty exclusion for retrofitted CNG | Maruti owner's manual (via GoMechanic/RideNRepair) | Dzire Owner's Manual (warranty section) | https://marutistoragenew.blob.core.windows.net/msilintiwebpdf/Dzire_Owners_Manual_1.pdf | — | warranty exclusions | Current | Not verified |
| W2-S56 | Rule 41 trade-certificate purposes | Indian Kanoon; transportinfo.in | The Central Motor Vehicles Rules, 1989 — Rule 41 | https://indiankanoon.org/doc/28072254/ ; https://www.transportinfo.in/simba/cmvr/cmvr.htm | — | Rule 41 | In force | Summary |
| W2-S57 | Rule 126 test agencies | PIB; TÜV | Rule 126 CMVR | https://www.pib.gov.in/newsite/erelcontent.aspx?relid=279664 ; https://www.tuv.com/market-access-services/en/certification-filter/cmvr-(central-motor-vehicle-rules-notified-by-the-government-of-india).html | — | Rule 126 | In force | Summary |
| W2-S58 | AIS-137 (Part 9) draft Amd 1 for CNG/LPG retrofit | Scribd (regulatory update) | Regulatory Updates: CMVR Nov–Dec 2022 | https://www.scribd.com/document/930900675/Regulatory-Update-Nov-Dec-2022 | 12-2022 | — | Draft | Fragment |
| W2-S59 | CMVR-TSC minutes (ARAI hydrogen projects) | ACMA | MoM 62nd CMVR-TSC | https://www.acma.in/uploads/docmanager/MoM_62_CMVR-TSC.pdf | — | — | — | Blocked |
| W2-S60 | Analyst: H2-ICE retrofit not yet legalised | S&P Global Mobility | Hydrogen trucks in India: green hydrogen potential | https://www.spglobal.com/automotive-insights/en/blogs/2025/03/hydrogen-trucks-in-india-green-hydrogen-potential | 03-2025 | — | — | Opinion |
| W2-S61 | CV OEM hydrogen shift | Business Standard | India's vehicles industry quietly begins shift to hydrogen-powered vehicles | https://www.business-standard.com/industry/auto/indian-cv-majors-drive-hydrogen-transition-with-h2-vehicles-125061600725_1.html | 16-06-2025 | — | — | Press |
| W2-S62 | Gazette 27-03-2026 doc citing AIS 157:2020 | Gazette of India via Scribd | CG-DL-E-27032026-… | https://www.scribd.com/document/1019065447/271279 | 27-03-2026 | — | Unknown | Content unresolved |
| W2-S63 | MNRE NGHM regulations directory | MNRE | National Green Hydrogen Mission Portal — Regulations | https://nghm.mnre.gov.in/regulations?language=en | — | — | Current | Blocked |
| W2-S64 | State regulation of retrofit centres | Kerala MVD | Conversion of vehicles into CNG/LPG mode — Regulation and control of Retro fitment centres | https://mvd.kerala.gov.in/en/node/1386 | — | — | — | Blocked |
| W2-S65 | NCR age restrictions after CNG conversion | Delhi Govt (Switch Delhi) / press | Switch Delhi — retro-fitment | https://ev.delhi.gov.in/retro-fitment | — | — | — | Secondary |
| W2-S66 | 87 standards adopted / 59 under development | CEEW | How can India fix gaps for green hydrogen safety standards? | https://www.ceew.in/publications/how-can-india-adopt-international-standards-for-national-green-hydrogen-mission | 2024 | — | — | Think-tank |
| W2-S67 | PESO hydrogen presentations at PNGRB | PESO via PNGRB | Statutory rules/regulations for CNG stations (PESO); Stakeholders interaction on hydrogen | https://www.pngrb.gov.in/pdf/press-note/PESO_15032024.pdf ; https://pngrb.gov.in/pdf/press-note/20250306_PESO.pdf | 03-2024 / 03-2025 | — | — | Blocked |
| W3-S1 | 72.10 lakh CNG vehicles registered | AIR News (newsonair.gov.in) reporting Lok Sabha written reply, MoS PNG Suresh Gopi | Total sales of CNG vehicles have increased over the past few years: Suresh Gopi | https://www.newsonair.gov.in/total-sales-of-cng-vehicles-have-increased-over-the-past-few-years-suresh-gopi |  |  | observed (Vahan) | page blocked; figure from search passage |
| W3-S2 | 58.61 lakh (Mar 2023) → 81.95 lakh (Mar 2025) CNG vehicles; +25% FY25; PNGRB VAT plea | ET Energyworld / UNI / The Hindu BusinessLine / Energywatch quoting PNGRB | PNGRB expands CGD network to 784 districts, sets 2034 target of 12.6 crore PNG connections; City Gas Distribution network now covers nearly 100 pc…; PNGRB in talks with states to rationalise VAT | https://energy.economictimes.indiatimes.com/news/oil-and-gas/pngrb-expands-cgd-network-to-784-districts-sets-2034-target-of-12-6-crore-png-connections/123553997 ; https://www.thehindubusinessline.com/markets/commodities/pngrb-in-talks-with-states-to-rationalise-vat-on-natural-gas-cng/article69991018.ece |  |  | observed (PNGRB citing Vahan) | secondary reporting of PNGRB statement |
| W3-S3 | 18,336 CNG stations & 12.63 crore PNG by 2034; ~1,206 stations added FY25 | Business Standard; IBEF | PNGRB targets 12.6 crore PNG connections, 18,336 CNG stations by 2034 | https://www.business-standard.com/economy/news/pngrb-targets-12-6-crore-png-connections-18336-cng-stations-by-2034-125082700329_1.html ; https://www.ibef.org/news/india-eyes-126-million-png-connections-18-000-cng-stations-by-2034 |  |  | target / observed | secondary |
| W3-S4 | 738 (2014) → 8,150+ CNG stations; 18,000 by 2030 | Hardeep Singh Puri (MoPNG) on X; The Hindu BusinessLine | One Nation, One Gas Grid (X post); India to double CNG stations to 18,000 by 2030 amid One Nation One Gas Grid plan: Puri | https://x.com/HardeepSPuri/status/1961324393344491893 ; https://www.thehindubusinessline.com/economy/logistics/india-to-double-cng-stations-to-18000-by-2030-amid-one-nation-one-gas-grid-plan-puri/article69988711.ece |  |  | observed (ministerial) / target | ministerial statement, not a PNGRB table |
| W3-S5 | outdated "2,830 stations" figure | PRS Legislative Research | National Gas Grid Including PNG and CNG (Standing Committee report summary) | https://prsindia.org/policy/report-summaries/national-gas-grid-including-png-and-cng |  |  | outdated | used only to flag staleness |
| W3-S6 | 17,700 stations by 2030 (MWP, rounds to 11A); "17,500 by 2030"; "7,400 in 2025" | Autocar Professional; Construction World | India's CNG infra development in high gear; India Plans 17,500 CNG-PNG Stations… by 2030 | https://www.autocarpro.in/feature/indias-auto-industry-set-for-a-cng-boom-as-gas-infra-inches-to-100-penetration-117791 ; https://www.constructionworld.in/policy-updates-and-economic-news/india-plans-17500-cng-png-stations-aims-for-120-million-connections-by-2030/52110 |  |  | commitment/target | industry press citing MoPNG/PNGRB |
| W3-S7 | CNG ₹86.98/kg Delhi (IGL, +₹3.89); ₹88/kg Mumbai (MGL, +₹2); reasons | Goodreturns; Equentis; ETV Bharat; Infralog | CNG Price in New Delhi Today; CNG, PNG Price Hike: New Rates in Mumbai and Delhi-NCR; CNG Price Hiked By Rs 2 Per Kg In Mumbai; MGL hikes CNG… amid rising spot RLNG costs | https://www.goodreturns.in/cng-price-in-new-delhi.html ; https://www.equentis.com/blog/cng-png-price-hike-new-rates-in-mumbai-and-delhi-ncr/ ; https://www.etvbharat.com/en/business/cng-price-hiked-by-rs-2-per-kg-in-mumbai-enn26090104129 |  |  | observed (retail) | aggregator pages; IGL/MGL notices not opened |
| W3-S8 | Pune CNG ₹97.75/kg from 1 Sep 2026 | MNGL (official rate card); Times of India | CNG Rate Card — Revision in Prices w.e.f. midnight of 1st Sep 2026: Pune GA Rs 97.75/kg; MNGL hikes CNG price by Rs2 per kg in Pune | https://mngl.in/cng/rate-card ; https://timesofindia.indiatimes.com/city/pune/mngl-hikes-cng-price-by-rs2-per-kg-in-pune/articleshow/133662320.cms |  |  | observed | primary text visible in passage |
| W3-S9 | Ahmedabad CNG ₹94.02/kg | CarDekho / Goodreturns city price pages | CNG price in India Today | https://www.cardekho.com/cng-price ; https://www.goodreturns.in/cng-price.html |  |  | observed (aggregator) | supplier notice not verified |
| W3-S10 | Maruti FY25 CNG 6.12–6.2 lakh, 33%; 71.6% CNG-segment share (2024) | Autocar India; Autocar Professional | One in every three Maruti sold in FY2025 was a CNG; One-in-Three Maruti Vehicles Sold in FY25 Ran on CNG | https://www.autocarindia.com/industry/one-in-every-three-maruti-sold-in-fy2025-was-a-cng-434994 ; https://www.autocarpro.in/news/one-in-three-maruti-vehicles-sold-in-fy25-ran-on-cng-125695 |  |  | observed (company/industry data) | secondary |
| W3-S11 | Maruti FY26 sales 24.22 lakh, exports 4.47 lakh; biogas plants ₹561 crore | Maruti Suzuki (press releases, not accessible); Automotive World; scanx | Maruti Suzuki announces Financial Results for FY2025-26; Annual Integrated Report 2025-26 | https://www.marutisuzuki.com/corporate/media/press-releases/2026/april/maruti-suzuki-announces-financial-results-for-fy2025-26 ; https://www.marutisuzuki.com/corporate/media/press-releases/2026/august/maruti-suzuki-annual-integrated-report-2025-26 ; https://www.automotiveworld.com/news/annual-integrated-report-maps-maruti-suzukis-growth-push/ |  |  | observed | primary blocked; via secondary |
| W3-S12 | Maruti FY26 CNG >7 lakh; 42% share "last month"; FY27 target 9 lakh | ET Auto; Business Standard; Moneycontrol | Maruti Suzuki targets nearly 30% jump in CNG sales to 9 lakh units in FY27; Maruti Suzuki targets 900,000 units of CNG vehicle sales this fiscal | https://auto.economictimes.indiatimes.com/news/passenger-vehicle/maruti-suzuki-aims-for-30-increase-in-cng-vehicle-sales/132637759 ; https://www.business-standard.com/companies/news/maruti-suzuki-targets-900-000-units-of-cng-vehicle-sales-this-fiscal-126072600153_1.html |  |  | observed / target | 42% is a monthly figure |
| W3-S13 | PV fuel mix FY26: petrol 52.6% (24.61 of 46.83 lakh), CNG 21.7%; Jul 2026 CNG 24.67% | Autopunditz (Vahan-based analyst) | Passenger Cars Fuel-wise Sales Analysis – India FY2026 | https://www.autopunditz.com/post/india-passenger-vehicle-fuelwise-sales-analysis-fy2026 |  |  | observed (analyst compilation) | not SIAM; CNG % seen via social summary |
| W3-S14 | SIAM quarterly highlights FY26 (not accessible) | SIAM | Key Highlights from Q1/Q2 FY2025-26 | https://www.siam.in/pressrelease-details.aspx?mpgid=48&pgidtrail=50&pid=583 ; …pid=591 |  |  | — | blocked; no CNG split retrieved |
| W3-S15 | "4,000+ authorised installation centres"; aftermarket ₹2,500–3,500 crore/yr; brand shares | LetsGO CNG (vendor blog) | CNG Conversion Statistics India 2026: 50 Key Facts | https://letsgocng.in/cng-conversion-statistics-india-2026/ |  |  | vendor estimate | unofficial; low confidence |
| W3-S16 | RFC registries (Vahan CNG Maker portal; IGL list; Delhi RFC application) | MoRTH Vahan; IGL; Delhi Transport Dept | Available Fitment Center (CNG Maker); RTO Approved CNG Kit Retrofit Centres; Application for CNG Retro Fitment | https://vahan.parivahan.gov.in/cngmaker/vahan/user/retrofitment/availableFitmentCenter.xhtml ; https://www.iglonline.net/cng-retrofitment-center ; https://transport.delhi.gov.in/transport/application-cng-retro-fitment |  |  | registry | no totals retrievable |
| W3-S17 | Delhi aggregator scheme EV milestones; notified 21 Nov 2023 | Legitquest (text reproduction); Hindustan Times; Delhi Transport Dept (X); Medianama | Delhi Motor Vehicle Aggregator and Delivery Service Provider Scheme, 2023 | https://www.legitquest.com/act/delhi-motor-vehicle-aggregator-and-delivery-service-provider-scheme-2023/D483 ; https://www.hindustantimes.com/cities/delhi-news/delhi-govt-notifies-cab-aggregator-policy-fleets-to-switch-to-electric-101701281802617.html ; https://x.com/TransportDelhi/status/1730101116590059896 |  |  | rule | 6-month/1-year milestones only via HT |
| W3-S18 | ~60,000 taxis registered in Delhi; CNG/clean-fuel taxi permits 15 years; SC CNG requirement | Outlook India | Delhi: All Taxis Running On CNG, Other Clean Fuels To Have Permit Validity Of 15 Years | https://www.outlookindia.com/national/delhi-all-taxis-running-on-cng-other-clean-fuels-to-have-permit-validity-of-15-years-news-296571 |  |  | observed (dept data, undated) | secondary |
| W3-S19 | CAQM: only CNG/EV new aggregator/delivery vehicles in NCR from 1 Jan 2026; Haryana rules | Deccan Herald; Free Press Journal; Swarajya | No new petrol or diesel vehicles for cab aggregators, delivery firms in NCR from January 2026: CAQM; Haryana Cabinet Mandates CNG, EV Or Cleaner Fuel For All New Aggregator Vehicles In NCR From 2026 | https://www.deccanherald.com/amp/story/india%2Fno-new-petrol-or-diesel-vehicles-for-cab-aggregators-delivery-firms-in-ncr-from-january-2026-caqm-3571433 ; https://www.freepressjournal.in/amp/india/haryana-cabinet-mandates-cng-ev-or-cleaner-fuel-for-all-new-aggregator-vehicles-in-ncr-from-2026 |  |  | rule | secondary |
| W3-S20 | Delhi 8.76 m vehicles (19 Mar 2026); taxis limited growth; e-buses 4,338 | The Wire (reporting Delhi Economic Survey 2025-26); Delhi Planning Dept PDF (blocked) | Service Sector, Transport, Water: Highlights of Delhi's Economic Survey 2025-26 | https://m.thewire.in/article/economy/service-sector-transport-water-highlights-of-delhis-economic-survey-2025-26/amp ; https://delhiplanning.delhi.gov.in/sites/default/files/2026-03/economic_survey_english_0.pdf |  |  | observed | PDF blocked |
| W3-S21 | 60-driver Delhi study; longer hours | Springer (book chapter) | Are Ride-Sharing Platforms Good for Indian Drivers? An Investigation of Taxi and Auto-Rickshaw Drivers in Delhi | https://link.springer.com/chapter/10.1007/978-3-030-64697-4_10 |  |  | observed (survey) | small sample |
| W3-S22 | ICCT ride-hailing TCO; BEV ₹8.8→5.8/km (2019→2020); 2019 not competitive | ICCT | Updated analysis of incentives for electrifying India's four-wheeler ride-hailing fleet; Are electric four-wheelers becoming more cost attractive as ride-hailing vehicles in India?; (vendor rule-of-thumb: Zevpoint EV fleet charging guide) | https://theicct.org/publication/updated-analysis-of-incentives-for-electrifying-indias-four-wheeler-ride-hailing-fleet/ ; https://theicct.org/are-electric-four-wheelers-becoming-more-cost-attractive-as-ride-hailing-vehicles-in-india/ ; https://www.zevpoint.com/blogs/guides/ev-fleet-charging-solutions-taxi-logistics-india |  |  | modelled | PDFs blocked; daily-km assumption not seen |
| W3-S23 | BluSmart collapse; SEBI order 15 Apr 2025; ₹978 crore diversion; leasing model | Business Today; Down To Earth; Zentrum Law | SEBI turns heat on Gensol, BluSmart's house of cards wobbles; What BluSmart got wrong—and right | https://www.businesstoday.in/latest/corporate/story/sebi-turns-heat-on-gensol-blusmarts-house-of-cards-wobbles-472292-2025-04-16 ; https://www.downtoearth.org.in/air/what-blusmart-got-wrongand-right |  |  | observed | secondary |
| W3-S24 | Everest (20,500 cars, ~2,000 EVs, 10,000 EV target); Refex 2,000+ EVs; Evera 4,500 by Jun 2026; fleet constraints | Business Standard; EVreporter; EV Drive | Next stop for BluSmart's EVs could be Everest Fleet; India's Leading Electric 4-Wheeler Passenger Fleet Operators; Evera Fleet Expansion 4500 EVs by June 2026 | https://www.business-standard.com/companies/start-ups/everest-fleet-in-talks-to-acquire-blusmart-evs-for-uber-green-125051201066_1.html ; https://evreporter.com/indias-leading-electric-4-wheeler-passenger-fleet-operators/ ; https://evdrive.in/evera-fleet-expansion-4500-electric-vehicles/ |  |  | observed / announced | company statements |
| W3-S25 | Uber–Tata 25,000 Xpres-T EVs; Xpres-T price ₹13.04 lakh | Autocar India; Tata Motors Xpres-T site | Tata Motors to supply 25,000 XPRES-T EVs in largest ever fleet order; Tata XPRES-T EV Price | https://www.autocarindia.com/car-news/tata-motors-to-supply-25000-xpres-t-evs-in-largest-ever-fleet-order-427364 ; https://xprest.tatamotors.com/electric/price |  |  | announced / list price | delivery status unknown |
| W3-S26 | used Dzire CNG from ₹3.71 lakh (Delhi); WagonR ₹1.6–6.0 lakh; new Dzire Tour S on-road ₹7.35–8.33 lakh | Cars24; Orange Book Value; CarDekho | Second Hand Maruti Dzire CNG Cars in New Delhi; Used Wagon R price and valuation; Dzire Tour S On Road Price in New Delhi | https://www.cars24.com/buy-used-maruti-dzire-cng-cars-new-delhi/ ; https://orangebookvalue.com/used-cars/maruti-suzuki-wagon-r-price ; https://www.cardekho.com/maruti/dzire-tour-s/price-in-new-delhi |  |  | listings (indicative) | not valuations |
| W3-S27 | Tata EV prices/specs (Tiago, Punch, Nexon, Tigor) | Tata.ev; ZigWheels; Team-BHP; V3Cars | Tiago EV / Punch EV / Nexon EV price pages; Tigor EV 2026 | https://ev.tata.cars/tiago/ev/price.html ; https://ev.tata.cars/punch/ev/price.html ; https://ev.tata.cars/nexon/ev/price.html ; https://www.v3cars.com/tata-cars/tigor-ev ; https://www.zigwheels.com/tata-cars/tiago-ev/ |  |  | list price / ARAI | ex-showroom, variant-dependent |
| W3-S28 | MG Comet, Windsor prices/specs | JSW MG Motor India; ZigWheels | MG Comet EV page; MG Windsor EV 2026 | https://www.mgmotor.co.in/vehicles/comet-ev-electric-car-in-india ; https://www.zigwheels.com/mg-motor-cars/windsor-ev/ |  |  | list price | — |
| W3-S29 | e Vitara, Creta Electric, eC3X prices/specs | Autocar India; CarWale/Team-BHP | Price and specs of Maruti e Vitara, Hyundai Creta Electric, MG Windsor EV… compared; 2026 Citroen eC3X launched at Rs 11.99 lakh; Hyundai Creta Electric | https://www.autocarindia.com/auto-features/maruti-e-vitara-vs-rivals-prices-compared-440249 ; https://www.autocarindia.com/car-news/2026-citroen-ec3x-launched-at-rs-1025-lakh-439987 ; https://www.carwale.com/hyundai-cars/creta-electric/ |  |  | list price | — |
| W3-S30 | real-world range/efficiency tests | Autocar India; ZigWheels | Real world EV range tested of top EV cars in India; Tata EVs and their real-world range | https://www.autocarindia.com/auto-features/every-mass-market-ev-real-world-range-tested-434105 ; https://www.zigwheels.com/news-features/ev-news/heres-a-look-at-all-the-tata-evs-and-their-real-world-range/54737/ |  |  | observed (media test) | single-test conditions |
| W3-S31 | Tata battery warranty (8 yr/1.6 lakh km; lifetime for Curvv.ev/Nexon.ev 45) | Tata Motors; Tata.ev; Spinny | Tata Motors introduces Lifetime HV Battery Warranty…; Tata EV warranty page; Tata EV Warranty Explained | https://www.tatamotors.com/press-releases/reimagining-ev-ownership-tata-motors-introduces-lifetime-hv-battery-warranty-for-curvv-ev-and-nexon-ev-45-kwh/ ; https://ev.tata.cars/service/warranty.html ; https://www.spinny.com/blog/tata-ev-warranty/ |  |  | company terms | fleet terms differ |
| W3-S32 | Delhi EV tariff ₹4.50/kWh LT, ₹4.00/kVAh HT; DERC PM E-DRIVE order | Mercom India; thediscombill; EMobility+ | Delhi Slashes Tariffs for EV Charging; Delhi Electricity Tariff 2025-26; DERC directs EV charging infra costs… borne by operators | https://www.mercomindia.com/delhi-tariffs-ev-charging-electric-mobility ; https://thediscombill.com/tariffs/delhi/ ; https://emobilityplus.com/2026/07/03/derc-directs-ev-charging-infrastructure-costs-under-pm-e-drive-scheme-to-be-borne-by-operators-not-consumers-in-delhi/ |  |  | tariff order (secondary) | DERC order not opened |
| W3-S33 | public DC ₹18–25/kWh; AC ₹8–18; Jio-bp ₹15.99 | Pulse Energy; Team-BHP | EV Charging Rates in India 2026; Pricing of just Rs 15.99 per kWh — Jio BP | https://pulseenergy.io/blog/ev-charging-rates ; https://www.team-bhp.com/forum/electric-cars/273196-pricing-just-rs-15-99-per-kwh-will-jio-bp-disrupt-ev-charging-market.html |  |  | observed (operator prices) | vendor blog |
| W3-S34 | GST: EV 5%; small cars 18%; large 40% (GST 2.0, Sep 2025) | Bajaj Finserv; The Munim | GST on Vehicles in 2026; GST on Electric Vehicles in India 2026 | https://www.bajajfinserv.in/gst-on-vehicles ; https://themunim.com/blog/gst-on-electric-vehicles/ |  |  | rule | secondary explainer |
| W3-S35 | Delhi EV Policy 2026 (w.e.f. 1 Jul 2026) provisions | Business Standard; YourStory; EV Infrastructure News | Delhi govt notifies EV Policy 2026, rolls out subsidies, tax exemptions; Delhi government notifies Delhi EV Policy 2026 | https://www.business-standard.com/india-news/delhi-govt-notifies-ev-policy-2026-rolls-out-subsidies-tax-exemptions-126070100488_1.html ; https://yourstory.com/2026/07/delhi-government-notifies-delhi-ev-policy-2026 ; https://www.evinfrastructurenews.com/ev-policies/delhi-notifies-ev-policy-2026-with-tax-exemptions-for-ev |  |  | rule (notified) | gazette not opened |
| W3-S36 | 5 pilot projects: 37 vehicles, 9 HRS, 10 routes, ₹208 crore; awardees; ARAI SIA | PIB (PRID 2107795, blocked); Down To Earth; DD News; Energetica India; Swarajya | As told to Parliament (March 25, 2025): 37 cross-country vehicles, 9 fuelling stations…; India rolls out pilot projects for hydrogen-powered buses and trucks; MNRE Initiates 5 Pilot Projects… | https://www.pib.gov.in/PressReleasePage.aspx?PRID=2107795 ; https://www.downtoearth.org.in/environment/as-told-to-parliament-march-25-2025-37-cross-country-vehicles-9-fuelling-stations-planned-under-green-hydrogen-mission ; https://ddnews.gov.in/en/india-rolls-out-pilot-projects-for-hydrogen-powered-buses-and-trucks/ ; https://www.energetica-india.net/news/mnre-initiates-5-pilot-projects-for-using-hydrogen-in-buses-and-trucks |  |  | sanctioned | PIB blocked |
| W3-S37 | 12 pilot projects: 70 vehicles (27 buses, 43 trucks), 16 HRS, 21 routes, ₹410.77 crore, ₹41.582 crore released | PIB / MNRE (PRID 2291541, blocked); fiinews; EQ Mag; MNRE minister social posts | Centre Launches 12 Pilot Projects Under National Green Hydrogen Mission for Deployment of 70 Hydrogen-Powered Vehicles Across 21 Routes; Project: 12 NGHM are under implementation | https://www.pib.gov.in/PressReleasePage.aspx?PRID=2291541&reg=48&lang=1 ; https://www.fiinews.com/2026/08/01/project-12-nghm-are-under-implementation/ ; https://www.eqmagpro.com/centre-launches-12-pilot-projects-under-national-green-hydrogen-mission-for-deployment-of-70-hydrogen-powered-vehicles-across-21-routes-eq/ |  |  | sanctioned / under implementation | PIB blocked; 21-route list not retrieved |
| W3-S38 | SIGHT 8.62 lakh tpa awarded (19 cos); 3,000 MW electrolyser awards; ~8,000 tpa commissioned (Feb 2026); ₹292 crore spent; 30 KTPA to 4 refineries; Tranche II 450,000 tpa; NGHM outlay/target | Business Today (column); Business Standard; ANI; Mercom India | India's green hydrogen ambitions face a reality check; Govt spends ₹292 crore under NGHM so far: MNRE; Centre awards 30 KTPA…; MNRE to Incentivize 450,000 MTPA… | https://www.businesstoday.in/opinion/columns/story/indias-green-hydrogen-ambitions-face-a-reality-check-536276-2026-06-11 ; https://www.business-standard.com/amp/industry/news/govt-spends-292-crore-under-national-green-hydrogen-mission-so-far-mnre-126081201853_1.html ; https://www.aninews.in/news/business/centre-awards-30-ktpa-green-hydrogen-capacity-to-four-oil-refineries-under-national-green-hydrogen-mission20260811180606/ ; https://www.mercomindia.com/mnre-450000-mtpa-green-hydrogen |  |  | awarded vs operating | secondary reporting of MNRE/SECI data |
| W3-S39 | Numaligarh ₹279/kg; SECI green ammonia ₹55.75/kg; SIGHT auction Mar 2025; cost estimates | S&P Global; PIB (PRID 2153006, blocked); Renewable Watch; Reslink (vendor) | India's Numaligarh hydrogen tender ends at record low Rupees 279/kg; SECI conducts first-ever auction for procurement of Green Ammonia; SECI announces results of green hydrogen auction under SIGHT; India Green Hydrogen Price Per Kg 2026 | https://www.spglobal.com/energy/en/news-research/latest-news/energy-transition/021226-indias-numaligarh-hydrogen-tender-ends-at-record-low-rupees-279kg ; https://www.pib.gov.in/PressReleasePage.aspx?PRID=2153006 ; https://renewablewatch.in/2025/03/20/seci-announces-results-of-green-hydrogen-auction-under-sight-programme/ ; https://www.reslink.org/blogs/india-s-green-hydrogen-mission-is-moving-from-paper-to-projects/ |  |  | observed (tender) | ₹397/kg and $5.3–6.7/kg are vendor figures |
| W3-S40 | California retail H2 $32–36/kg; avg $34.55 (Sep 2024); True Zero $36 | S&P Global; Stillwater Associates; hydrogenfuelfinder | California hydrogen pump prices for light-duty vehicles reach new highs; 2025 Cost Showdown for Drivers; Why Hydrogen Fuel Costs $36/kg in 2025 | https://www.spglobal.com/commodity-insights/en/news-research/latest-news/energy-transition/100124-california-hydrogen-pump-prices-for-light-duty-vehicles-reach-new-highs ; https://stillwaterassociates.com/2025-cost-showdown-for-drivers-is-hydrogen-fuel-cheaper-than-gasoline/ ; https://hydrogenfuelfinder.com/blog/hydrogen-fuel-cost-2025-reality-check.html |  |  | observed (retail) | — |
| W3-S41 | Germany €12.95/kg avg; €12.85; Hylane–H2 Mobility €8/kg (2026, trucks) | Atawey; Energy News; Driving Hydrogen; glpautogas | Understanding the Cost and Price of Hydrogen (2026); Hydrogen price goes to €12.85/kg; €8/kg hydrogen enables cost parity with diesel: Hylane & H2 Mobility | https://atawey.com/en/hydrogen-prices-in-2026-cost-per-kg-refuelling-costs-comparisons-and-2030-outlook/ ; https://energynews.biz/hydrogen-price-goes-up-to-e12-85-kg/ ; https://drivinghydrogen.com/2025/11/20/e8-kg-hydrogen-enables-cost-parity-with-diesel-hylane-h2-mobility-milestone/ |  |  | observed / contracted | dates of averages not exact |
| W3-S42 | Japan ¥700/kg subsidy; Tokyo auction ~$23/kg; METI priority regions | Driving Hydrogen; METI; Hydrogen Insight | Japan to introduce ¥700/kg hydrogen fuel subsidy…; First Selection of Priority Regions for FC Commercial Vehicles; Hydrogen prices of $23/kg — Tokyo… auction | https://drivinghydrogen.com/2025/05/30/japan-to-introduce-%C2%A5700-kg-hydrogen-fuel-subsidy-to-kickstart-commercial-uptake/ ; https://www.meti.go.jp/english/press/2025/0519_002.html ; https://www.hydrogeninsight.com/production/hydrogen-prices-of-23-kg-tokyo-city-government-announces-results-for-first-double-sided-green-hydrogen-auction/2-1-1830862 |  |  | policy / auction | pump price NOT retrieved |
| W3-S43 | Standards: 34 standards in 14 months; PESO/OISD/PNGRB/MoRTH/BIS; Gas Cylinder Rules 2016; ISO 19880 harmonisation; CMVR 4th Amdt 2025 H2 plates; MoRTH 18 Aug 2026 draft | PIB (static backgrounder); PNGRB/PESO; autopdp; Business Standard | Green Hydrogen Standards and Approval Systems in India; Stakeholders interaction on pathways for transmission of hydrogen; Automotive Regulation Updates India; Govt proposes 5-yr age extension for EV, CNG, hydrogen commercial vehicles | https://static.pib.gov.in/WriteReadData/specificdocs/documents/2024/may/doc2024510336201.pdf ; https://www.pngrb.gov.in/pdf/press-note/PESO_15032024.pdf ; https://autopdp.com/regulatory-updates ; https://www.business-standard.com/industry/auto/govt-proposes-5-yr-age-extension-for-ev-cng-hydrogen-commercial-vehicles-126081800231_1.html |  |  | rule / draft | PDFs blocked; passages only |
| W3-S44 | NTPC Leh: 5 FC buses; 80 kg/day; 350 bar; 1.7 MW solar; Nov 2024 flag-off; Jun 2025 handover | NTPC; UT Ladakh; pv magazine India; AIR News | NTPC Hands Over Five Hydrogen Fuel Cell Buses to Leh Administration; Union Minister Manohar Lal flags off the fleet of green hydrogen buses of NTPC in Ladakh | https://ntpc.co.in/media/press-releases/ntpc-hands-over-five-hydrogen-fuel-cell-buses-leh-administration ; https://ladakh.gov.in/ntpc-hands-over-five-hydrogen-fuel-cell-buses-to-sidco-at-green-hydrogen-mobility-station-leh/ ; https://www.pv-magazine-india.com/2025/06/19/the-hydrogen-stream-ntpc-hands-over-five-hydrogen-fuel-cell-buses-to-leh-administration/ ; https://www.newsonair.gov.in/union-minister-manohar-lal-flags-off-the-fleet-of-green-hydrogen-buses-of-ntpc-in-ladakh |  |  | operating | costs/H2 price not disclosed |
| W3-S45 | Tata 16-truck trials (4 Mar 2025); 40 H2-ICE trucks (Feb 2026); IOCL HRS at 4 sites | Tata Motors; AIR News; electrive | Tata Motors drives India's green future with country's first hydrogen truck trials; First trials of hydrogen powered heavy duty trucks flagged off; Tata Motors to deploy 40 H2 ICE trucks in India | https://www.tatamotors.com/press-releases/tata-motors-drives-indias-green-future-with-countrys-first-hydrogen-truck-trials/ ; https://www.newsonair.gov.in/first-trials-of-hydrogen-powered-heavy-duty-trucks-flagged-off ; https://www.electrive.com/2026/02/27/tata-motors-to-deploy-40-h2-ice-trucks-in-india/ |  |  | trial / announced | primary blocked |
| W3-S46 | IOCL Faridabad & Vadodara stations; 15-bus trials; Panipat 10 kTPA by Dec 2027; Air Products "first station" | IOCL; Indian Chemical News; Air Products; VahanBazaar (unofficial) | Green Drive: IOC plans hydrogen production units; Panipat green hydrogen unit on track for December 2027 completion; India's First Hydrogen Fuelling Station Inaugurated | https://iocl.com/MediaDetails/59096 ; https://www.indianchemicalnews.com/interviews/panipat-green-hydrogen-unit-on-track-for-december-2027-completion-dr-alok-sharma-director-rd-iocl-551 ; https://investors.airproducts.com/news-releases/news-release-details/indias-first-hydrogen-fuelling-station-inaugurated |  |  | operating (captive) / under construction | dates & pressures not verified |
| W3-S47 | HCNG 50-bus Delhi trial (20 Oct 2020); 70% CO / 25% HC reduction; NOx same | fuelcellsworks; Busworld; IOCL media; pv magazine India | Shri Pradhan inaugurates H-CNG plant and launches trials in Delhi of 50 hydrogen-powered buses; Trial run of H-CNG powered city buses to begin in Delhi Oct 20; Hydrogen CNG, biogas: the many faces of new auto fuel | https://fuelcellsworks.com/news/india-shri-pradhan-inaugurates-h-cng-plant-and-launches-trials-in-delhi-of-50-hydrogen-powered-buses ; https://news.busworld.org/article/11296/trial-run-of-hcng-powered-city-buses-to-begin-in-delhi-oct-20 ; https://www.iocl.com/MediaDetails/56688 ; https://www.pv-magazine-india.com/2020/10/21/pollution-hit-delhi-starts-hydrogen-enhanced-cng-trial-for-buses/ |  |  | pilot (IOCL-reported) | formal report not found |
| W3-S48 | Ashok Leyland–Reliance H2-ICE truck; launch in 18–24 months (Oct 2024) | Business Standard; Rushlane | Ashok Leyland to launch first H2-powered truck in the next 2 years; Ashok Leyland, Reliance Unveil India's First Hydrogen-ICE Powered Heavy Duty Truck | https://www.business-standard.com/companies/news/ashok-leyland-to-launch-first-h2-powered-truck-in-the-next-2-years-124100700992_1.html ; https://www.rushlane.com/first-hydrogen-ice-powered-heavy-duty-truck-12460051.html |  |  | prototype / announced | launch not verified |
| W3-S49 | Toyota Mirai–ICAT pilot, Mar 2022 | Autocar Professional; India TV | Toyota Kirloskar, iCAT begin pilot project with Toyota Mirai; Nitin Gadkari drives India's first green hydrogen-powered car to Parliament | https://www.autocarpro.in/news-national/toyota-kirloskar-icat-begin-pilot-project-with-toyota-mirai-81384 ; https://www.indiatvnews.com/news/india/nitin-gadkari-drives-green-hydrogen-powered-car-to-parliament-toyota-mirai-pics-2022-03-30-766664 |  |  | pilot | — |
| W3-S50 | TVS hydrogen iQube; Bajaj exploring H2 2W/3W | Autocar India | TVS working on hydrogen powered scooter; Bajaj exploring hydrogen-powered two-wheelers under Chetak brand | https://www.autocarindia.com/bike-news/hydrogen-powered-tvs-scooter-in-the-works-425268 ; https://www.autocarindia.com/bike-news/bajaj-exploring-hydrogen-powered-two-wheelers-under-chetak-brand-431355 |  |  | concept | — |
| W3-S51 | Mirai sales US 499 (2024) / 210 (2025); global 1,702 (Jan–Nov 2024); cumulative 21,475 (Nov 2022); 2026 fuel credit | GoodCarBadCar; CarFigures; CarBuzz; Wikipedia; Toyota.com | Toyota Mirai Sales Figures; Toyota's Biggest Flop Of 2025…; Toyota Mirai (Wikipedia, "Markets and sales"); 2026 Toyota Mirai | https://www.goodcarbadcar.net/toyota-mirai-sales-figures-usa-canada-monthly-yearly/ ; https://carfigures.com/us-market-brand/toyota/mirai ; https://carbuzz.com/toyotas-biggest-flop-2025-sold-fewer-than-1000/ ; https://en.wikipedia.org/wiki/Toyota_Mirai ; https://www.toyota.com/mirai/ |  |  | observed | 2025 global total not found |
| W3-S52 | Nexo gen-2 (Seoul 2025; Korea sales Jun 2025; 720 km); CR-V e:FCEV (~300/yr; 2,000 plan) | electrive; TopElectricSUV; Honda News; Green Car Reports | Next-gen Hyundai Nexo to debut in Q2 2025; First Look Review: Next-gen Hyundai Nexo; Honda Reveals 2025 Honda CR-V e:FCEV; First Drive: 2025 Honda CR-V e:FCEV | https://www.electrive.com/2024/05/10/next-gen-hyundai-nexo-2025/ ; https://topelectricsuv.com/first-look-review/hyundai-nexo-next-gen/ ; https://hondanews.com/en-US/honda-automobiles/releases/honda-reveals-2025-honda-cr-v-efcev-americas-first-production-plug-in-hydrogen-fuel-cell-electric-vehicle ; https://www.greencarreports.com/news/1142702_2025-honda-cr-v-efcev-test-drive-review |  |  | production | sales figures partial |
| W3-S53 | Global FCEV sales +24.4% (2025); H1 2026 4,643 units (+13%); Europe 14 | electrive; SNE Research | Global H₂ vehicle sales rise by nearly 25%; Global FCEV Sales Reach 4,643 Units in H1 2026, Up 13.0% YoY | https://www.electrive.com/2026/02/23/global-h%E2%82%82-vehicle-sales-rise-by-nearly-25/ ; https://www.sneresearch.com/en/insight/release_view/707/page/0 |  |  | observed (SNE) | absolute 2025 total not seen |
| W3-S54 | IEA GHR 2025: heavy trucks only fast-growing FCEV market; China ~95% FC CV stock; OMV closure; low-emissions H2 <1%; ~1,160 HRS end-2024; country counts | IEA; TÜV SÜD; Interact Analysis; IEA AFC TCP | Global Hydrogen Review 2025 (Executive summary; PDF); Over 1,000 hydrogen refuelling stations in operation worldwide in 2024; 80% of global HRS in five countries | https://www.iea.org/reports/global-hydrogen-review-2025/executive-summary ; https://iea.blob.core.windows.net/assets/12d92ecc-e960-40f3-aff5-b2de6690ab6b/GlobalHydrogenReview2025.pdf ; https://www.tuvsud.com/en/newsroom/press-releases/2025/february/over-1000-hydrogen-refuelling-stations-in-operation-worldwide-in-2024 ; https://interactanalysis.com/insight/80-of-global-hydrogen-refueling-stations-are-located-in-just-five-countries/ |  |  | observed | IEA site blocked; FCEV stock number not retrieved; "72,193 FCEVs / 1,022 HRS" seen without year |
| W3-S55 | Hydrogen HiAce pilot (CPB, 2024) results; 2025 hybrid trial | Toyota Australia; CPB Contractors; Roads Australia; just-auto | Toyota Unveils Hydrogen HiAce; CPB Contractors completes Australian-first Toyota hydrogen-fuelled vehicle pilot; Toyota to test hydrogen-electric hybrid vans in 2025 | https://www.toyota.com.au/discover/news/toyota-unveils-hydrogen-powertrain-technology-for-pilot-program-in-australia ; https://www.cpbcon.com.au/news-and-media/2024/cpb-contractors-completes-australian-first-toyota-hydrogen-fuelled-vehicle-pilot ; https://roads.org.au/cpb-contractors-completes-historic-hydrogen-vehicle-trial/ ; https://www.just-auto.com/news/toyota-to-begin-testing-hydrogen-electric-hybrid-vans-in-2025/ |  |  | pilot | 2025 results not found |
| W3-S56 | GR Corolla H2 (LH2) 2025 racing status | Toyota Global Newsroom; Driving Hydrogen | Pushing Hydrogen Engine Technology to New Heights in the Super Taikyu Series Final Race; Hydrogen-Powered GR Corolla… Fuji 24 Hours; Toyota's superconducting pump and liquid H2 take to the grid | https://global.toyota/en/newsroom/corporate/43550115.html ; https://global.toyota/en/newsroom/corporate/44458612.html ; https://drivinghydrogen.com/2025/11/21/toyotas-superconducting-pump-and-liquid-h2-take-to-the-grid/ |  |  | development | — |
| W3-S57 | HySE members; Dakar 2024 4th, 2025 2nd in class | Global Suzuki; Toyota Times | HySE to participate in the Dakar 2025 with the HySE-X2; HySE Buggy 2nd in Class at Dakar 2025 | https://www.globalsuzuki.com/globalnews/2024/1112.html ; https://toyotatimes.jp/en/spotlights/1082.html |  |  | research | — |
| W3-S58 | Burgman Hydrogen at JMS 2025; "not launching anytime soon"; Access CNG concept | Global Suzuki; BikeWale; Team-BHP | Hydrogen Engine Burgman — Japan Mobility Show 2025; Suzuki Burgman Hydrogen Showcased at JMS 2025; Suzuki Access CNG & hydrogen Burgman showcased | https://www.globalsuzuki.com/mobilityshow2025/burgman/ ; https://www.bikewale.com/news/suzuki-burgman-hydrogen-showcased-at-japan-mobility-show-2025/ ; https://www.team-bhp.com/forum/motorbikes/301240-suzuki-access-cng-hydrogen-powered-burgman-400-showcased-japan-mobility-show-2025-a.html |  |  | concept | 70 MPa spec not restated |
| W3-S59 | Cummins B6.7H production Mar 2025 (India, Tata); X15H 2027 | The Buzz EV News; Cummins; Truckinginfo; autonocion | Cummins starts H2-ICE production at new India facility; Cummins X15 Off-Highway… Agritechnica; Cummins Debuts 15L Hydrogen Engine | https://www.thebuzzevnews.com/cummins-h2-ice/ ; https://www.cummins.com/news/releases/2025/11/10/cummins-x15-highway-receives-powertrain-magazines-2026-alternative-engine ; https://www.truckinginfo.com/10170361/cummins-debuts-15l-hydrogen-engine |  |  | production (limited) | X15H date secondary |
| W3-S60 | JCB H2 engine EU type-approval (May 2025); 11 authorities (Jan 2025); 130+ engines | JCB; Power Progress | JCB secures full EU type-approval for hydrogen engine; Landmark start to 2025…; JCB receives type approval from 11 licensing authorities | https://www.jcb.com/en-GB/explore/insight/news/2025/05/jcb-secures-full-eu-type-approval-for-pioneering-hydrogen-engine/ ; https://www.jcb.com/en-GB/explore/insight/news/2025/01/landmark-start-to-2025-as-jcbs-hydrogen-engine-approved-for-use/ ; https://www.powerprogress.com/news/jcb-receives-type-approval-from-11-licensing-authorities-for-h2-ice/8049767.article |  |  | approved / evaluation | off-highway only |
| W3-S61 | KEYOU conversions; Daimler H2-ICE 2027 | Daimler Truck; gasworld; Allison Transmission | Daimler Truck Partners with KEYOU on Hydrogen Combustion Technology; Daimler hydrogen ICE truck to launch in 2027; KEYOU hands over first hydrogen 18-ton truck… to EP-Trans | https://www.daimlertruck.com/en/newsroom/pressrelease/daimler-truck-partners-with-keyou-on-hydrogen-combustion-technology-53511488 ; https://www.gasworld.com/story/daimler-hydrogen-ice-truck-to-launch-in-2027-under-keyou-partnership/2252665.article/ ; https://www.allisontransmission.com/newsroom/article/2024/12/12/zero-emission-and-efficient---keyou-hands-over-first-hydrogen-18-ton-truck-with-allison-fully-automatic-transmission-to-ep-trans |  |  | pioneer fleet / announced | — |
| W3-S62 | MAN hTGX 200 units; Deutz TCG 7.8 H2; Westport H2 HPDI | Hydrogen Insight; MAN; Deutz; IAA/trans.info | MAN to produce limited series of 200 vehicles with H2 combustion engines; DEUTZ TCG 7.8 H2; Westport unveils H2 HPDI; Westport successfully demonstrates H2 HPDI | https://www.hydrogeninsight.com/transport/hydrogen-sceptical-truck-maker-man-to-produce-limited-series-of-200-vehicles-with-h2-combustion-engines/2-1-1622908 ; https://press.mantruckandbus.com/corporate/man-expands-its-zero-emission-portfolio/ ; https://www.deutz.com/en/products/product-highlights/deutz-tcg-78-h2 ; https://trans.info/en/westport-successfully-demonstrates-h2-hpdi-fuel-system-374302 |  |  | limited series / prototype | — |
| W3-S63 | ULEMCo conversions (Aberdeen 35 vehicles; ambulance; H2/HVO) | Van Fleet World; ULEMCo; Autocar UK | ULEMCo wins deal for largest fleet-wide hydrogen conversion to date; ULEMCo completes first hydrogen/HVO dual-fuel conversion; Ulemco reveals first hydrogen-fuelled ambulance | https://vanfleetworld.co.uk/ulemco-wins-deal-for-largest-fleet-wide-hydrogen-conversion-to-date/ ; https://ulemco.com/ulemco-completes-first-hydrogen-hvo-dual-fuel-conversion/ ; https://www.autocar.co.uk/car-news/new-cars/ulemco-reveals-first-hydrogen-fuelled-ambulance |  |  | commercial (small) | prices NOT FOUND |
| W3-S64 | Stellantis ends FC programme 16 Jul 2025; Symbio fallout | Stellantis (blocked); Driving Hydrogen; US News/Reuters; Global Banking & Finance | Stellantis Discontinues Hydrogen Fuel Cell Technology Development Program; Stellantis halts hydrogen van plans…; Symbio cuts 70% of staff…; Stellantis to exit Symbio JV, faces €235 million cost | https://www.stellantis.com/en/news/press-releases/2025/july/stellantis-discontinues-hydrogen-fuel-cell-technology-development-program ; https://drivinghydrogen.com/2025/07/16/stellantis-halts-hydrogen-van-plans-ends-fuel-cell-development-programme/ ; https://money.usnews.com/investing/news/articles/2025-07-16/stellantis-discontinues-hydrogen-fuel-cell-programme-and-van-production ; https://drivinghydrogen.com/2025/12/08/symbio-cuts-70-of-staff-as-stellantis-walks-away/ |  |  | decision (observed) | primary blocked |
| W3-S65 | Shell closes 7 California LD stations (6 Feb 2024) | S&P Global; InsideEVs; Automotive News | Shell permanently closes light-duty hydrogen fueling stations; Shell Is Immediately Closing All Of Its California Hydrogen Stations | https://www.spglobal.com/energy/en/news-research/latest-news/electric-power/021224-shell-permanently-closes-light-duty-hydrogen-fueling-stations ; https://insideevs.com/news/708156/shell-closes-california-hydrogen-stations/ ; https://www.autonews.com/mobility-report/shell-closes-california-light-duty-hydrogen-fueling-stations/ |  |  | observed | — |
| W3-S66 | H2 Mobility: 22 closures by mid-2025; 14 more 31 Dec 2025 | electrive; MobilityEnergy; H2.LIVE; CleanTechnica | H2 Mobility to shut down 22 hydrogen fuel stations in Germany; Fourteen hydrogen filling stations in Germany to close this year; Network consolidation 700 bar | https://www.electrive.com/2025/03/03/h2-mobility-to-shut-down-22-hydrogen-fuel-stations-in-germany/ ; https://www.mobilityenergy.com/en/industry/2025/11/17/fourteen-hydrogen-filling-stations-in-germany-to-close-this-year-focusing-on-forward-looking-locations/ ; https://h2.live/en/news/3815/ |  |  | observed | — |
| W3-S67 | Honda Clarity ended Aug 2021; GM–Honda FCSM JV to end before end-2026 | InsideEVs; Automotive Manufacturing Solutions; Headlight.news | Official: Honda Discontinues Clarity PHEV And Clarity Fuel Cell; Honda-GM fuel cell venture ends as EV priorities reshape | https://insideevs.com/news/514412/honda-discontinues-clarity-phev-fcv/ ; https://www.automotivemanufacturingsolutions.com/electrification/hondagm-fuel-cell-venture-ends-as-ev-priorities-reshape/2591276 ; https://headlight.news/2025/02/24/honda-general-motors-continue-to-drift-apart-ending-fuel-cell-partnership/ |  |  | observed | — |
| W3-S68 | Suzuki Growth Strategy FY2030 (CNG/biogas/ethanol; ~70% CNG share); multi-pathway statements 2025 | Suzuki Motor Corp (globalsuzuki.com blocked; mirror suzuki.hu; Automotive World); Autocar Professional; S&P Global Mobility | Suzuki Announces Growth Strategy for FY2030; Suzuki backs multi-pathway route to carbon neutrality, says only BEVs not practical; India's multifuel path to automotive sustainability | https://www.globalsuzuki.com/globalnews/2023/0126.html ; https://www.suzuki.hu/corporate/en/company/growth-strategy-2030/growth-strategy ; https://www.autocarpro.in/news/suzuki-backs-multi-pathway-route-to-carbon-neutrality-says-only-bevs-not-practical-129454 ; https://www.spglobal.com/automotive-insights/en/blogs/2025/11/indias-multifuel-path-to-automotive-sustainability |  |  | company strategy | context only |
| W3-S69 | SRDI established 28 Aug 2022; role; Banas Suzuki biogas plant 6 Dec 2025; 11 biogas plants | Global Suzuki; SRDI site; MarketScreener; BioEnergy Times | Suzuki establishes R&D center in India; About Suzuki R&D India; Suzuki holds the opening ceremony of Banas Suzuki Biogas Plant in India; Suzuki, Maruti Suzuki to add 11 biogas plants | https://www.globalsuzuki.com/globalnews/2022/0828b.html ; https://www.suzukirndindia.com/about-us ; https://uk.marketscreener.com/news/suzuki-motor-december-9suzuki-holds-the-opening-ceremony-of-banas-suzuki-biogas-plant-in-india-ce7d51d2db8ff22d ; https://bioenergytimes.com/suzuki-maruti-suzuki-to-add-11-biogas-plants-using-cow-dung-farm-waste/ |  |  | company | context only |
| W3-S70 | Delhi draft EV Policy 2.0 proposed phasing out CNG autos; previous policy extended to 31 Mar 2025 | Deccan Herald | Draft EV policy 2.0 recommends phasing out of CNG-driven autorickshaws in Delhi; Delhi electric vehicle policy extended till March 31, 2025 | https://www.deccanherald.com/amp/story/india%2Fdelhi%2Fdraft-ev-policy-20-recommends-phasing-out-of-cng-driven-autorickshaws-in-delhi-3482789 ; https://www.deccanherald.com/amp/story/india%2Fdelhi%2Fdelhi-electric-vehicle-policy-extended-till-march-31-2025-3295696 |  |  | draft / rule | superseded by 2026 policy |
| W3-S71 | Context: India's alternative-fuel shift "becoming a CNG story" | Down To Earth | India's CNG Dominance: A Step Towards Cleaner Transport or a Roadblock to Zero Emissions? | https://www.downtoearth.org.in/energy/indias-transport-sectors-alternative-fuel-shift-is-becoming-a-cng-story-but-what-comes-next |  |  | commentary | — |
| W3-S72 | CNG kit prices ₹40–70k (sequential) | Cars24; Spinny | CNG Kit Price in India; CNG Kit in India: Price, Function, Conversion | https://www.cars24.com/article/cng-kit-price-in-india/ ; https://www.spinny.com/blog/cng-kit-in-india-price-function-conversion/ |  |  | market guide | vendor content |
| W3-S73 | Creta Electric BaaS ₹10.99 lakh + ₹3.90/km | Team-BHP / CarDekho | Hyundai Creta Electric 2026 | https://www.team-bhp.com/new-cars/hyundai/creta-electric/ |  |  | list price | — |
| W3-S74 | ~85,000 taxis in Delhi (2022) | Statista (summary only) | Number of taxis in Delhi | (URL not captured — search summary only) |  |  | observed (secondary) | unverified; low confidence |
| W3-S76 | expert scepticism on 2030 gas targets | IEEFA | Is India's 2030 gas consumption target feasible? | https://ieefa.org/resources/indias-2030-gas-consumption-target-feasible |  |  | analysis | title only seen |
| W3-S77 | PNGRB 2030/2040 projections (CGD 87 mmscmd 2030 base case) and critique | PNGRB press note; PSU Watch | Natural Gas Projections 2030–40 (press note 7 Apr 2025); PNGRB projects a natural gas boom, but experts urge a sobering reality check | https://pngrb.gov.in/pdf/press-note/20250407_NG_Projections2030_40.pdf ; https://psuwatch.com/newsupdates/pngrb-projects-a-natural-gas-boom-but-experts-urge-a-sobering-reality-check |  |  | modelled | PDF blocked |
| W4-S1 | 700-bar Type IV system $12.1–13.3/kWh (median 12.7, 2020$) at 100k/yr; carbon fibre dominant | US DOE Hydrogen Program | Record #24006 "Onboard Type IV Compressed Hydrogen Storage System—Cost and Performance Status" | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/24006-onboard-storage-cost-performance-status.pdf | 28 Apr 2025 / 2024 analysis | Record summary (page not verified) | modelled | Host blocked; values from search summary; 10k & 500k rows unread |
| W4-S2 | $14.2 / 15.7 / 21.3 per kWh at 500k / 100k / 10k systems/yr (2016$) | US DOE | Record #19008 onboard storage cost & performance status | https://www.hydrogen.energy.gov/pdfs/19008_onboard_storage_cost_performance_status.pdf | 25 Nov 2019 | Record p.1 summary | modelled | search summary |
| W4-S3 | 2013: 350-bar $15/kWh, 700-bar $19/kWh single tank (500k/yr); Record 13010 $16–20/kWh | US DOE / Strategic Analysis | Record #13010; "Hydrogen Storage Cost Analysis Final Public Report" (OSTI 1082754) | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/13010_onboard_storage_performance_cost.pdf ; https://digital.library.unt.edu/ark:/67531/metadc827980/m2/1/high_res_d/1082754.pdf | 11 Jun 2013 / 2013 | abstract/summary | modelled | old dollar-year |
| W4-S4 | DOE targets $10/9/8 per kWh = $333/300/266 per kg | US DOE | Technical Targets for Onboard Hydrogen Storage for LDVs | https://www.energy.gov/cmei/fuels/doe-technical-targets-onboard-hydrogen-storage-light-duty-vehicles | current page | Storage system cost row | target | targets not costs |
| W4-S5 | 12% cost reduction 2013→2015; volumes 10k–500k/yr | US DOE FCTO | Webinar: Onboard Type IV compressed H2 storage system cost | https://www.energy.gov/sites/prod/files/2016/03/f30/fcto_webinarslides_compressed_h2_storage_system_cost_022516.pdf | 25 Feb 2016 | slide 8 | modelled | snippet |
| W4-S6 | Compressed-gas storage USD 400–700 per kg H2 (Cerri et al.) | MDPI Energies 16(13):5233 | A Review on the Cost Analysis of Hydrogen Gas Storage Tanks for Fuel Cell Vehicles | https://www.mdpi.com/1996-1073/16/13/5233 | 2023 | Table citing Cerri et al. [33] | literature | secondary citation |
| W4-S7 | Type IV 700-bar tanks ~$700–1,200 per kg capacity | Dataintelo (market report) | Cryo-Compressed Hydrogen Storage Market | https://dataintelo.com/report/cryo-compressed-hydrogen-storage-market | 2025 | Opportunities & threats | estimate | low-grade commercial source |
| W4-S8 | Time Technoplast PESO approvals (Mar/Jun 2024), 250 L, BV cert, ₹2.30 crore trial order, ₹555 crore 9MFY26 | Business Standard; Autocar Professional; Time Technoplast press release; Scanx | multiple | https://www.business-standard.com/markets/capital-market-news/time-technoplast-gets-final-approval-from-peso-for-manufacturing-composite-cylinders-124062100121_1.html ; https://www.autocarpro.in/news/time-technoplast-secures-international-certification-for-hydrogen-composite-cylinder-126444 ; https://www.timetechnoplast.com/pdf/investors-calendar/press-release/approval-for-manufacturing-of-type-iv-composite-cylinders-for-hydrogen.pdf | 2024–2026 | news | observed | no unit prices |
| W4-S10 | Bosch H2 PFI "suitable for retrofitting existing fleets" | Bosch Mobility | Port fuel injection for hydrogen engines | https://www.bosch-mobility.com/en/solutions/powertrain/gas/port-fuel-injection-for-hydrogen-engines/ | current | product page | observed | no price |
| W4-S11 | CNG kit prices ₹45–65k (BS6 OBD-II incl. install), Lovato ₹31,999–58,000, Landi Renzo ₹32,000, install ₹3–7k, RTO ₹2–3k | mycarhelpline; Cars24; IndiaMART; Autonexa; IOAGPL | Best CNG kit brands 2026; CNG kit price in India; listings | https://www.mycarhelpline.com/index.php?option=com_easyblog&view=entry&id=557&Itemid=91 ; https://www.cars24.com/article/cng-kit-price-in-india/ ; https://www.indiamart.com/proddetail/landirenzo-cng-kit-23070488673.html ; https://autonexa.com/cng-fitting-cost-in-india-complete-guide-price-break-down-kits-and-savings ; https://ioagpl.com/cng-kit-price/ | 2025–2026 | retail/trade listings | observed (retail) | listings vary by city |
| W4-S12 | ARAI retrofit approval under Rule 115; "full approval in 45* days"; fee schedule not public; Scribd cost-breakdown doc exists; ₹15–25 lakh homologation anecdote | ARAI; Scribd; Quora | Retrofitment certification; Vehicle type approval; "Retrofitment Kit Approval" | https://www.araiindia.com/certification/certification/retrofitment ; https://www.araiindia.com/certification/certification/vehicle ; https://www.scribd.com/document/975757543/Retrofitment-Kit-Approval-1 | current | pages | observed / anecdote | fees NOT FOUND |
| W4-S13 | PFI gasoline conversion needs injectors + plugs; DI needs ignition/injection; custom pistons | Ultimate Cell UK; OSTI 1008335 (NETL small H2 engine) | trade guides | https://ultimatecelldistributors.co.uk/insights/hydrogen-retrofit-for-diesel-petrol-engines/ ; https://www.osti.gov/servlets/purl/1008335 | 2020s / 2011 | — | qualitative | low grade |
| W4-S14 | HORIBA H2-ICE test bed Pune, ₹28–30 crore, ≤380 kW, Dec 2024 | HORIBA; Autocar Professional | HORIBA inaugurates first H2-ICE test bed facility in India | https://www.horiba.com/int/mobility/news/detail/news/12/2024/20241210/ ; https://www.autocarpro.in/news/horiba-opens-hydrogen-internal-combustion-engine-test-bed-facility-in-pune-123903 | 10 Dec 2024 | press release | observed | — |
| W4-S15 | ARAI ₹500 crore H2-ICE test infra (from 2025); Takwe cylinder test facility; ₹100 crore labs | Autocar Professional; Pune.news; Telematics Wire | ARAI to invest Rs 500 crore…; ARAI to establish hydrogen cylinder testing facility | https://www.autocarpro.in/news/arai-to-invest-rs-500-crore-to-enhance-infra-h2ice-testing-likely-from-2025-120865 ; https://www.autocarpro.in/news/arai-to-establish-a-hydrogen-cylinder-testing-facility-at-greenfield-takwe-near-pune-115086 | 2023–2024 | news | announced | plans, not completions |
| W4-S16 | CNG Delhi ₹86.98/kg | Goodreturns; CarDekho | CNG price in New Delhi today | https://www.goodreturns.in/cng-price-in-new-delhi.html ; https://www.cardekho.com/cng-price-in-delhi-state | 14 Sep 2026 | price page | observed (aggregator) | IGL site blocked |
| W4-S17 | MGL CNG ₹88/kg from 1 Sep 2026 (+₹2), PNG ₹53; spot RLNG cited | BusinessToday; IndianPSU; Goodreturns | MGL hikes CNG, PNG prices in Mumbai from September 1 | https://www.businesstoday.in/latest/story/mgl-hikes-cng-png-prices-in-mumbai-from-september-1-check-revised-rates-552419-2026-08-31 ; https://indianpsu.com/mgl-cng-price-hike-mumbai-88-september-2026/ ; https://www.goodreturns.in/cng-price-in-mumbai.html | 31 Aug–14 Sep 2026 | news | observed | — |
| W4-S18 | Ahmedabad CNG ₹94.02 (7 Sep 2026); ₹86.02 (16 May 2026); ₹83.77 (3 Apr 2026) | Goodreturns; DeshGujarat; Vibes of India | CNG price in Ahmedabad; Adani CNG hikes | https://www.goodreturns.in/cng-price-in-ahmedabad.html ; https://deshgujarat.com/2026/05/16/adani-gas-imposes-another-price-hike-cng-rate-in-ahmedabad-now-at-rs-86-02-per-kg/ ; https://deshgujarat.com/2026/04/03/adani-cng-price-hiked-by-%E2%82%B91-50-rate-reaches-%E2%82%B983-77-amid-global-gas-crisis/ | 2026 | news/price page | observed | — |
| W4-S19 | Petrol Delhi ₹102.12/L (25 May 2026), crude basket $113–114; aggregator ₹111.21 (14 Sep 2026); Ahmedabad ₹102.15 vs ₹94.59 conflict | Univest; PingTV; CarDekho; Goodreturns; DriveSpark; Business Standard | Petrol price hike to Rs 100 in Delhi…; petrol price pages | https://univest.in/blogs/petrol-price-hike-crosses-100-delhi-fourth-hike-10-days-may-2026 ; https://www.pingtvindia.com/fuel-price-hike-india-petrol-price-delhi-crossed-100/ ; https://www.cardekho.com/petrol-price ; https://www.goodreturns.in/petrol-price-in-ahmedabad.html ; https://www.drivespark.com/petrol-price-in-ahmedabad/ | May–Sep 2026 | news/price pages | observed | Sep figures unverified vs IOCL |
| W4-S20 | DERC EV tariff ₹4.50/kWh LT, ₹4.00/kVAh HT, zero fixed; domestic ₹3–8; DERC 1 Jul 2026 upstream-cost order | thediscombill; bridgewaypower; easyEVplug; Energetica India; MeraEV | Delhi electricity tariff 2025-26; DERC directs… | https://thediscombill.com/tariffs/delhi/ ; https://www.easyevplug.com/ev-charger-installation-delhi ; https://energetica-india.net/news/derc-directs-full-upstream-ev-charging-infra-costs-to-be-included-in-demand-notes-under-pm-e-drive | 2025–2026 | tariff summaries | observed (secondary) | DERC order not opened |
| W4-S21 | Public DC fast charging ₹16–24/kWh; Tata Power ₹17; ChargeZone ₹20–25; Statiq ₹15–22 | Pulse Energy; searchev.co; MeraEV; GreenTax | EV charging rates in India 2026 | https://pulseenergy.io/blog/ev-charging-rates ; https://searchev.co/cost-for-charging-electric-cars/ ; https://www.meraev.com/blog/ev-charging-stations-india-types-costs-guide | 2026 | guides | observed (secondary) | operator tariffs vary by state |
| W4-S22 | CEEW TCO: charging tariffs; H2 production/dispensing/delivered cost 2030–2050; taxi/4W findings; H2 bus competitiveness; maintenance Table 5; 3W/2W ₹/km | CEEW | What Drives Vehicle Ownership Costs in India? / Cost of Ownership for India's Road Transport Sector | https://www.ceew.in/publications/cost-of-ownership-for-road-transport-sector-for-different-vehicle-segments-fuels-and-powertrains ; https://www.ceew.in/sites/default/files/cost-of-ownership-for-different-vehicle-segments-fuels-and-powertrains.pdf | Jun–Jul 2025 / 2024 base | Exec summary; Fuel costs; Green hydrogen; Maintenance; Results §4 (verbatim extracts) | modelled | Tables 3 & 5 values not readable |
| W4-S23 | ADAC charging losses AC 5–10% (wallbox), 24–30% (socket), DC 5–15% | Zecar; InsideEVs; TheNextWeb; vision-mobility (reporting ADAC) | EV charging losses up to 24%…; How efficient is each type of EV charger | https://zecar.com/reviews/ev-charging-losses-adac-study ; https://insideevs.com/features/711659/ev-charger-efficiency-losses/ ; https://vision-mobility.de/en/news/adac-analysis-less-energy-is-lost-at-fast-chargers-384109.html | 2022–2024 | test summaries | measured | ADAC report not opened |
| W4-S24 | GST: CNG small cars & parts 28%→18% (Sep 2025); CNG-tank classification dispute 18% vs 28% | EY; TaxO; SAG Infotech | GST Council announces major rate rationalization (4 Sep 2025); GST notices to CNG kit providers | https://www.ey.com/content/dam/ey-unified-site/ey-com/en-in/alerts-hub/2025/09/gst-council-announces-major-rate-rationalization-and-trade-facilitation-measures.pdf ; https://taxo.online/latest-news/28-12-2024-gst-notices-sent-to-over-a-dozen-cng-kit-providers/ | Sep 2025; Dec 2024 | alert; news | observed | kit rate unresolved |
| W4-S25 | Delhi EV Policy 2026: notified 30 Jun 2026, in force 1 Jul 2026–31 Mar 2030; 100% road tax & registration waiver BEV ≤₹30 lakh; hybrids excluded | EV Infrastructure News; Akashvani News; Coverfox; MeraEV | Delhi notifies EV Policy 2026… | https://www.evinfrastructurenews.com/ev-policies/delhi-notifies-ev-policy-2026-with-tax-exemptions-for-ev ; https://newsonair.gov.in/electric-cars-priced-upto-30-lakh-rupees-and-registered-in-delhi-to-be-granted-100-exemption-on-road-tax/ | Jun–Jul 2026 | news | observed | gazette not opened |
| W4-S26 | Tata 8 yr/160,000 km (Tiago.ev, Punch.ev, Curvv.ev, Nexon.ev 45), lifetime first owner; MG ZS EV 8 yr/150,000 km; Windsor/Comet lifetime | Tata.ev; Cars24 news; ev.care | Tata EV warranty; MG ZS EV warranty explained | https://ev.tata.cars/service/warranty.html ; https://www.cars24.com/news/auto/tata-nexon-ev-lifetime-battery-warranty-details-revealed/ ; https://ev.care/blog/mg-zs-ev-warranty-guide | 2025–2026 | warranty pages | observed | — |
| W4-S27 | Geotab degradation 1.8%/yr (2024) → 2.3%/yr (2025 update); >90% after 160,000 km | Geotab | EV battery health study; 2024 battery degradation update | https://www.geotab.com/blog/ev-battery-health/ ; https://www.geotab.com/uk/press-release/2024-battery-degradation/ | 2024–2025 | blog/press | measured | fleet mix not Indian |
| W4-S28 | Used Dzire CNG 2022 ₹6.09 lakh (56k km); True Value 2022 ₹6.40 lakh / 2024 ₹7.40 lakh; new Tour S CNG ₹7.09 lakh; on-road ₹6.24–8.51 lakh | Cars24; Maruti True Value; Cars24 new cars; CarDekho | listings | https://www.cars24.com/buy-used-maruti-dzire-cars-new-delhi/ ; https://www.marutisuzukitruevalue.com/used-dzire ; https://www.cars24.com/new-cars/maruti-suzuki/dzire-tour/ ; https://www.cardekho.com/maruti/dzire-tour-s/price-in-new-delhi | Sep 2026 | listings | observed (asking prices) | not taxi-registered cars |
| W4-S29 | NBFC rates: Shriram from 10%; taxi 9–15%; Chola 9.35–17.35% used car, 10–21% CV; NBFC 13–18% | Shriram Finance; upniva (Cholamandalam); Asymmetrica/calcwise | rate pages | https://www.shriramfinance.in/articles/used-car-loan/2025/understanding-used-car-loan-interest-rates-for-taxi-drivers ; https://upniva.com/loans/cholamandalam-used-car-loan ; https://asymmetrica.in/calculators/car-loan-rates/ | 2025–2026 | rate pages | observed | indicative |
| W4-S30 | Taxi 70,000 km/yr; car 11,500; bus 75,000 (Table A.3) | NITI Aayog | Scenarios Towards Viksit Bharat and Net Zero — Sectoral Insights: Transport | https://www.niti.gov.in/sites/default/files/2026-02/Scenarios-Towards-Viksit-Bharat-and-Net-Zero-Sectoral-Insights-Transport.pdf | Feb 2026 | Annex Table A.3 (verbatim) | modelling assumption | — |
| W4-S31 | ICCT ride-hailing TCO ₹7.6/km; −34% Delhi / −20% Hyderabad with incentives; ₹60,000 bulk discount | ICCT | Updated analysis of incentives for electrifying India's four-wheeler ride-hailing fleet | https://theicct.org/wp-content/uploads/2021/06/update-electrifying-india-ride-hailing-fleet-apr2021.pdf | Apr 2021 | working paper (snippets) | modelled | daily-km assumption not extracted |
| W4-S32 | Driver anecdotes ~300 km/day incl. empty running | Quora | — | https://www.quora.com/How-many-kilometers-on-average-do-Ola-or-Uber-cab-drivers-drive-in-a-single-day | n.d. | — | anecdotal | low grade |
| W4-S33 | Green H2 USD 3.5–5/kg (industry); breakeven <USD 2; USD 1.8/kg policy levers | CEEW | How can India Boost Investment for Domestic Green Hydrogen? | https://www.ceew.in/publications/how-can-indian-policymakers-boost-investments-for-domestic-green-hydrogen-financing | 2024–2025 | Challenge 1 (verbatim) | estimate | — |
| W4-S34 | ₹397–560/kg ($4.6–6.7/kg), 2–3× grey | GH2 India | India's Green Molecules Moment: ACME's new deals… | https://www.gh2.org.in/indias-green-molecules-moment-acmes-new-deals-show-the-market-is-finally-moving-from-announcements-to-offtake | 2025–2026 | article (verbatim) | estimate | advocacy body |
| W4-S36 | Green H2 $4–7/kg today; $1.60 (2030), $0.70 (2050) best case; $1.4/kg most aggressive; $1/kg goal | NITI Aayog & RMI | Harnessing Green Hydrogen | https://www.niti.gov.in/sites/default/files/2022-06/Harnessing_Green_Hydrogen_V21_DIGITAL_29062022.pdf ; https://www.pib.gov.in/PressReleasePage.aspx?PRID=1838104 | 29 Jun 2022 | cost chapter (summary) | modelled/target | search summary |
| W4-S37 | At-pump green H2 ~$5/kg (₹350) mid, $3.5 (₹245) optimistic (2050) | ICCT; EurekAlert | Hydrogen fuel for transport in India | https://theicct.org/publication/india-fuel-hydrogen-cost-jan22/ ; https://www.eurekalert.org/news-releases/941197 | Jan 2022 | summary | modelled | year attribution uncertain |
| W4-S38 | Green ammonia auctions ₹55.75 (first), ₹49.75 lowest, ₹53.27 weighted avg, 724,000 t/yr; ₹54.73 8th; ₹52.50 Onix | PIB; pv magazine India; S&P Global; ACME; Fuel Cells Works | SECI first green ammonia auction…; Green ammonia only 10% costlier…; Low-priced Indian renewable ammonia auctions | https://www.pib.gov.in/PressReleasePage.aspx?PRID=2153006 ; https://www.pv-magazine-india.com/2025/08/28/green-ammonia-is-only-10-costlier-than-grey-in-the-latest-sight-mode-2a-auctions/ ; https://www.spglobal.com/commodity-insights/en/news-research/latest-news/energy-transition/090425-low-priced-indian-renewable-ammonia-auctions-a-meaningful-signal-h2global | Jul–Sep 2025 | news | observed (auction) | conversion to H2 is a calculation |
| W4-S39 | SIGHT Mode-1 Tranche-II 450,000 t/yr, ₹2,239 crore; ₹50/40/30 per kg | Renewable Watch; JMK; SolarQuarter | SECI announces results of green hydrogen auction under SIGHT | https://renewablewatch.in/2025/03/20/seci-announces-results-of-green-hydrogen-auction-under-sight-programme/ ; https://jmkresearch.com/seci-awards-450000-mt-annual-capacity-under-sight-tranche-ii-for-green-hydrogen-production/ | Mar 2025 | news | observed | — |
| W4-S40 | Electrolyser Tranche-II 1.5 GW, 11 winners, ₹4,440 crore FY26–30 | Mercom; pv magazine India; MNRE | MNRE allocates ₹44.4 bn…; SECI awards LoAs… | https://www.mercomindia.com/mnre-electrolyzer-manufacturing-sight-ii-program ; https://www.pv-magazine-india.com/2025/01/21/the-hydrogen-stream-seci-awards-loas-for-electrolyser-production-linked-incentives/ | Jan 2025 | news | observed | — |
| W4-S41 | Electrolyser ₹30,000/kW alkaline, ₹42,000/kW PEM (1 MW, 300 MW/yr) | JMK Research / World Bank technical report | India's $2.1bn leap towards its green hydrogen vision; Technical and Economic Characteristics… | https://jmkresearch.com/wp-content/uploads/2025/02/Indias-2.1bn-leap-towards-its-Green-Hydrogen-vision.pdf ; https://documents1.worldbank.org/curated/en/099022326123011297/pdf/P506220-b45b0865-8706-4862-8792-d8cc71f84569.pdf | Feb 2025 / 2026 | summary | modelled | attribution between the two docs unclear |
| W4-S42 | Grey H2 $1.5–1.6/kg stable, $3–7.5 under LNG shock; gap $1.6–3.2; SMR ₹181–229/kg (LinkedIn) | RMI; LinkedIn post | Green Hydrogen Production Pathways for India | https://rmi.org/green-hydrogen-production-pathways-for-india/ ; https://www.linkedin.com/posts/sanjeevsharmagh2_refineryhydrogen-greenhydrogen-iocl-activity-7454393902108889088-B4dk | 2024–2026 | exec summary (verbatim) | estimate | LinkedIn low grade |
| W4-S43 | Station cost $1,200–3,000 per kg/day (770–1,620 kg/day, 700 bar) | US DOE | Record 21002 Hydrogen Fueling Station Cost | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/21002-hydrogen-fueling-station-cost.pdf | 2 Nov 2020 | summary | observed (grants) | blocked; snippet |
| W4-S44 | Delivery+dispensing ~$11/kg @450 kg/d, ~$8/kg @1,000 kg/d (liquid; HDSAM 3.1) | US DOE | Record 20007 Hydrogen Delivery and Dispensing Cost | https://www.hydrogen.energy.gov/docs/hydrogenprogramlibraries/pdfs/20007-hydrogen-delivery-dispensing-cost.pdf | 25 Aug 2020 | summary | modelled | blocked; snippet |
| W4-S45 | 350-bar delivery+dispensing $2.70–3.70/gge (HDSAM 2.3, mature) | US DOE | Record 16011 H2 delivery cost 2015 | https://www.hydrogen.energy.gov/pdfs/16011_h2_delivery_cost_2015.pdf | 2015 | summary | modelled | mature-market case |
| W4-S46 | 200 kg/day station $6–8/kg (gaseous), $8–9 (liquid); utilisation 50% (2030), 70% (2050) | ICCT | Cost of renewable hydrogen produced onsite at HRS in Europe | https://theicct.org/publication/fuels-eu-onsite-hydro-cost-feb22/ | Feb 2022 | white paper summary | modelled | snippet |
| W4-S47 | NREL 88818 scope (700 bar, 2022$); ATB 2024 H2 price ranges 6.2–12.0 / 8.2–18.0; future 6.2–7.0; 4.0 target | NREL | Levelized Cost of Dispensed Hydrogen for Heavy-Duty Vehicles (TP-5400-88818); ATB 2024 Hydrogen | https://docs.nrel.gov/docs/fy24osti/88818.pdf ; https://atb.nlr.gov/transportation/2024/Hydrogen | Mar 2024 | ATB table (verbatim) | modelled | 88818 tables unread |
| W4-S48 | LCOH $11.51 (2025) → $4.26/kg (2050); UR 0.3/0.5/1.0; breakeven $4.90–5.85 | Elsevier (journal article, S2590116826000421) | Breakeven levelized cost of hydrogen for the zero-emission freight transition | https://www.sciencedirect.com/science/article/pii/S2590116826000421 | 2026 | Highlights; §4.3 (verbatim) | modelled | freight context |
| W4-S49 | 350-bar compressor/storage cost inputs used by CEEW | Wiley (Chem. Ing. Tech. 96) | Detailed Cost Analysis of Hydrogen Refueling Costs for Fleets (Eißler et al.) | https://onlinelibrary.wiley.com/doi/full/10.1002/cite.202300123 | 2024 | — | modelled | not opened |
| W4-S50 | California $32–36/kg; True Zero $36 | S&P Global; hydrogenfuelfinder; Stillwater | California hydrogen pump prices reach new highs; Why hydrogen fuel costs $36/kg in 2025 | https://www.spglobal.com/energy/en/news-research/latest-news/energy-transition/100124-california-hydrogen-pump-prices-for-light-duty-vehicles-reach-new-highs ; https://hydrogenfuelfinder.com/blog/hydrogen-fuel-cost-2025-reality-check.html | 2024–2025 | news | observed | — |
| W4-S51 | Germany €10–15/kg retail; €8/kg hylane deal from 1 Jan 2026 | H2.LIVE / H2 MOBILITY; Driving Hydrogen; glpautogas | Hydrogen for 8 euros per kilogram now possible | https://h2.live/en/press/hydrogen-for-8-euros-per-kilogram-now-possible-hylane-and-h2-mobility-enable-cost-effective-operation-of-hydrogen-trucks/ ; https://www.glpautogas.info/en/hydrogen-sale-price-germany.html | Nov 2025 | press release | observed | fleet deal |
| W4-S52 | Japan Iwatani ¥1,210 → ¥1,650/kg (Jun 2025); ¥700/kg subsidy | Hydrogen Insight; Driving Hydrogen | Price hike…; Japan to introduce ¥700/kg subsidy | https://www.hydrogeninsight.com/transport/price-hike-hydrogen-vehicle-drivers-to-be-charged-33-more-for-fuel-at-majority-of-h2-filling-stations-in-japan/2-1-1628765 ; https://drivinghydrogen.com/2025/05/30/japan-to-introduce-%C2%A5700-kg-hydrogen-fuel-subsidy-to-kickstart-commercial-uptake/ | 2025 | news | observed | — |
| W4-S53 | NTPC Leh: 1.7 MW solar, 80 kg/day 350-bar station, 5 FC buses (25 kg, 300 km) | NTPC | NTPC hands over five hydrogen fuel cell buses to Leh Administration | https://ntpc.co.in/media/press-releases/ntpc-hands-over-five-hydrogen-fuel-cell-buses-leh-administration | Jun 2025 | press release | observed | no cost data |
| W4-S54 | MNRE Phase-1 pilots: 5 projects, 37 vehicles (15 FCEV + 22 H2-ICE), 9 HRS, ₹208 crore, 10 routes, 18–24 months | PIB; Down To Earth | Pilot Projects on Hydrogen Fuelled Buses and Trucks Launched under NGHM | https://www.pib.gov.in/PressReleasePage.aspx?PRID=2107795 ; https://www.downtoearth.org.in/environment/as-told-to-parliament-march-25-2025-37-cross-country-vehicles-9-fuelling-stations-planned-under-green-hydrogen-mission | Mar 2025 | press release | observed (sanction) | — |
| W4-S55 | Phase-2: 12 projects, 70 vehicles (27 buses/43 trucks), 16 HRS, 21 routes, ₹410.77 crore (₹41.58 released); ₹292 crore NGHM spend to date | pv magazine India; Fuel Cells Works; EQ; Business Standard; Renewable Watch | India awards 12 green hydrogen mobility pilots…; Govt spends ₹292 crore under NGHM so far | https://www.pv-magazine-india.com/2026/08/03/the-hydrogen-stream-india-awards-12-green-hydrogen-mobility-pilots-with-inr-410-77-crore-support/ ; https://fuelcellsworks.com/2026/08/02/fuel-cells/india-s-hydrogen-vehicle-pilot-nearly-doubles-70-vehicles-16-refuelling-stations-410-77-crore ; https://www.business-standard.com/amp/industry/news/govt-spends-292-crore-under-national-green-hydrogen-mission-so-far-mnre-126081201853_1.html | Jul–Aug 2026 | news | observed (sanction) | MNRE order not opened |
| W4-S56 | HRS ₹14–20 crore; 200–500 kg/day break-even; "₹4/km" hydrogen | VahanBazaar; Tata AIG blog | Hydrogen refuelling stations India 2030 | https://vahanbazaar.in/tips/hydrogen-refuelling-stations-india-2030-vahanbazaar ; https://www.tataaig.com/knowledge-center/car-insurance/hydrogen-cars-in-india | 2026 | blog | estimate | low grade |
| W4-S57 | IEA GHR 2026: India/Middle East/Chile "just above USD 3/kg" by 2030; China <USD 2 | Green Hydrogen Organisation (reporting IEA GHR 2026) | IEA Global Hydrogen Review 2026… | https://gh2.org/iea-global-hydrogen-review-2026-green-hydrogen-can-help-prevent-next-food-and-energy-crisis | 18–19 Jun 2026 | article (verbatim) | modelled | IEA site blocked |
| W4-S58 | IEA GHR 2025: project pipeline shrank; electrolysis >80% of drop; assumptions annex | IEA | Global Hydrogen Review 2025 (+ Assumptions Annex) | https://www.iea.org/reports/global-hydrogen-review-2025 ; https://iea.blob.core.windows.net/assets/15673ab3-a86a-4434-bff4-490bb42d3563/GlobalHydrogenReview2025AssumptionsAnnex.pdf | Sep 2025 | exec summary | modelled | blocked; India LCOH NOT FOUND |
| W4-S59 | CEA WAEF FY2024-25 0.710 (prov.), FY2023-24 0.727; BM 0.512; CM definition | CEA (user guide v21.0); batchwise.ai; reclimatize; LinkedIn (R. Goenka) | CO2 Baseline Database for the Indian Power Sector, User Guide v21.0 | https://cea.nic.in/wp-content/uploads/baseline/2025/12/User_Guide_V_21.0.pdf ; https://batchwise.ai/methodology/cea-grid-emission-factors/all-india/ | Dec 2025 / FY2024-25 | CM definition (verbatim); WAEF (secondary) | observed (provisional) | primary blocked; T&D note unread |
| W4-S60 | NEP 2022-32: EF 0.548 (2026-27), 0.430 (2031-32) kg/kWh net; non-fossil 57.4% / 68.4% | CEA; PIB | National Electricity Plan 2022-32 (Vol. I Generation) | https://cea.nic.in/wp-content/uploads/irp/2023/05/NEP_2022_32_FINAL_GAZETTE-1.pdf ; https://www.pib.gov.in/PressReleaseIframePage.aspx?PRID=1928750&reg=48&lang=2 | May 2023 | summary | projected | snippet |
| W4-S61 | India default fuel EFs exist (GLEC-aligned) | Smart Freight Centre India | India Default GHG Emission Values V1.0 | https://smart-freight-centre-media.s3.amazonaws.com/documents/India_Default_GHG_Emission_Values_V1_SFC_INDIA_latest.pdf | 2023–2024 | — | published | not opened |
| W4-S62 | IPCC default EFs 69,300 (gasoline), 56,100 (natural gas) kg CO2/TJ; NCV 44.3 / 48.0 TJ/Gg | IPCC | 2006 IPCC Guidelines Vol. 2 Energy, Ch. 1 Tables 1.2 & 1.4 | https://ghgprotocol.org/Third-Party-Databases/IPCC-Emissions-Factor-Database | 2006 | Tables 1.2, 1.4 | default | from memory — verify before use |
| W4-S63 | ICCT 2025 LCA values (235/234/203/188/163/63/52/50/175 g/km; H2 13.6 & 1.2 kg CO2e/kg; battery 72.8 kg/kWh; +40% production; 17,000 km; TTW non-CO2) | ICCT (Negri & Bieker) | Life-cycle GHG emissions from passenger cars in the EU: a 2025 update | https://theicct.org/wp-content/uploads/2025/07/ID-392-%E2%80%93-Life-cycle-GHG_report_final.pdf ; https://theicct.org/publication/electric-cars-life-cycle-analysis-emissions-europe-jul25/ | 8 Jul 2025 | Key findings; Fuel-cycle §; Figure 4 (verbatim extracts) | modelled | battery chemistry attribution unverified |
| W4-S64 | Unabated NG 10–14 kg CO2e/kg (upstream 1–5); solar PV electrolysis 0.9–2.5; downstream 0.4–2.7 | IEA; CRS R48196 | Towards hydrogen definitions based on their emissions intensity; Hydrogen Production: Overview and Issues for Congress | https://www.iea.org/reports/towards-hydrogen-definitions-based-on-their-emissions-intensity ; https://www.congress.gov/crs-product/R48196 | Apr 2023; Oct 2024 | exec summary | modelled | snippet |
| W4-S65 | India green-H2 standard ≤2 kg CO2e/kg well-to-gate | MNRE (via CSEP/RMI summaries) | Green Hydrogen Standard for India | https://csep.org/key-insights/green-hydrogen-pathways-in-india-from-a-grid-perspective/ | Aug 2023 | — | regulation | primary not opened |
| W4-S66 | Medium petrol car 54.1 t, PHEV 36.9 t, BEV 25.0 t (15 yr, 42 km/day, STEPS); BEV lower after 2 yr | IEA | EV Life Cycle Assessment Calculator | https://www.iea.org/data-and-statistics/data-tools/ev-life-cycle-assessment-calculator | 2024–2025 | tool defaults (verbatim) | modelled | global assumptions |
| W4-S67 | bZ4X 26 t (25–31); Corolla Cross hybrid 53 t (44–70) | Green NCAP | European LCA results 2024 | https://www.greenncap.com/european-lca-results/ | 2024 | results table (verbatim) | modelled | EU mix |
| W4-S68 | DI H2 engine λ 2.4–3.0, BTE 43%, WLTC tailpipe NOx <35 mg/km with urea-SCR | ACS Omega 11(7):12788 (PMC12947045) | NOx/N2O Formation and Coordinated Urea-SCR Control in an Ultra-Lean Hydrogen-Fueled Automotive SI Engine | https://pmc.ncbi.nlm.nih.gov/articles/PMC12947045/ | 2026 | abstract | measured/modelled | full text not indexed |
| W4-S69 | 55 kW 4-cyl PFI H2ICE NRMM tailpipe NOx variants | Environ. Sci.: Processes & Impacts (doi 10.1039/d4em00448e) | Evaluating low NOx hydrogen engines designed for off-road and construction applications | https://doi.org/10.1039/d4em00448e | Feb 2025 | abstract | measured | values inaccessible |
| W4-S70 | Base H2 car cannot stay ultra-lean at high load; hybrid EMS −90% engine-out NOx, −16% H2; Euro 7 60 mg/km fuel-neutral | arXiv (ETH Zürich / Bosch) | Energy Management of Hydrogen Hybrid Electric Vehicles — A Potential Study | https://arxiv.org/abs/2309.09804 | Sep 2023 | §1, §3.1–3.2, Table 4 (verbatim) | simulated | simulation only |
| W4-S71 | BS-VI M1 NOx 60 mg/km petrol; 80 diesel | ICCT; DieselNet; ARAI | India Bharat Stage VI emission standards (policy update); Indian Emission Regulation Booklet | https://theicct.org/wp-content/uploads/2021/06/India-BS-VI-Policy-Update-vF.pdf ; https://dieselnet.com/standards/in/ld.php ; https://www.araiindia.com/pdf/Indian_Emission_Regulation_Booklet.pdf | 2016/2021 | limit tables | regulation | CNG row not verbatim |
| W4-S72 | H2 indirect GWP100 11 ± 4; GWP20 37 ± 18; OH-bias 20% overestimate; leakage 1–10% | Nature 2025 (PMC12711571); Ocko & Hamburg 2022 (ACP); arXiv 2408.05127; PMC10976891 | The global hydrogen budget; Climate consequences of hydrogen leakage; Model underestimates of OH reactivity… | https://pmc.ncbi.nlm.nih.gov/articles/PMC12711571/ ; https://acp.copernicus.org/preprints/acp-2022-91/acp-2022-91.pdf ; https://arxiv.org/abs/2408.05127 | 2022–2025 | abstracts | modelled | O&H exact values not verified |
| W4-S73 | India VCM ₹1,350–2,800/t (2026); MSCI index $3.50/t (2025), $4.30 (2024); CCTS offset mechanism | Grow Billion Trees; Senken; Regreener; CEEW | Carbon credit rate in India 2026; Carbon credit prices 2026; India's voluntary offset scheme | https://growbilliontrees.com/pages/carbon-credit-rate-in-india-updated-monthly-prices ; https://www.senken.io/academy/understanding-carbon-markets/carbon-credit-price ; https://www.ceew.in/publications/voluntary-carbon-offset-mechanism-and-challenges-in-carbon-credit-trading-scheme-market-for-india | 2025–2026 | price trackers | observed | Indian tracker low grade |
| W4-S74 | Transport pilot guidelines 14 Feb 2024: ₹496 crore to FY26; buses/trucks/cars <3.5 t; steering committee | JSA; Lexology; Business Standard; Deloitte | MNRE issues Scheme Guidelines for pilot projects on use of green hydrogen in the transport sector | https://www.jsalaw.com/newsletters-and-updates/mnre-issues-scheme-guidelines-for-pilot-projects-on-use-of-green-hydrogen-in-the-transport-sector/ ; https://www.business-standard.com/industry/news/govt-issues-guidelines-for-transport-pilot-projects-on-green-hydrogen-s-use-124021401729_1.html | Feb 2024 | summaries | regulation | guideline PDF blocked; funding share per vehicle not extracted |
| W4-S75 | R&D scheme ₹400 crore; guidelines 15 Mar 2024; 1st call 23 projects; 2nd call 14 Jul–21 Aug 2025; TRL ≥4; 100%/80% funding; ₹200 crore testing-infra call | MNRE; Mercom; SolarQuarter; CSIR; pv magazine India | Second Call for Proposals under R&D Scheme of NGHM | https://mnre.gov.in/en/notice/second-call-for-proposals-under-research-and-development-scheme-of-national-green-hydrogen-mission-nghm/ ; https://www.mercomindia.com/mnre-invites-second-round-of-rd-proposals-under-green-hydrogen-mission ; https://www.csir.res.in/en/news-calls-and-events/second-call-proposals-under-research-and-development-scheme-national-green ; https://www.pv-magazine-india.com/2025/08/12/mnre-launches-second-call-for-proposals-to-build-and-upgrade-green-hydrogen-testing-infrastructure/ | Mar 2024–Aug 2025 | notices/news | regulation | remaining budget NOT FOUND |
| W4-S76 | Gujarat GH2 Policy 2025 incentives (electrolyser, HRS, buses, MSME) | Mercom India; iamrenew; JMK (policy PDF) | Gujarat's new Green Hydrogen Policy targets 3 MMTPA by 2035; Gujarat Green Hydrogen Policy 2025 | https://www.mercomindia.com/gujarats-new-green-hydrogen-policy-targets-3-mmtpa-by-2035 ; https://jmkresearch.com/wp-content/uploads/2025/12/Gujarat-Green-Hydrogen-Project-Policy-2025-.pdf | late 2025 | summaries | regulation | gazette not opened |
| W4-S77 | UP GH2 Policy 2024: 30% (40% first 5) capex, cap ₹225 crore; land ₹15,000/acre/yr | Invest UP; Business Standard | Uttar Pradesh Green Hydrogen Policy 2024 | https://invest.up.gov.in/wp-content/themes/investup/sector-assets/policies_schemes/Uttar_Pradesh_Green_Hydrogen_Policy_2024.pdf ; https://www.business-standard.com/economy/uttar-pradesh/uttar-pradesh-green-hydrogen-policy-2024-key-features-incentives-eligibility-and-implementation-126090700729_1.html | 2024 / Sep 2026 explainer | summaries | regulation | no mobility incentive found |
| W4-S78 | Maharashtra GH2 Policy 2023: ₹50/kg blending; 500 FCEV cars 30% ≤₹60 lakh; 20 HRS 30% ≤₹4.5 crore; ₹8,500 crore | MAHAURJA (FAQ); JMK; Swarajya; Global Trade Alert | Maharashtra Green Hydrogen Policy 2023 FAQs | https://www.mahaurja.maharashtra.gov.in/Site/Upload/Pdf/GHfaq.pdf ; https://jmkresearch.com/wp-content/uploads/2023/11/Maharashtra-Green-Hydrogen-Policy-2023.pdf | Oct 2023 | FAQ | regulation | fuel-cell wording |
| W4-S79 | AIS 195:2023 for hydrogen ICE M/N vehicles (GSR 746(E)); AIS 195A:2024 CEV | MoRTH | GSR 746(E) dated 16 Oct 2023 — Hydrogen ICE | https://morth.nic.in/sites/default/files/notifications_document/8th%20GSR%20746(E)%20dated%2016th%20october%202023%20Hydrogen%20ICE.pdf | 16 Oct 2023 | notification | regulation | not opened |
| W4-S80 | AIS-137 durability × DF; hydrogen reference fuel; CMVR 115/116/126 | ARAI (AIS-137 Parts 3/4/6) | AIS-137 Automotive Industry Standard | https://hmr.araiindia.com/Control/AIS/45201991403AMAIS_137_Part_3_F.pdf ; https://hmr.araiindia.com/Control/AIS/53201963840PMAIS_137_Part_4_F.pdf | 2019 + amendments | procedure sections | regulation | snippet |
| W4-S81 | IOCL Faridabad: H2 dispensing since 2005; solar-electrolysis refuelling; PESO permission; Fluitron dispenser Jan 2023 | IndianOil; BusinessWire | R&D Centre pages; Fluitron installs first domestically built hydrogen dispenser | https://iocl.com/pages/r-and-d-centre ; https://www.businesswire.com/news/home/20230116005549/en/Fluitron-Installs-First-Domestically-Built-Hydrogen-Dispenser-in-Faridabad-India | 2023–2025 | pages | observed | — |
| W4-S82 | Tata Prima H.55S H2 specs; trials 4 Mar 2025; 40 trucks VOC Port by 2028; Cummins Jamshedpur production Mar 2024; $425 m JV | Tata Motors; electrive; Cummins; Hydrogen Insight; Truck Junction | Tata Motors drives India's green future…; Tata Motors to deploy 40 H2 ICE trucks; Cummins produces H2-ICE for Tata Motors | https://www.tatamotors.com/press-releases/tata-motors-drives-indias-green-future-with-countrys-first-hydrogen-truck-trials/ ; https://www.electrive.com/2026/02/27/tata-motors-to-deploy-40-h2-ice-trucks-in-india/ ; https://www.cummins.com/en-na/news/releases/2024/05/01/cummins-produces-hydrogen-internal-combustion-engines-tata-motors-new | 2024–2026 | press | observed | — |
| W4-S83 | Ashok Leyland–Reliance H2-ICE truck; AL fleet 2026 | Team-BHP; Business Standard | Ashok Leyland & Reliance unveil India's first hydrogen-ICE truck; India's vehicles industry quietly begins shift… | https://www.team-bhp.com/news/ashok-leyland-reliance-unveil-india%E2%80%99s-first-hydrogen-ice-truck ; https://www.business-standard.com/industry/auto/indian-cv-majors-drive-hydrogen-transition-with-h2-vehicles-125061600725_1.html | Feb 2023; Jun 2025 | news | observed | — |
| W4-S84 | Suzuki hydrogen Burgman (JMS 2025); hydrogen Swift 1.4 L DI 100 kW/220 Nm; Toshihiro Suzuki BMGE 2025 quote; Maruti H-CNG | Suzuki (globalsuzuki); DriveSpark; Automotive World; evo India; V3Cars | Hydrogen Engine Burgman; Hydrogen-powered Suzuki Swift revealed; Suzuki unveils three new motorcycle models… | https://www.globalsuzuki.com/mobilityshow2025/burgman/ ; https://www.drivespark.com/four-wheelers/2026/suzuki-hydrogen-powered-swift-revealed-084763.html ; https://www.automotiveworld.com/news-releases/suzuki-unveils-three-new-motorcycle-models-including-an-ev-scooter-at-the-bharat-mobility-global-expo-2025/ ; https://www.v3cars.com/news/maruti-suzuki-hyrdogen-cng-hcng-engine-for-future-vehicles | 2025–2026 | press/news | observed | no retrofit statement |
| W4-S86 | IIT Madras–Hyundai HTWO centre operational by 2026 | Careers360 | IIT Madras unveils Hyundai HTWO Centre | https://news.careers360.com/iit-madras-unveils-hyundai-htwo-centre-on-campus-operational-by-2026-iitm-hydrogen-india-research/amp | 2025 | news | announced | — |

# A3. Cost model: bill of materials, ownership cost, break-even and sensitivities

## A3.1 Retrofit bill of materials (pilot batch versus commercial volume)

| # | Component or activity | Qty | Unit cost pilot (₹) | Unit cost volume (₹) | Pilot total (₹) | Volume total (₹) | Class | Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Storage and mounting |  |  |  |  |  |  |  |
| 1 | Composite hydrogen cylinder, 60 L water volume, 350 bar (Type III or IV), 15-year life | 1.00 | 250,000 | 95,000 | 250,000 | 95,000 | B | Scaled from DOE/Strategic Analysis Type IV system cost records (S-CYLCOST): about $33/kWh at low volume and $21/kWh at 10k/yr for a 5.6 kg 700 bar system, applied to a 1.45 kg (48 kWh) cylinder with a small-size penalty; example product Luxfer G-Stor Pro V060H35 (S-CYL) |
| 2 | On-tank valve with solenoid, excess-flow, manual shut-off and TPRD (UN R134 class) | 1.00 | 55,000 | 18,000 | 55,000 | 18,000 | C | Production parts exist (S-OTV); no public price; estimate |
| 3 | Mounting frame, straps, boot bulkhead shield, TPRD vent duct, boot ventilation | 1.00 | 30,000 | 11,000 | 30,000 | 11,000 | C | Fabricated steel frame per UN R110-type 20 g / 8 g loads; estimate |
|  | Fuel delivery |  |  |  |  |  |  |  |
| 4 | H35 receptacle, check valve, high-pressure filter | 1.00 | 25,000 | 8,000 | 25,000 | 8,000 | C | ISO 17268 H35 receptacle; estimate |
| 5 | Stainless (316L) high-pressure lines and fittings | 1.00 | 18,000 | 7,000 | 18,000 | 7,000 | C | Estimate |
| 6 | Two-stage pressure regulator 350 bar to injection pressure, with pressure and temperature sensors | 1.00 | 55,000 | 15,000 | 55,000 | 15,000 | C | Production FCEV regulators exist (S-REG); estimate for ICE outlet pressure |
| 7 | Hydrogen port-fuel injectors (4) and rail | 1.00 | 40,000 | 12,000 | 40,000 | 12,000 | C | Bosch / PHINIA H2 PFI injectors announced 2024-25 (S-INJ); estimate |
| 8 | Low-pressure hose, fuel shut-off, pressure relief | 1.00 | 8,000 | 3,500 | 8,000 | 3,500 | C | Estimate |
|  | Controls and safety |  |  |  |  |  |  |  |
| 9 | Hydrogen ECU (dual-map, CAN, automatic petrol fallback, diagnostics) with harness | 1.00 | 45,000 | 15,000 | 45,000 | 15,000 | C | Analogous to sequential CNG ECU at volume; pilot uses low-volume controller |
| 10 | Hydrogen leak sensors (boot and engine bay), controller, cabin warning, vent fan | 1.00 | 22,000 | 7,000 | 22,000 | 7,000 | C | Automotive H2 sensors exist (S-SENS); estimate |
|  | Engine and emissions |  |  |  |  |  |  |  |
| 11 | Cold-grade spark plugs, PCV/oil-separator upgrade, coolant and knock sensor check | 1.00 | 8,000 | 4,000 | 8,000 | 4,000 | C | Backfire and pre-ignition mitigation items (S-H2ICE) |
| 12 | Lean-NOx aftertreatment provision (lean NOx trap) and lambda sensor | 1.00 | 35,000 | 15,000 | 35,000 | 15,000 | C | Needed if lean calibration alone does not meet BS-VI NOx over the cycle (S-NOX); estimate |
|  | Installation and validation |  |  |  |  |  |  |  |
| 13 | Removal of CNG kit and cylinder, installation labour (hours) | 40.00 | 600 | 600 | 24,000 | 8,400 | C | Pilot 40 h; volume 14 h at Rs 600/h (see qty note) |
| 14 | Per-vehicle leak test, commissioning, road test and documentation | 1.00 | 10,000 | 2,500 | 10,000 | 2,500 | C | Estimate |
| 15 | Certification and engineering amortisation per kit | 1.00 | 0 | 8,000 | 0 | 8,000 | B | Volume: one-time development and type-approval cost (Pilot_Budget) spread over 50,000 kits; pilot carries it in the programme budget |
|  | Subtotal parts, labour and validation |  |  |  | 625,000 | 229,400 |  |  |
|  | Warranty provision (share of subtotal) |  | 0 | 0 | 31,250 | 9,176 |  |  |
|  | Installer and kit-maker margin (share of subtotal) |  | 0 | 0 | 0 | 34,410 |  |  |
|  | GST on kit and installation (rate assumed; CNG kits attract 28 percent, hydrogen kit classification to be confirmed) |  | 0 | 0 | 118,125 | 49,137 |  |  |
|  | Installed conversion cost per vehicle (incl. taxes) |  |  | Pilot (G) / Volume (H): | 774,375 | 774,375 | 322123.48 |  |
|  | Share of cylinder and valve in volume kit cost |  |  |  | 0 |  |  |  |
|  | Volume kit cost in Rs lakh |  |  |  | 3 |  |  |  |
|  | Pilot kit cost in Rs lakh |  |  |  | 8 |  |  |  |
|  | Reference: Round 1 assumed Rs 1.8 to 2.2 lakh installed. Reference: sequential CNG retrofit kit incl. cylinder and fitting in India about Rs 0.6 to 0.9 lakh (S-CNGKIT). |  |  |  |  |  |  |  |

Installed conversion cost per vehicle including taxes: pilot ₹774,375; commercial volume ₹322,123. Cylinder and valve are 49 percent of parts cost at volume. One-time development and type-approval costs sit in the programme budget (A9) and are amortised at ₹8,000 per kit over 50,000 kits in the volume case.

## A3.2 Ownership cost comparison (base scenario)

Owner decision in 2028 with a 3-year-old Dzire Tour S CNG; horizon 5 years; 55,000 km per year (167 km x 330 days); nominal discount rate 12 percent; fuel escalation 3 percent, electricity 2 percent, delivered hydrogen minus 10 percent per year from ₹800/kg. Option A keeps the car (residual value at year 5), option B converts it (kit at t0, residual at year 5 reduced by 10 percent), option C sells it at t0 (₹3.5 lakh) and buys a compact BEV (₹10.5 lakh, residual ₹4.2 lakh at year 5). In-shift refuelling or charging time is costed in all three options at ₹150 per hour; overnight charging is free idle time. No depreciation is charged in addition to capital; no battery replacement is assumed (8-year, 160,000 km warranties).

| Total cash cost by year (₹) | t0 (2028) | 2029 | 2030 | 2031 | 2032 | 2033 |
| --- | --- | --- | --- | --- | --- | --- |
| A: continue on CNG | 0 | 293,947 | 299,975 | 306,183 | 312,578 | 219,165 |
| B: hydrogen retrofit | 316,623 | 761,230 | 712,149 | 668,488 | 629,719 | 505,371 |
| C: sell and buy BEV | 730,000 | 166,623 | 168,310 | 170,030 | 171,784 | -246,426 |
| Delivered hydrogen price path (₹/kg) |  | 800 | 720 | 648 | 583 | 525 |

| Result (base scenario) | Value |
| --- | --- |
| NPV of costs: A / B / C (₹ lakh) | 10.4 / 27.3 / 11.0 |
| Levelised cost per km: A / B / C (₹/km) | 5.25 / 13.73 / 5.55 |
| Year-1 fuel or energy cost per km: CNG mode / hydrogen mode / petrol mode / BEV (₹/km) | 3.48 / 14.80 / 6.80 / 1.53 |
| Year-1 blended fuel cost per km: A / B (₹/km) | 3.65 / 12.00 |
| Year-1 non-fuel operating cost per km: A / B / C (₹/km) | 1.69 / 1.81 / 1.49 |
| Extra cost of B versus A; B versus C; C versus A over 5 years (₹ lakh) | 16.8; 16.2; 0.6 |
| Fuel-parity hydrogen price versus CNG; versus petrol (₹/kg) | 188; 368 |
| Full-TCO break-even year-1 hydrogen price versus A; versus C (₹/kg) | -41 (negative: no positive price breaks even); -11 |
| Maximum conversion cost for parity with A; with C at the base hydrogen price (₹) | 0; 0 |
| Simple payback of the conversion versus A | none |
| Minimum annual km for payback within the horizon | n/a |
| Decomposition check (should be zero) | 0.00 and 0.00 |

## A3.3 Sensitivity grids (₹ lakh extra cost over 5 years; positive means the retrofit costs more)

Grid 1: retrofit minus continue-on-CNG by year-1 hydrogen price (rows) and annual km (columns)

| Rs/kg \ km/yr | 30,000 km | 40,000 km | 50,000 km | 60,000 km | 70,000 km |
| --- | --- | --- | --- | --- | --- |
| ₹100/kg | +3.0 | +2.9 | +2.9 | +2.8 | +2.7 |
| ₹150/kg | +3.6 | +3.7 | +3.8 | +3.9 | +4.0 |
| ₹200/kg | +4.1 | +4.4 | +4.7 | +5.0 | +5.2 |
| ₹250/kg | +4.7 | +5.1 | +5.6 | +6.1 | +6.5 |
| ₹300/kg | +5.2 | +5.9 | +6.5 | +7.1 | +7.8 |
| ₹400/kg | +6.3 | +7.3 | +8.3 | +9.3 | +10.3 |
| ₹500/kg | +7.4 | +8.8 | +10.1 | +11.5 | +12.9 |
| ₹600/kg | +8.5 | +10.2 | +11.9 | +13.7 | +15.4 |
| ₹800/kg | +10.7 | +13.1 | +15.6 | +18.0 | +20.5 |

Grid 2: retrofit minus sell-and-buy-BEV by hydrogen price (rows) and BEV price in ₹ lakh (columns)

| Rs/kg \ BEV price | BEV ₹9 lakh | BEV ₹10 lakh | BEV ₹11 lakh | BEV ₹12 lakh | BEV ₹13 lakh | BEV ₹14 lakh |
| --- | --- | --- | --- | --- | --- | --- |
| ₹100/kg | +3.7 | +2.7 | +1.7 | +0.7 | -0.3 | -1.3 |
| ₹150/kg | +4.7 | +3.7 | +2.7 | +1.7 | +0.7 | -0.3 |
| ₹200/kg | +5.7 | +4.7 | +3.7 | +2.7 | +1.7 | +0.7 |
| ₹250/kg | +6.7 | +5.7 | +4.7 | +3.7 | +2.7 | +1.7 |
| ₹300/kg | +7.7 | +6.7 | +5.7 | +4.7 | +3.7 | +2.7 |
| ₹400/kg | +9.7 | +8.7 | +7.7 | +6.7 | +5.7 | +4.7 |
| ₹500/kg | +11.7 | +10.7 | +9.7 | +8.7 | +7.7 | +6.7 |
| ₹600/kg | +13.7 | +12.7 | +11.7 | +10.7 | +9.7 | +8.7 |
| ₹800/kg | +17.7 | +16.7 | +15.7 | +14.7 | +13.7 | +12.7 |

Grid 3: retrofit minus continue-on-CNG by hydrogen price (rows) and installed kit cost in ₹ lakh (columns)

| Rs/kg \ kit cost | kit ₹1.5 lakh | kit ₹2.0 lakh | kit ₹2.5 lakh | kit ₹3.0 lakh | kit ₹4.0 lakh | kit ₹6.0 lakh |
| --- | --- | --- | --- | --- | --- | --- |
| ₹100/kg | +1.1 | +1.6 | +2.1 | +2.6 | +3.6 | +5.6 |
| ₹150/kg | +2.1 | +2.6 | +3.1 | +3.6 | +4.6 | +6.6 |
| ₹200/kg | +3.1 | +3.6 | +4.1 | +4.6 | +5.6 | +7.6 |
| ₹250/kg | +4.1 | +4.6 | +5.1 | +5.6 | +6.6 | +8.6 |
| ₹300/kg | +5.1 | +5.6 | +6.1 | +6.6 | +7.6 | +9.6 |
| ₹400/kg | +7.1 | +7.6 | +8.1 | +8.6 | +9.6 | +11.6 |
| ₹500/kg | +9.1 | +9.6 | +10.1 | +10.6 | +11.6 | +13.6 |
| ₹600/kg | +11.1 | +11.6 | +12.1 | +12.6 | +13.6 | +15.6 |
| ₹800/kg | +15.1 | +15.6 | +16.1 | +16.6 | +17.6 | +19.6 |

| One-at-a-time driver | Low value | High value | Extra cost at low (₹ lakh) | Extra cost at high (₹ lakh) |
| --- | --- | --- | --- | --- |
| Year-1 delivered hydrogen price (Rs/kg) | 550.00 | 1,000 | +11.8 | +20.8 |
| Installed kit cost (Rs) | 273,805 | 402,654 | +16.4 | +17.6 |
| Annual km | 30,000 | 70,000 | +10.7 | +20.5 |
| Hydrogen consumption (kg/100 km) | 1.60 | 2.30 | +14.7 | +20.7 |
| CNG price (Rs/kg) | 95.00 | 78.00 | +16.2 | +17.6 |
| Share of km on hydrogen | 0.85 | 0.50 | +18.9 | +15.3 |

Reading: the extra cost of the retrofit stays positive across every grid cell; the smallest values occur at low hydrogen prices, low mileage and low kit cost. The BEV comparison is closest at high BEV prices and low hydrogen prices.

# A4. Packaging, energy balance and performance calculations

| Quantity | Value | Unit | Method or note |
| --- | --- | --- | --- |
| Energy use per km on CNG (fleet real-world) | 1.920 | MJ/km | LHV / km per kg |
| Energy use per km on petrol (fleet real-world) | 2.147 | MJ/km |  |
| Hydrogen consumption assumed (input) | 1.850 | kg/100 km | Input h2_cons |
| Energy use per km on hydrogen | 2.220 | MJ/km |  |
| Implied H2-ICE efficiency relative to CNG operation (>1 = better) | 0.865 | ratio | Cross-check: lean PFI hydrogen at part load can match or exceed CNG efficiency; retrofit calibration usually loses some of that |
| Hydrogen economy | 54.054 | km/kg |  |
| Cylinder internal volume | 60.000 | L |  |
| Nominal hydrogen at 350 bar, 15 C | 1.440 | kg | Density 24.0 kg/m3 (S-DENS) |
| Nominal hydrogen if 700 bar cylinder of same volume | 2.412 | kg | Design option; needs 700 bar dispensing |
| Hydrogen after a non-cooled 350 bar fill | 1.224 | kg | Fill SOC input; hot ambient, no pre-cooling |
| Residual hydrogen at minimum usable pressure | 0.102 | kg | About 20 bar residual |
| Usable hydrogen per fill | 1.122 | kg | Key output |
| Range on hydrogen per fill | 60.649 | km | Key output |
| Range on hydrogen per fill, 700 bar option | 105.308 | km |  |
| Petrol backup range | 555.000 | km | Existing tank retained |
| Daily hydrogen need at full hydrogen operation | 3.090 | kg/day |  |
| Fills per day needed for 100 percent hydrogen operation | 2.754 | fills/day |  |
| Maximum share of km on hydrogen with the available depot fills | 0.726 | fraction | Compare with input h2_share |
| Share of km on hydrogen used in the model (input) | 0.650 | fraction |  |
| Hydrogen used per operating day in the model | 2.008 | kg/day |  |
| Mass added: cylinder + valve/frame/regulator/lines/ECU | 50.000 | kg |  |
| Mass removed: CNG cylinder + CNG kit | 63.000 | kg |  |
| Net kerb mass change | -13.000 | kg | Negative = lighter than CNG configuration |
| Peak power on petrol | 81.600 | PS | Donor spec |
| Peak power on CNG (factory) | 70.000 | PS | Third-party figure; official CNG rating not captured |
| Estimated peak power on hydrogen (lean PFI) | 42.432 | PS | About 30 percent air displacement plus lean operation (S-H2PWR) |
| Power loss versus petrol | 0.480 | fraction |  |
| Power loss versus CNG | 0.394 | fraction |  |
| Power-to-mass, hydrogen mode, 4 occupants (kerb 1045 kg + 300 kg) | 31.856 | PS per tonne | Urban duty reference: a loaded 800 cc hatchback is about 40 PS per tonne |
| HCNG 18 vol% blend: hydrogen share of energy | 0.062 | fraction | Volumetric LHVs 10.8 and 35.8 MJ/Nm3 (S-HCNG); no vehicle modification |

Packaging basis (concept, to be verified by mock-up): the Dzire Tour S carries a 55 L Type I steel CNG cylinder (OD 316 to 325 mm, about 0.85 to 1.0 m long, 48 to 69 kg) transversely in the boot. The replacement is a 60 L composite cylinder of the Luxfer G-Stor Pro V060H35 class (Type III, 350 bar, OD 400 mm, 767 mm long, 36 kg, 1.45 kg hydrogen; Type IV equivalents from Quantum or NPROXX exist in the 26 to 80 L range but no accessible datasheet was found). The larger diameter consumes an estimated further 40 to 60 L of luggage space; seating is unchanged; ground clearance of 163 mm rules out under-floor mounting. Mounting frame designed to the UN R110 M1 loads of 20 g longitudinal and 8 g lateral (hydrogen-specific installation clauses of AIS-195 to be applied once accessible). Non-cooled 350 bar fills are assumed to reach 85 percent state of charge in hot ambient conditions; SAE J2601 light-duty tables start at 49.7 L storage volume, so the fill protocol must be agreed with the station.

Consumption cross-check: the fleet CNG car at 25 km/kg uses 1.92 MJ/km; the hydrogen car at 1.85 kg/100 km uses 2.22 MJ/km, an efficiency ratio of 0.87 (lean port-injection retrofit calibration below CNG stoichiometric efficiency). Comparable vehicles: Ford Focus H2RV 1.4 kg/100 km (supercharged hybrid, claimed), Mazda RX-8 Hydrogen RE 2.4 kg/100 km (rotary), BMW Hydrogen 7 3.7 kg/100 km (V12, FTP-75), Toyota hydrogen HiAce about 3.1 kg/100 km (3.4 L V6 van, derived). Power: port-injected hydrogen displaces about 30 percent of intake air (about 20 percent power loss at stoichiometric) and lean operation at lambda 2 costs a further 35 to 40 percent; the model uses 52 percent of petrol power (42 PS) with lambda 1.6 to 2.2.

# A5. Regulatory compliance matrix and hazard analysis

## A5.1 Compliance matrix

| Requirement | Applicable technology / category | Current source | Required evidence or test | Responsible party | Status / uncertainty |
| --- | --- | --- | --- | --- | --- |
| Whole-vehicle safety type approval for a hydrogen ICE vehicle | H2-ICE, categories M (incl. M1) and N | CMVR Rule 125M (GSR 746(E), 16 Oct 2023) referencing AIS 195:2023 with Amendment 1 (May 2025) | AIS-195 tests: storage qualification, TPRD, leak and concentration limits, post-crash, labelling | Kit maker with a Rule 126 test agency (ARAI or ICAT) | In force for new vehicles; applicability to retrofitted in-use vehicles not found in the document (not accessible in this study) |
| Legal gateway for alteration or retrofit | Any retrofit | MV Act s.52; CMVR Rule 126 proviso (retrofitted vehicles tested and type-approved by test agencies, or self-certified by OEM or State-authorised workshops; 2026 amendment reported); Rule 115 CNG/LPG kit provisions | Kit type approval; installer authorisation; installation certificate | Kit maker, test agency, State transport department, RTO | Gateway exists in the abstract; no hydrogen kit standard or test procedure exists |
| Emissions of the converted vehicle | H2-ICE conversion | AIS-137 (retrofit part used for CNG/LPG kits); BS-VI M1 NOx 60 mg/km | NOx (and lubricant-derived HC, PM) on the certification cycle; OBD | Test agency | No H2-ICE retrofit emission procedure found; hydrogen combustion produces thermal NOx (US DOE) |
| On-board hydrogen cylinder design approval | Type III or IV composite, 350 bar (700 bar option) | Gas Cylinders Rules 2016 as amended by GSR 225(E), 11 Apr 2025 (definition covers hydrogen cylinders fitted to motor vehicles); Schedule I codes incl. UN R134, CGA S-1.x, IS 5903 for relief devices | PESO design and prototype approval; manufacturer approval; QR or RFID marking | Cylinder manufacturer; PESO Chief Controller | Type IV approvals exist for 150 to 250 L truck cylinders (Time Technoplast); no car-size approved cylinder found |
| Periodic cylinder inspection | All on-board hydrogen cylinders | GCR 2025: hydrostatic or hydrostatic-stretch test every 3 years; 15-year service life | Test certificate from a PESO-approved test station with composite-trained staff | Owner; test station; PESO | Test-station network for composite hydrogen cylinders is immature |
| Installation, venting, TPRD discharge direction | H2-ICE conversion | AIS-195 (assumed) plus UN GTR 13 / R134 discharge rules; GCR nitrogen purge for venting | Installation layout vetted by the test agency (CNG precedent); TPRD routing away from occupants, wheel wells and electrics | Kit maker; installer; test agency | AIS-195 clauses not captured |
| Hydrogen fuel quality | H2-ICE (ISO 14687 grade A/B technically sufficient); FCEV grade D | GCR 2025: fuel conforming to IS 16061 or ISO 14687; FCEV rule (GSR 579(E)) ISO 14687, later IS 16061:2021 | Fuel-quality certificate at the dispenser; IS 18538 (Part 8) station quality control | Fuel supplier; station operator | Indian rules silent on an ICE grade |
| Refuelling station licence | H35 dispensing for cars | GCR 2016 Form-H licence (online module since 14 May 2025); SMPV(U) 2025 amendment for bulk storage; NFPA 2 / ISO 19880-1 (IS 18538 series); receptacle IS/ISO 17268:2020 | PESO licence; station safety case | Station operator; PESO | Very few stations; no public retail station as of 2026 |
| Fuelling protocol | H35 light duty | SAE J2601 (2020): storage systems of 49.7 L or more; 35 MPa tables | Dispenser compliance; agreed fill protocol for a 60 L system | Station operator | No Indian adoption found; small systems near the scope boundary |
| Registration mark | Hydrogen-fuelled vehicle | Central Motor Vehicles (Fourth Amendment) Rules 2025, GSR 819(E), 3 Nov 2025 | Plate or mark per rule | Owner; RTO | Application to converted vehicles is an inference |
| RC endorsement of the retrofit | Any retrofit | Rule 47B(1), Form 22D; s.52 (14 days) | Form 22D with kit type approval, installer certificate, PESO cylinder certificate, layout drawings | Owner; RTO | Requires the kit approval and cylinder approval to exist |
| Insurance | Bi-fuel kits | IMT-25 (CNG/LPG kit endorsement) | Declared kit value; endorsement | Owner; insurer | No hydrogen endorsement exists; needs insurer product |
| OEM warranty | Retrofitted vehicles | Maruti owner's manual reportedly excludes retrofitted CNG | Programme warranty instead | Kit integrator | Powertrain warranty void after conversion (inference) |
| Prototype road trials | Converted prototypes | CMVR Rules 33 to 42 (trade certificate; Rule 41); Rule 126 test-agency evaluation; MNRE transport pilot scheme (buses, trucks and 4-wheelers) | Trade certificate; test-agency programme; pilot sanction | Kit maker; ARAI/ICAT; MNRE and MoRTH steering committee | Precedents: ICAT Mirai pilot (2022), Tata truck trials (2025) |
| Workshop safety | Installers and service | NFPA 2 (2023) repair garages: ventilation at or above 1 cfm per sq ft, high-level exhaust, interlock; PESO composite-cylinder training | Ventilation design, hydrogen detection, permit-to-work | Installer network | No Indian workshop code specific to hydrogen found |
| HCNG blend step | Existing CNG vehicles | GSR 585(E), 25 Sep 2020 (Rule 115): HCNG per IS 17314:2019; vehicles per AIS 024 / AIS 028 Rev 1 | OEM confirmation that the CNG type approval covers HCNG; PESO-licensed HCNG dispensing | OEM; fuel supplier; PESO | Legal as a fuel; dispensing limited to demonstrations |

## A5.2 Hazard analysis (prevention, detection and response, validation, residual risk)

| Hazard | Prevention | Detection and response | Validation | Residual risk |
| --- | --- | --- | --- | --- |
| Hydrogen leak into boot, cabin, garage or basement | Sealed boot bulkhead; boot vented to outside; no ignition sources in the cylinder bay; tank valve closed with ignition off; hydrogen-rated fittings | Two hydrogen sensors (alarm 1 percent, shutdown 2 percent); on-tank valve closes within 1 s; ECU switches to petrol; cabin warning; vent fan | Leak and concentration tests per GTR 13 / AIS-195 (below 4 percent in cabin, below 8 percent at vents); enclosed-space test with the vehicle parked | Residual: undetected slow permeation in an unventilated basement; mitigated by parking rules for the fleet and sensor self-test |
| Material degradation, embrittlement, permeation | 316L stainless lines, hydrogen-compatible elastomers, composite liner; no reuse of CNG steel parts | Leak check at each service; 3-yearly cylinder requalification (GCR 2025); 15-year life | Supplier qualification to ISO 19881 / UN R134; cycling and permeation tests; field inspection log | Residual: long-term liner ageing in hot climates; tracked through pilot sampling |
| Crash damage | Cylinder behind the rear seat inside the crush-free zone; steel frame designed for 20 g longitudinal and 8 g lateral; protected valve orientation | Inertia switch closes the tank valve; post-crash leak limit | Sled test of the mounting; full-vehicle rear crash under AIS-195 procedures; Euro NCAP precedent (Mirai and Nexo five stars) shows hydrogen systems can be packaged safely | Residual: severe side or under-ride impacts beyond test conditions |
| Overpressure and fire | TPRD vents away from occupants, wheel wells and electrics; burst ratio 2.25 times working pressure; heat shielding from exhaust | TPRD opens on heat, giving a controlled release; hydrogen detection in the bay | Bonfire, burst, drop and pressure-cycling tests (GTR 13); vent-line routing review; workshop nitrogen purge for venting | Residual: localised fire impinging on the cylinder without activating the TPRD; mitigated by TPRD placement study |
| Detection or control failure | Redundant sensors; ECU watchdog; manual shut-off valve; limp-home on petrol | Fault codes; forced petrol mode; service lock-out | Hardware-in-the-loop and fault-injection tests; 500 h engine durability; software audit | Residual: common-mode sensor drift; addressed by periodic calibration |
| Workshop and refuelling incidents | NFPA 2 type ventilation, hydrogen detection, permit-to-work, PESO-trained staff; dispenser interlocks; SAE J2601 fill protocol; assembly torque verification (Kjørbo lesson); written procedures for trailer transfers (Santa Clara lesson) | Gas detection interlocks; emergency stop; incident reporting | PESO Form-H licence; commissioning tests; drills | Residual: human error during maintenance; mitigated by two-person rule on high-pressure work |
| Hydrogen combustion emissions | Lean burn (lambda 1.6 to 2.2) with retarded spark; lean NOx trap provision; load limiting with automatic petrol switch | Lambda sensor monitoring; OBD | Emission tests on the BS-VI cycle at the test agency; quarterly checks in the pilot | Residual: transient NOx above the limit in real driving; hence aftertreatment budget |
| Electrical risk | No high-voltage system in the H2-ICE retrofit (12 V only); sensor and valve wiring fused and routed away from hydrogen lines | Standard fault detection | Standard automotive electrical checks | Not applicable beyond conventional vehicle risk (a fuel-cell conversion would add high-voltage hazards) |

Hydrogen properties used: flammability 4.0 to 75.0 percent in air; detonation 18.3 to 59 percent; minimum ignition energy 0.017 mJ; autoignition 585 C; diffusivity about 3.8 times natural gas; nearly invisible flame (H2Tools, US DOE). Prototype work and high-pressure testing are confined to qualified facilities (test cells at HORIBA Chakan, ARAI, IOCL R&D; PESO-approved test stations). This annexure is a development and validation plan, not conversion instructions.

# A6. Market sizing method and bottom-up funnel

| Step | Vehicles (base) | Share applied | Basis |
| --- | --- | --- | --- |
| CNG vehicles in India, all categories | 8,195,000 |  | PNGRB citing Vahan (Mar 2025), class A |
| Passenger cars among them | 4,507,250 | 55 percent | Estimate from cumulative factory-CNG car sales (Maruti about 2.6 million FY20-FY26 plus other OEMs) and conversions; Round 1 said 65 percent (unverified) |
| Technically eligible: factory-fitted, boot-cylinder layout, under 6 years old at 2028 | 2,253,625 | 50 percent | Factory-fitted, boot cylinder, under 6 years old at 2028 (FY23-FY26 sales) |
| In commercial fleets (taxi, aggregator, corporate, institutional) | 450,725 | 20 percent | Delhi alone registers 60,000 to 85,000 taxis; no national split published; assumption |
| In cities with a hydrogen hub station by 2030 | 180,290 | 40 percent | MNRE corridors: Delhi-NCR, Ahmedabad-Vadodara-Surat, Pune-Mumbai, Kochi and others |
| Plausible early adopters (return-to-base fleets willing to convert) | 9,014 | 5 percent | Hypothesis to be validated in Phase 0 |
| Supply-constrained capacity: stations x cars per station | 1,394 |  | 8 hub stations x 174 cars per station (500 kg/day at 70 percent utilisation, 2.0 kg per car per day) |
| Realistic 2030 converted fleet (minimum of demand and supply) | 1,394 |  | Minimum of demand and supply |
| Round 1 claim for 2030 | 100,000 |  | For comparison |

Downside and upside funnels (workbook scenario selector) give roughly 700 to 2,600 cars by 2030; in every case supply (stations) rather than demand binds. Private owners are excluded from the first segment because they lack depot fuelling and demand aggregation; they could follow only after hub stations exist.

# A7. Global benchmark evidence: progress and setbacks

| Programme | Technology and class | Status (date) | Scale | Fuel-supply model | Commercial limitation | What transfers to an Indian retrofit |
| --- | --- | --- | --- | --- | --- | --- |
| Toyota Mirai | FCEV sedan | Production since 2014; MY2026 refresh with fuel credits | US sales 499 (2024), 210 (2025) | Public retail (California, Japan) | Station closures and USD 32 to 36/kg fuel | Transfers: nothing on cost; warns against retail-network dependence |
| Hyundai Nexo (2nd gen) | FCEV SUV | Korea sales from June 2025 | Korea about 36,000 FC cars in circulation | Public retail with subsidy | Dependence on Korean subsidies | Transfers: policy dependence |
| Honda CR-V e:FCEV | Plug-in FCEV | On lease in California since July 2024 | About 300 per year | California retail | GM-Honda fuel-cell plant ends 2026 | Transfers: OEM appetite is falling |
| Toyota GR Corolla H2 | Liquid-hydrogen ICE race car | Racing development 2021 to 2025 | 1 car | Trackside liquid hydrogen | No production plan | Transfers: combustion durability learning only; not a cost proof |
| Toyota hydrogen HiAce (Australia) | 3.4 L twin-turbo V6 H2-ICE van | Pilot with CPB Contractors (2024); hybrid variant announced 2025 | Few prototypes | Captive depot (Toyota Altona) | Range below 200 to 250 km; SCR needed for Euro VI | Transfers: depot model, NOx aftertreatment, range reality |
| HySE (Kawasaki, Suzuki, Honda, Yamaha, Toyota) | Small H2-ICE research | Dakar 2024 and 2025 buggies | Research | Rally logistics | Pre-commercial | Transfers: small-engine hydrogen combustion know-how |
| Suzuki hydrogen Burgman; Suzuki-AVL hydrogen Swift | H2-ICE scooter concept (70 MPa); 1.4 L DI hydrogen Swift prototype (100 kW, 220 Nm) | JMS 2023 and 2025; Vienna Motor Symposium May 2026 | Concepts | Not stated | 'Not launching anytime soon' (Burgman); Swift is a prototype | Transfers: Suzuki-relevant small-car hydrogen combustion exists; DI, not retrofit |
| Cummins B6.7H (India) | 6.7 L H2-ICE | First production engine Mar 2025, Jamshedpur, for Tata trucks | Limited | Truck stations (350 bar) | Fuel availability and TCO | Transfers: Indian supply of hydrogen injection and controls |
| JCB hydrogen engine | 4.8 L off-highway H2-ICE | EU type approval (Stage V) May 2025; above 130 evaluation engines | Evaluation fleet | On-site bowsers | Not road-vehicle homologation | Transfers: regulatory precedent for hydrogen combustion approval |
| KEYOU (Germany) | H2-ICE conversion of 18 t trucks and 12 m buses | First truck delivered Dec 2024; Daimler Truck partnership for 2027 | Tens of vehicles | H2 Mobility truck stations | Conversion cost; fuel price | Transfers: the only commercial H2-ICE retrofit model, heavy-duty, depot-based |
| ULEMCo (UK) | Hydrogen dual-fuel conversions of vans and municipal vehicles | Commercial conversions (Aberdeen fleet) | Tens of vehicles | Council depot hydrogen | No passenger-car conversions; prices not public | Transfers: municipal fleet model |
| Stellantis hydrogen vans | FCEV LCVs | Programme ended 16 Jul 2025 | Series production cancelled | Public retail | Infrastructure, capital, incentives | Transfers: light-duty hydrogen retail model failed even for an OEM |
| Shell California; H2 Mobility Germany; OMV Austria | Retail hydrogen stations | Shell closed 7 car stations (Feb 2024); H2 Mobility closed 36 car stations (2025); OMV closed all (2025) | Network contraction | Retail | Low utilisation by cars | Transfers: utilisation, not technology, kills stations |
| India: MNRE pilots; Tata truck trials; NTPC Leh; IOCL HCNG buses; Mirai-ICAT pilot | Buses, trucks, HCNG, one FCEV car | 12 projects, 70 vehicles, 16 stations sanctioned (₹410.77 crore, ₹41.58 crore released); Tata 16 trucks (2025), 40 more (2026); NTPC Leh 5 buses at 80 kg/day; 50 HCNG buses (2020); Mirai pilot 2022 | Tens of heavy vehicles | Captive stations | No cars; only three captive dispensing points | Transfers: corridor stations that a car pilot can share; scheme names 4-wheelers |

# A8. Pilot design, instrumentation and first 90 days

| Element | Design |
| --- | --- |
| Fleet size and why | 30 converted cars: large enough to spread across three fleets and two duty patterns (staff shuttle and pool car), to accumulate about 45 vehicle-years of operation in 18 months, and to load one dispensing point at about 60 kg/day; small enough to insure and support from two workshops. |
| Duration and duty cycle | 18 months of operation (April 2029 to September 2030), 150 to 210 km per operating day, 330 days per year, two depot fills per day, hydrogen mode by default with automatic petrol fallback. |
| Comparison vehicles | 10 unmodified Dzire Tour S CNG cars and 10 compact BEVs from the same fleets on the same duty, instrumented identically, to give matched cost, availability and emissions references. |
| Data collected | Fuel and energy use per trip (dispenser logs, CAN mass-flow, meter readings); range and fills; availability and downtime by cause; maintenance events and cost; refuelling time and queue; emission tests quarterly (NOx, HC, PM); hydrogen sensor logs; luggage and passenger complaints; driver ratings; station utilisation and delivery reliability. |
| Instrumentation and logs | CAN logger with GPS on all 50 cars; hydrogen concentration logging in boot and engine bay; dispenser data (mass, pressure, temperature, fill time); workshop job cards; incident and near-miss register with a safety officer sign-off. |
| Maintenance and safety reporting | Monthly safety review; every hydrogen alarm investigated within 24 hours; 3-yearly cylinder requalification scheduled; PESO and test-agency informed of reportable events; independent evaluator receives raw data quarterly. |
| Customer and operator feedback | Quarterly structured interviews with fleet managers and drivers on power, boot space, refuelling, confidence; willingness-to-pay survey at month 12. |
| Independent validation | An academic or think-tank evaluator (proposed IIT Delhi or CEEW) publishes consumption, availability, emissions and cost results; test-agency emission and safety tests are the formal evidence. |
| What a pilot cannot prove | 30 cars over 18 months cannot establish a statistical safety case; safety evidence comes from the type tests (storage, leak, crash, bonfire) and the standards route, not from incident-free operation. |

## A8.1 Success measures for gate 4 (September 2030)

Consumption at or below 2.2 kg/100 km in fleet use; range per fill at or above 55 km; hydrogen share of km at or above 60 percent.

Availability at or above 95 percent; conversion-related downtime below 2 percent of operating days.

Zero reportable hydrogen incidents; all alarms investigated; no unplanned venting.

Tailpipe NOx at or below 60 mg/km on the certification cycle and on-road spot checks within 1.5 times the limit.

Measured service and maintenance cost within ₹1.2/km; refuelling time under 6 minutes per fill; luggage and passenger acceptance from fleet managers.

Delivered hydrogen price at or below ₹250/kg with a documented path to ₹190/kg; station utilisation trend above 50 percent.

## A8.2 First 90 days (from programme start, October 2026)

| Window | Actions |
| --- | --- |
| Days 1 to 15 | Appoint SRDI engineering lead and programme manager; agree governance with NIT Warangal team; freeze the evidence register; write to AISC secretariat, MoRTH, ARAI, ICAT and PESO requesting a technical meeting on the retrofit route. |
| Days 15 to 30 | Shortlist fleets (PSU staff transport, airport-contract operators, corporate fleets in Faridabad and Gurugram); start 30 structured interviews; fit telematics to 20 donor cars for duty-cycle data. |
| Days 30 to 45 | Request cylinder samples and quotations (Luxfer, Quantum, NPROXX, Time Technoplast); injector and regulator quotations (Bosch, PHINIA, OMB Saleri, Poppe + Potthoff); test-cell availability (HORIBA Chakan, ARAI, IOCL R&D). |
| Days 45 to 60 | Boot mock-up of a Dzire Tour S with a 400 mm cylinder dummy; luggage and seating assessment; CAD packaging and mounting concept; preliminary hazard analysis workshop. |
| Days 60 to 75 | Meet IOCL R&D Faridabad and the MNRE corridor awardees on shared dispensing; draft a proposal under the MNRE transport pilot scheme (4-wheeler category) with ARAI; check Gujarat station subsidy for the second hub. |
| Days 75 to 90 | Gate 1 dossier: regulator view, partner letters, demand evidence, packaging result, updated cost model; proceed, redesign or stop decision by the steering group. |

# A9. Phase budgets, responsibilities and funding

| Phase / line item | Quantity | Unit cost (₹) | Total (₹) |
| --- | --- | --- | --- |
| Phase 0: Feasibility and validation (Oct 2026 - Mar 2027) |  |  |  |
| Regulator and test-agency consultations, route definition (AISC, MoRTH, ARAI/ICAT, PESO) | 1.0 | 2,500,000 | 2,500,000 |
| Fleet and fuel-partner validation: 30 operator interviews, duty-cycle telematics on 20 cars | 1.0 | 2,500,000 | 2,500,000 |
| Donor inspection, packaging study, CAD and mule vehicle (boot mock-up) | 1.0 | 4,000,000 | 4,000,000 |
| Phase subtotal |  |  | 9,000,000 |
| Phase 1: Engineering and controlled validation (Apr 2027 - Mar 2028) |  |  |  |
| Hydrogen engine test-cell time and instrumentation (calibration, NOx, abnormal combustion, 500 h) | 1.0 | 30,000,000 | 30,000,000 |
| Prototype conversions (3 vehicles) at pilot kit cost | 3.0 | 774,375 | 2,323,125 |
| Prototype donor vehicles (used Tour S CNG) | 3.0 | 500,000 | 1,500,000 |
| Storage system qualification samples and tests (burst, cycling, bonfire, drop, permeation) | 1.0 | 15,000,000 | 15,000,000 |
| Vehicle-level safety tests (leak, sled/crash structure, ventilation) at test agency | 1.0 | 12,000,000 | 12,000,000 |
| Engineering team (8 FTE x 12 months) | 96.0 | 250,000 | 24,000,000 |
| Phase subtotal |  |  | 84,823,125 |
| Phase 2: Certification and pilot build (Apr 2028 - Mar 2029) |  |  |  |
| Kit type approval and emissions testing at ARAI/ICAT (analogous to CNG kit approval) | 1.0 | 10,000,000 | 10,000,000 |
| Durability demonstration (2 vehicles x 80,000 km accelerated) | 1.0 | 8,000,000 | 8,000,000 |
| Pilot conversions (30 vehicles) at pilot kit cost | 30.0 | 774,375 | 23,231,250 |
| Instrumentation and telematics (30 cars + 20 comparison cars) | 50.0 | 60,000 | 3,000,000 |
| Hydrogen dispensing point at partner site (compressor, cascade, one dispenser, safety, PESO) | 1.0 | 100,000,000 | 100,000,000 |
| Workshop upgrade (ventilation, detection, training) at 2 service points | 2.0 | 4,000,000 | 8,000,000 |
| Phase subtotal |  |  | 152,231,250 |
| Phase 3: Pilot operation and evaluation (Apr 2029 - Sep 2030) |  |  |  |
| Hydrogen fuel for 30 cars, 18 months (kg at delivered pilot price) | 29,821 | 1,433 | 42,729,079 |
| Operations, safety officer, data analysis and independent evaluation (18 months) | 18.0 | 1,500,000 | 27,000,000 |
| Quarterly emissions and consumption tests at test agency | 6.0 | 800,000 | 4,800,000 |
| Insurance, incident reserve and driver training | 1.0 | 6,000,000 | 6,000,000 |
| Phase subtotal |  |  | 80,529,079 |
| Subtotal all phases |  |  | 326,583,454 |
| Contingency (share of subtotal) | 0.1 |  | 48,987,518 |
| Total programme budget to September 2030 (Rs) |  |  | 375,570,972 |
| Total in Rs crore |  |  | 38 |
| Suzuki R&D Center India / Maruti Suzuki (engineering, prototypes, test cells) | 0.4 |  | 150,228,389 |
| Public support sought (MNRE NGHM R&D or pilot scheme, state hydrogen policy) | 0.3 |  | 131,449,840 |
| Fuel and station partner (OMC or gas utility): dispensing point | 0.1 |  | 56,335,646 |
| Fleet partner (vehicles, drivers, operations) | 0.1 |  | 37,557,097 |
| Item | 2028 | 2029 | 2030 |
| Converted cars in operation (cumulative) | 3.0 | 30 | 1,394 |
| Fleet-hub stations serving cars | 1.0 | 1 | 8 |
| Hydrogen demand from cars (t/yr) | 2.0 | 20 | 924 |

Programme total to September 2030 including 15 percent contingency: ₹37.6 crore (phases ₹0.9, ₹8.5, ₹15.2, ₹8.1 crore). Proposed funding split: Suzuki R&D Center India and Maruti Suzuki 40 percent; public support 35 percent (MNRE transport pilot scheme, which names buses, trucks and 4-wheelers, or the R&D scheme with up to 80 percent support for industry at TRL 4 or higher; Gujarat's 30 percent station subsidy for the second hub); oil marketing company partner 15 percent; fleet partner 10 percent. For calibration, MNRE's heavy-vehicle pilots average ₹5.6 to 5.9 crore of support per vehicle including stations. All partners are proposed, not confirmed.

| Responsibility | Proposed holder | Rationale |
| --- | --- | --- |
| Engineering lead, calibration, prototypes | Suzuki R&D Center India with Maruti Suzuki engineering | CNG engine know-how; Suzuki hydrogen combustion prototypes |
| Kit integration, warranty, installer training | Kit integrator (aftermarket gaseous-fuel specialist) under SRDI oversight | Existing CNG kit approval experience |
| Type approval and testing | ARAI or ICAT (Rule 126 agencies); PESO for cylinders and station | Statutory |
| Dispensing point ownership and operation | Oil marketing company (proposed IOCL, Faridabad) | Existing hydrogen dispensing experience; corridor awardee |
| Hydrogen supply | SIGHT-awarded green hydrogen producer by tube trailer | Certified carbon intensity |
| Vehicles and operations | Fleet partner (PSU or corporate staff transport, airport contract) | Return-to-base duty |
| Independent evaluation | IIT Delhi or CEEW (proposed) | Credibility of results |
| Development risk | Programme (consortium) | Gated spend |
| Utilisation risk | Station partner with take-or-pay from the fleet | Aligns incentives |
| Residual-value risk | Fleet, backed by programme buy-back | Removes owner downside in the pilot |

# A10. Judge Q&A, validation log and change log

## A10.1 Judge Q&A (20 questions)

| Question | Answer |
| --- | --- |
| What is distinctive about the proposal? | It converts the installed factory-CNG base rather than building new vehicles, keeps petrol as an automatic backup so no car is stranded by missing stations, and, unlike most hydrogen pitches, states the price at which it works (₹188/kg delivered for CNG fuel parity) and the gates at which it stops. |
| Why retrofit this vehicle? | The Dzire Tour S CNG is the fleet workhorse with a 4-cylinder 1.2 L engine (power margin after the hydrogen derate), a boot bay that already carries a 55 L cylinder, hardened valve seats, a 37 L petrol tank for backup, and the largest donor pool (Maruti sells about 70 percent of India's CNG cars). |
| Why would the customer choose it over staying on CNG or buying a BEV? | On money alone they would not: continuing on CNG costs ₹5.2/km, a compact BEV ₹5.6/km including purchase, and the retrofit ₹13.7/km at 2028 hydrogen prices. The customer for the pilot is a fleet with a decarbonisation mandate or a public programme paying for abatement and data; the commercial customer appears only if delivered hydrogen falls below about ₹190/kg. |
| Does the hydrogen storage system actually fit? | A 60 L composite cylinder (OD 400 mm, 767 mm long, 36 kg) fits the transverse bay used by the 55 L CNG cylinder (OD 316 mm, about 900 mm), but it is 84 mm larger in diameter, so the boot loses a further 40 to 60 L. No under-floor option exists at 163 mm ground clearance. Phase 0 includes a physical mock-up because this is a concept, not validated design work. |
| What is sacrificed in passenger capacity, luggage or payload? | Seating is unchanged (five). Luggage falls from roughly 200 L with CNG to about 150 L. Kerb mass falls by about 13 kg, so payload is unchanged or slightly better. Peak power in hydrogen mode is about 42 PS, 48 percent below petrol; the car switches to petrol automatically for high load. |
| Which components cannot be reused? | Every CNG fuel-system part: the steel cylinder (ISO 11439 steels are limited to 2 percent hydrogen), regulator, rail, injectors, hoses and lines. Hydrogen parts need UN R134 or ISO 19881 class qualification. The engine, gearbox, petrol system, catalyst and body are retained. |
| What is the credible pilot conversion cost? | ₹7.7 lakh per car including GST, dominated by the imported cylinder and valve at small quantities; ₹3.2 lakh installed at 10,000 kits per year (₹2.3 lakh ex-works). Round 1's ₹1.8 to 2.2 lakh is roughly the ex-works volume figure without taxes and margin. |
| Where will hydrogen come from and at what delivered price? | Certified green hydrogen by tube trailer from a SIGHT-awarded producer to a shared dispensing point in Faridabad. Our build-up gives about ₹1,433/kg at the pilot point (150 kg/day, 50 percent used) and about ₹584/kg at a 2030 hub (500 kg/day, 70 percent used); CEEW's central 2030 estimate is ₹360/kg. India's record refinery-gate tender is ₹279/kg before compression and dispensing. |
| What if fuel cost stays higher than expected? | The pilot still delivers its purpose (approval route, real-world data, supplier readiness) because fuel is a programme cost. The scale decision at gate 4 simply fails, and the programme stops and publishes. |
| What if the station is unavailable? | Every car runs on petrol automatically (555 km on a tank); the fleet loses the hydrogen benefit, not the service. The station partner carries utilisation risk through a take-or-pay contract, and a 1.5-day cascade buffer covers delivery slips. |
| What is the exact approval uncertainty? | There is no standard or rule for a hydrogen retrofit kit. AIS-195 covers new hydrogen ICE vehicles; AIS-157 (fuel cell) explicitly excludes retrofits; Rule 126 and Rule 115 provide the retrofit machinery only for CNG and LPG. We need either an AIS-195 extension or a Rule 115 hydrogen-kit provision; the trial itself can run under a trade certificate with test-agency evaluation, as the Mirai and Tata trials did. |
| How is NOx addressed? | Lean burn above lambda 2 gives near-zero engine-out NOx at steady state; transients do not stay lean, so the kit carries a lean NOx trap provision and lambda control, and the gate-2 test requires 60 mg/km or better on the BS-VI cycle. This is a target to demonstrate. |
| What is the environmental benefit under matched assumptions? | With certified green hydrogen (2 kg CO2e/kg) the converted car emits about 89 g CO2e/km well-to-wheel including the kit, against 137 on CNG and 134 for a new BEV on the 2030 grid including its manufacturing. With grid-electrolysis hydrogen it would be four times worse than CNG, so the fuel's certification is a condition, not a detail. Hydrogen combustion also emits NOx. |
| Who funds development and underutilised infrastructure? | Proposed split of ₹37.6 crore: Suzuki R&D Center India and Maruti 40 percent (engineering, prototypes, test cells), public support 35 percent (the MNRE transport pilot scheme names 4-wheelers; ARAI runs its calls), an oil marketing company 15 percent (dispensing point), the fleet 10 percent. Station utilisation risk sits with the station partner under take-or-pay. None of this is committed. |
| What result would make you stop? | No written regulator route by March 2027; backfire or NOx not controlled within 500 test-cell hours; delivered hydrogen still above ₹250/kg in 2030 with no credible path to ₹190/kg; or fleets refusing the power and boot loss in Phase 0 interviews. |
| What can realistically be achieved by 2030? | An approved retrofit route, 30 cars with 18 months of independent data, a shared hydrogen dispensing point in Faridabad, supplier quotations for car-size cylinders, and a decision. Conditional expansion to 1,000 to 1,500 cars at up to 8 hub stations is possible only after gate 4; Round 1's 1,00,000 cars is not. |
| Why not a fuel-cell conversion for better efficiency? | It halves hydrogen use per km but replaces the entire powertrain (stack, battery, motor, inverter, cooling, tank, controls) at ₹15 to 25 lakh per car in a pilot, more than a new BEV. It is not a retrofit in cost terms and has no retrofit approval route either. |
| Why keep the HCNG step if it displaces only 6 percent of energy? | Because it is legal today (GSR 585(E), IS 17314), needs no vehicle change, and lowers CO and HC; it is an air-quality measure that lets CGD companies start blending. We no longer present it as decarbonisation. |
| How do you know fleets want this? | We do not yet. Customer demand is a stated hypothesis; Phase 0 funds 30 structured interviews and telematics on 20 donor cars, and gate 1 requires evidence from at least three fleets before any engineering spend. |
| Is Suzuki endorsing this? | No. Suzuki R&D Center India is proposed as engineering lead because of its CNG engine and calibration know-how and Suzuki's own hydrogen combustion prototypes (Burgman, Swift with AVL). No endorsement or partnership is claimed. |

## A10.2 Validation log

| Item | What was done |
| --- | --- |
| Attachments | Both PDFs rendered and inspected page by page (brief: 3 pages; submission: 4 slides at 1440 x 810 pt). Title slide preserved as the native 1376 x 768 raster with the source's 3.14 pt white bands; pixel comparison of renders confirms visual identity. |
| Research | Four workstreams (engineering; safety and regulation; market, infrastructure and benchmarks; economics and emissions) plus a gap-closure pass. Research cut-off 14 September 2026. 309 register entries. |
| Access limitation | The build environment's network policy blocked direct access to most primary hosts (ARAI, MoRTH, PIB, PESO, MNRE, DOE, NIST, IEA, OEM sites). Findings rely on indexed excerpts of those pages; PIB releases of 14 Feb 2024 and 3 Mar 2025 and the Scribd copy of AIS-157 were read in full through the search index. Entries marked 'excerpt' or 'summary' should be re-verified from the originals before external use. |
| Model | Workbook built with formulas only, recalculated with LibreOffice Calc; zero error cells; decomposition checks (a + b km + c km P) reproduce the TCO differences to zero; scenario selector tested for 1, 2, 3. |
| Consistency | Slides, annexure and pitch read numbers from the same output file exported from the recalculated workbook; budgets reconcile to the funding ask; rollout quantities reconcile to station capacity. |
| Design | Every slide rendered and inspected; overlaps and clipping fixed; fonts limited to Arial for cross-platform fidelity; PPTX and PDF each below 2 MB. |
| Not done | No customer interviews, letters of interest, test results, supplier quotations or partnerships exist; all are labelled as hypotheses or proposals. The submission portal's annexure handling could not be verified (site inaccessible). |

## A10.3 Change log: Round 1 to Round 2

| Element | Round 1 | Round 2 |
| --- | --- | --- |
| Core idea | Convert the installed CNG car base to hydrogen, fleets first, aggregate demand at hubs | Retained |
| Fallback fuel | CNG retained with automatic H2/CNG switching (second cylinder) | Petrol retained (already in every factory CNG car); CNG hardware removed; two cylinders do not fit |
| Donor | CNG cars in general (WagonR image) | Dzire Tour S CNG primary; WagonR, Ertiga, Eeco assessed as alternatives |
| Retrofit ladder | HCNG (2026-27), bi-fuel H2 plus CNG (2027-28), dedicated H2-ICE (2029-30) | HCNG as an air-quality step (about 6 percent of energy), H2-ICE retrofit pilot (2027-30), conditional 700 bar or hub-scale option after 2030 |
| Hydrogen price basis | Production-cost trajectory to ₹160-200/kg treated as pump price | Delivered price build-up: ₹1,433/kg pilot, ₹584/kg 2030 hub (our conservative), ₹360/kg CEEW central |
| Economics | ₹1.5 lakh annual saving versus petrol, 18-month payback | Fair three-way TCO (continue CNG, retrofit, sell and buy BEV): retrofit never pays back within 5 years; parity conditions stated |
| Kit cost | ₹1.8 to 2.2 lakh | ₹7.7 lakh pilot; ₹3.2 lakh installed at volume (₹2.3 lakh ex-works) |
| Range and power | 400 plus km combined | 61 km per hydrogen fill plus 555 km petrol; 42 PS in hydrogen mode |
| Regulation | AIS-157 plus PESO plus ISO/SAE | AIS-195 (new H2-ICE vehicles), Rule 126/115 retrofit template, PESO GCR 2025, Form-H; no retrofit route exists; gate 1 added |
| Roadmap | 500 cars and 100 stations (2026-27), 25,000 kits (2027-28), 1,00,000 plus cars (2029-30) | 3 prototypes (2027-28), 30 pilot cars (2028-30), conditional 1,000 to 1,500 cars after 2030; four gates |
| Impact | 1,00,000 cars, 0.9 Mt CO2, 15,000 t/yr hydrogen, ₹2,000 crore industry, 10,000 jobs | About 1,400 cars, 3,600 t CO2e/yr, 924 t/yr hydrogen by 2030; jobs not claimed |
| Market figures | 7.5 million CNG vehicles, 65 percent cars, 4,000 retrofit centres | 8.2 million (Mar 2025); car share estimated 55 percent; retrofit-centre count unverified |
| Benchmarks | Toyota GR Corolla and Hilux H2-ICE | HiAce (Australia) is the hydrogen combustion van; Hilux was a fuel-cell prototype; Suzuki-AVL Swift added; global setbacks added |
| Carbon credits | USD 10 to 15/t as revenue | Immaterial; treated as upside only |
| Claims removed | Job creation, 'zero range anxiety' as absolute, 'CNG cylinder retained', implied component reuse, 'BS-VI compliance pathway' as achieved | Replaced by evidence-labelled statements |

The Round 1 deck lists Raunak Jain first but does not name a team leader. The submission filename stem Raunak_TeamWookies_NITWarangal assumes Raunak Jain is the leader; confirm before uploading.